| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0262_r1 |
| --- | --- |

| Title: | AHG9: NAL Unit Priority SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eun-Seok Ryu Yan Ye Yuwen He Yong He 9710 Scranton Rd, Suite 250 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4808 eun.ryu@interdigital.com yan.ye@interdigital.co,m yuwen.he@interdigital.com yong.he@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

Video streaming server and media-aware router need high-level syntax to indicate NALU priority without parsing video bitstream. However, current HEVC working draft 8 does not indicate packet priority within the same temporal layer. This contribution proposes a new SEI message that indicates the priority of following NAL units until the next NAL unit priority SEI message is present.

# Problem Statement

In video compression and transmission, packet prioritization is of utmost importance for the role it plays in UEP (Unequal Error Protection), packet dropping for bandwidth adaptation, as well as QP (Quantization Parameter) control for enhanced video quality, to name a few. As such, packet priority is essential for optimal QoS handling in applications such as video streaming. However, the current HEVC draft specification [1] does not provide any information for prioritization. For example, priority ID is not specified. And although video streaming system and/or media-aware router know(s) the temporal level in hierarchical-b structure of video packets, video packets with the same temporal ID also could have different priorities. That is, pictures in the same temporal level in hierarchical-B structure can have different influence on error propagation and on decoded video quality.

## Uniform Prioritization

Fig. 1 denotes the current uniform prioritization in hierarchical-B structure [2] with four dyadic stages in temporal domain. Although pictures in lower temporal levels have higher priority, in the current HEVC working draft, however, there are no additional methods to assign different priorities to pictures at the same temporal level.

Fig. 1. The current uniform prioritization in hierarchical-B structure with four temporal levels.

Fig. 1 shows the random access setting in the common test condition [3]. In Fig. 1, POC 2 and POC 6 have the same priority. However, uniform prioritization at the same temporal level presents a problem because the importance of two POCs could be different according to both the RPS (Reference Picture Set) and the size of the reference picture list. The following sections explain why these two pictures have different importance.

## The Features of Picture Referencing

To compare the importance of two POCs located in the same temporal level, we define Position A as the pictures with POC equal to 2 + N × GOP (Group of Picture), and Position B as the pictures with POC equal to 6 + N × GOP, where GOP is 8 and N represents the number of GOP(s). Then, POC 2, 10, 18, and 26 belong to Position A, and POC 6, 14, 22, and 30 belong to Position B.

In the RA setting, Position A and Position B appear in the reference picture lists (L0 and L1) with different frequencies during each intra period. Table 1 shows the reference picture lists for each POC and the total number of times Position A and Position B were referenced, using intra period equal to 32 as an example. In the table, POCs in Position A (blue) were referenced 12 times and POCs in Position B (red) 16 times during the intra period (= 32 POCs). Thus, compared to Position A, Position B is referenced more, indicating the pictures in Position B are more likely to cause severe error propagation if they are dropped during transmission.

Table 1. The amount of appearance in reference picture list (RA setting, GOP 8, IntraPeriod 32).

| Picture | QP Offset | Reference Buffer size (L0 and L1) | Temporal ID | Reference Picture Set (RPS) |  |  |  | Reference Picture Lists |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | L0 |  |  |  | L1 |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 1 | 4 | 0 | -8 | -10 | -12 | -16 | 0 |  |  |  | 0 |  |  |  |
| 4 | 2 | 2 | 0 | -4 | -6 | 4 |  | 0 | 8 |  |  | 8 | 0 |  |  |
| 2 | 3 | 2 | 0 | -2 | -4 | 2 | 6 | 0 | 4 |  |  | 4 | 8 |  |  |
| 1 | 4 | 2 | 0 | -1 | 1 | 3 | 7 | 0 | 2 |  |  | 2 | 4 |  |  |
| 3 | 4 | 2 | 0 | -1 | -3 | 1 | 5 | 2 | 0 |  |  | 4 | 8 |  |  |
| 6 | 3 | 2 | 0 | -2 | -4 | -6 | 2 | 4 | 2 |  |  | 8 | 4 |  |  |
| 5 | 4 | 2 | 0 | -1 | -5 | 1 | 3 | 4 | 0 |  |  | 6 | 8 |  |  |
| 7 | 4 | 2 | 0 | -1 | -3 | -7 | 1 | 6 | 4 |  |  | 8 | 6 |  |  |
| 16 | 1 | 4 | 0 | -8 | -10 | -12 | -16 | 8 | 6 | 4 | 0 | 8 | 6 | 4 | 0 |
| 12 | 2 | 2 | 0 | -4 | -6 | 4 |  | 8 | 6 |  |  | 16 | 8 |  |  |
| 10 | 3 | 2 | 0 | -2 | -4 | 2 | 6 | 8 | 6 |  |  | 12 | 16 |  |  |
| 9 | 4 | 2 | 0 | -1 | 1 | 3 | 7 | 8 | 10 |  |  | 10 | 12 |  |  |
| 11 | 4 | 2 | 0 | -1 | -3 | 1 | 5 | 10 | 8 |  |  | 12 | 16 |  |  |
| 14 | 3 | 2 | 0 | -2 | -4 | -6 | 2 | 12 | 10 |  |  | 16 | 12 |  |  |
| 13 | 4 | 2 | 0 | -1 | -5 | 1 | 3 | 12 | 8 |  |  | 14 | 16 |  |  |
| 15 | 4 | 2 | 0 | -1 | -3 | -7 | 1 | 14 | 12 |  |  | 16 | 14 |  |  |
| 24 | 1 | 4 | 0 | -8 | -10 | -12 | -16 | 16 | 14 | 12 | 8 | 16 | 14 | 12 | 8 |
| 20 | 2 | 2 | 0 | -4 | -6 | 4 |  | 16 | 14 |  |  | 24 | 16 |  |  |
| 18 | 3 | 2 | 0 | -2 | -4 | 2 | 6 | 16 | 14 |  |  | 20 | 24 |  |  |
| 17 | 4 | 2 | 0 | -1 | 1 | 3 | 7 | 16 | 18 |  |  | 18 | 20 |  |  |
| 19 | 4 | 2 | 0 | -1 | -3 | 1 | 5 | 18 | 16 |  |  | 20 | 24 |  |  |
| 22 | 3 | 2 | 0 | -2 | -4 | -6 | 2 | 20 | 18 |  |  | 24 | 20 |  |  |
| 21 | 4 | 2 | 0 | -1 | -5 | 1 | 3 | 20 | 16 |  |  | 22 | 24 |  |  |
| 23 | 4 | 2 | 0 | -1 | -3 | -7 | 1 | 22 | 20 |  |  | 24 | 22 |  |  |
| 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | 2 | 2 | 0 | -4 | -6 | 4 |  | 24 | 22 |  |  | 32 | 24 |  |  |
| 26 | 3 | 2 | 0 | -2 | -4 | 2 | 6 | 24 | 22 |  |  | 28 | 32 |  |  |
| 25 | 4 | 2 | 0 | -1 | 1 | 3 | 7 | 24 | 26 |  |  | 26 | 28 |  |  |
| 27 | 4 | 2 | 0 | -1 | -3 | 1 | 5 | 26 | 24 |  |  | 28 | 32 |  |  |
| 30 | 3 | 2 | 0 | -2 | -4 | -6 | 2 | 28 | 26 |  |  | 32 | 28 |  |  |
| 29 | 4 | 2 | 0 | -1 | -5 | 1 | 3 | 28 | 24 |  |  | 30 | 32 |  |  |
| 31 | 4 | 2 | 0 | -1 | -3 | -7 | 1 | 30 | 28 |  |  | 32 | 30 |  |  |
| The number of appearances in reference picture list (L0 and L1) *counted once if the ref. POC. number is in both L0 and L1 |  |  |  |  |  |  |  | Position A |  |  |  | Position B |  |  |  |
|  |  |  |  |  |  |  |  | 12 |  |  |  | 16 |  |  |  |

## The effect of Error Propagation

In order to quantify video quality degradation, we created packet dropping tests with encoded bitstreams; i.e., we dropped a picture per every IntraPeriod in Position A for 1st test and a picture per every IntraPeriod in Position B for 2nd test. Specifically, the Position A picture or the Position B picture in the second GOP of an intra period is dropped; that is, for the first intra period in the sequence, this corresponds to dropping POC 10 (Position A) vs. POC 14 (Position B), respectively. To measure the difference of objective video quality in two tests, the HM 6.1 video decoder decodes the damaged bitstream using frame copy. In HM 6.1, if a packet is lost, the decoder copies from the previous picture. This means for position A picture loss, a higher quality picture (POC 8) is copied compared to position B picture loss (where POC 12 is copied). To provide a more fair comparison, we also modified the HM6.1 decoder such that POC 16 instead of POC 12 is copied in the case of Position B loss. The test results for this case are provided below as “modified EC”.

The testing was conducted on Windows 64-bit operating system, and using HM 6.1 decoder [4] (as well as a modified version as explained above) and three test sequences (Traffic, PeopleOnStreet, and ParkScene) from HEVC common test conditions. The PSNR degradation caused by dropping pictures in Position A was less than the PSNR degradation caused by dropping pictures in Position B. The difference in error propagation effect is up to 2.8dB in luma PSNR (PeopleOnStreet, QP22). Fig. 2 shows the corresponding rate-distortion curves, showing that the picture in Position B (purple-colored curve) is more important than the picture in Position A (red-colored curve). The curves in Fig. 2 also show that the different impact on error propagation due to Position A loss and Position B loss largely holds regardless of the different error concealment strategies used.

Table 2. Comparison of Position A drop with Position B drop (RA setting).

|  |  | Drop Position B |  |  |  | Drop Position A |  |  |  | Drop Position B (mod. EC) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | QP | Kbps | Y psnr | U psnr | V psnr | Kbps | Y psnr | U psnr | V psnr | kbps | Y psnr | U psnr | V psnr |
| Traffic | 22 | 12752 | 36.19 | 40.66 | 42.78 | 12751 | 38.18 | 40.94 | 43.39 | 12752 | 36.37 | 40.68 | 42.83 |
|  | 27 | 5231 | 34.96 | 39.27 | 41.42 | 5231 | 36.72 | 39.36 | 41.69 | 5231 | 35.10 | 39.28 | 41.47 |
|  | 32 | 2532 | 33.42 | 38.01 | 40.18 | 2532 | 34.91 | 38.06 | 40.30 | 2532 | 33.54 | 38.03 | 40.20 |
|  | 37 | 1321 | 31.86 | 37.16 | 39.34 | 1321 | 32.77 | 37.19 | 39.44 | 1321 | 31.96 | 37.18 | 39.37 |
| PeopleOnStreet | 22 | 31596 | 33.25 | 43.64 | 43.72 | 31614 | 36.04 | 44.04 | 44.01 | 31596 | 33.57 | 43.65 | 43.68 |
|  | 27 | 15247 | 31.55 | 42.09 | 42.59 | 15257 | 34.15 | 42.43 | 42.88 | 15247 | 31.78 | 42.14 | 42.59 |
|  | 32 | 8050 | 29.97 | 40.79 | 41.54 | 8054 | 32.04 | 40.98 | 41.68 | 8050 | 30.08 | 40.83 | 41.57 |
|  | 37 | 4515 | 28.27 | 39.71 | 40.60 | 4517 | 29.88 | 39.88 | 40.74 | 4515 | 28.40 | 39.75 | 40.63 |
| ParkScene | 22 | 7441 | 37.13 | 41.46 | 43.01 | 7523 | 38.10 | 41.75 | 43.25 | 7441 | 37.06 | 41.45 | 43.00 |
|  | 27 | 3274 | 35.52 | 39.97 | 41.19 | 3304 | 36.15 | 40.05 | 41.21 | 3274 | 35.46 | 39.97 | 41.18 |
|  | 32 | 1527 | 33.33 | 38.49 | 39.73 | 1539 | 33.88 | 38.59 | 39.82 | 1527 | 33.30 | 38.50 | 39.74 |
|  | 37 | 718 | 31.36 | 37.44 | 38.95 | 723 | 31.64 | 37.50 | 39.00 | 718 | 31.33 | 37.44 | 38.95 |

(a)

(b)

(c)

Fig. 2. The RD curves by Position A drop and Position B drop ((a),(b),(c): Drop 1 picture per IntraPeriod)

# Proposed SEI Message Syntax for NAL Unit Priority

# This section proposes a SEI message to indicate the priority of following NAL units.

- Provide the priority information using a new SEI message.

We can set NAL unit priority to SEI message as shown in Table 33 and Table 44. In this table, we added priority_id in SEI payload syntax.

Table 3. SEI payload syntax.

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| ……….. |  |
| else if( payloadType = = 133) |  |
| priority_info( payloadSize ) |  |
| ……….. |  |

Table 4. Definition of a priority_info for SEI.

| priority_info (payloadSize ) { | Descriptor |
| --- | --- |
| priority_id | u(4) |
| Reserved | u(4) |
| } |  |

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, High efficiency video coding (HEVC) text specification draft 8, Document no. JCTVC-J1003, July 2012.

- H. Schwarz, D. Marpe, T. Wiegand, Analysis of hierarchical B pictures and MCTF, IEEE International Conference on Multimedia and Expo, pp. 1929-1932, 2006.

- F. Bossen, Common test conditions and software reference configurations, Document no. JCTVC-H1100, Feb. 2012.

- HEVC Reference Software ver. 6.1 (HM 6.1), url: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-6.1?rev=2385

# Patent rights declaration(s)

InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: DIS Text

Section D.2.23: D.2.23 NALU Priority SEI message semantics This SEI message indicates the priority of following NAL units until the next NALU priority SEI message is present. priority_id specifies a priority identifier for the following NAL units with 4 bits. The values of priority_id shall be in the range of 0 to 15, where 0 indicates the lowest priority, and 15 indicates the highest priority. D.2.23 D.2.24 Reserved SEI message semantics This SEI message consists of data reserved for future backward-compatible use by ITU-T ¦ ISO/IEC. It is a requirement of bitstream conformance that bitstreams shall not contain reserved SEI messages until and unless the use of such messages has been specified by ITU-T ¦ ISO/IEC. Decoders that encounter reserved SEI messages shall discard their content without effect on the decoding process, except as specified in the future by ITU-T ¦ ISO/IEC.