| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0263 |
| --- | --- |

| Title: | Sample scale factor in VUI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This document proposes signaling a sample scale factor to support transitions across two coded video sequences that have the same sample aspect ratio and picture aspect ratio but different picture resolutions.  The proposed sample scale factor is required to support transitions between 1920x1080 and 1280x720 pictures, and transitions between 960x540 and 1920x1080 pictures.  The proposed sample scale factor would allow entering a bitstream at any RAP while maintaining constant the spatial resolution of the sequence of pictures derived from output DPB pictures.

# Introduction

A number of video applications require keeping constant the 2D size and aspect ratio of the spatial span of the picture sequence derived from the output DPB pictures.  Specifically, the physical clock driving the output stage of a receiver must not change from one to the next CVS to avoid instigating disruptions.  A sample scale factor is proposed to allow picture resolution changes across coded video sequences (CVSes) that have the same sample aspect ratio and picture aspect ratio but different picture resolutions, such as transitions between 1280x720 and 1920x1080 pictures.

When 1920x1080 is the main picture resolution in a bitstream, CVSes with 1920x1080 pictures have aspect_ratio_idc = 1 (i.e., square pixels) and an implied sample scale factor = 1, whereas CVSes with 1280x720 pictures are signalled with  a sample scale factor equal to three over two (i.e., 1.5).

If 1280x720 is the main picture resolution, CVSes with 1920x1080 pictures are signalled with a sample scale factor equal to two over three (i.e., 0.6667).

CVSes with quarter-sized pictures, such as 960x540, are signalled with sample scale factor equal to 2.

In the following, sample_scale_factor_flag = 1 specifies the presence of sample_scale_factor_index which is an index to a corresponding sample scale factor value in Table E.X.   These sample scale factors in the table are relevant to picture resolution transitions in commercial insertion applications.

Relevant VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| if( aspect_ratio_info_present_flag ) { |  |
| aspect_ratio_idc | u(8) |
| if( aspect_ratio_idc = = Extended_SAR ) { |  |
| sar_width | u(16) |
| sar_height | u(16) |
| } |  |
| sample_scale_factor_flag | u(1) |
| if(sample_scale_factor_flag) |  |
| sample_scale_factor_index | u(3) |
| } |  |
| } |  |

Table E.X -   Sample Scale Factor Table

| sample_scale_factor_index | sample_scale_factor |
| --- | --- |
| 0 | 1.0 |
| 1 | 1.5 (3/2) |
| 2 | 0.667 (2/3) |
| 3 | 2.0 |
| 4-7 | Reserved |

Proposed Semantics

sample_scale_factor_flag equal to 1 specifies that sample_scale_factor_index is present. sample_scale_factor_flag equal to 0 specifies that sample_scale_factor_index is not present. When sample_scale_factor_flag is not present, its value is inferred to be equal to 0.

sample_scale_factor_index specifies a sample scale factor in Table E.X.  When sample_scale_factor_index is not present, its value is inferred to be equal to 0.

Alternatively, it is proposed to use aspect_ratio_idc 17, 18, and 19 to signal the following square samples with a sample scale factor not equal to one:

| aspect_ratio_idc | Interpretation of aspect_ratio_idc (Informative - Examples of Use) |
| --- | --- |
| 17 | 1.5 : 1.5 sample aspect ratio; square sample with a scale factor equal to 3/2 (1280x720 16:9 frame output as 1920x1080 without horizontal overscan) |
| 18 | 0.667 : 0.667 sample aspect ratio; square sample with a scale factor equal to 2/3 (1920x1080 16:9 frame output as 1280x720 without horizontal overscan) |
| 19 | 2:2 sample aspect ratio; square sample with a scale factor equal to 2 (960x540 16:9 frame output as 1920x1080 without horizontal overscan) |

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).