| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0264 |
| --- | --- |

| Title: | Hierarchical inter-layer prediction in multi-loop scalable extension of HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1861 madhukar@ti.com +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The multi-loop scalable HEVC is asserted to be easier to extend from single-layer HEVC from an architecture point of view when compared to single-loop scalable coding. However, multi-loop coding increases memory bandwidth at decoder side. In this contribution, it is proposed to signal in NAL unit header whether a picture is used as a reference for next layers and as a temporal reference for other pictures that are referred to by next layers. In intermediate layers, the pictures not being used as references for next layers can be discarded without decoding, which reduces memory bandwidth in decoder. Moreover, only the pictures being used as temporal references for other pictures that are referred by next layer need to be stored, which reduces DPB size as well in decoder. When inter-layer prediction is enabled for the half number of pictures (instead of being enabled for all pictures) in enhancement layer, it is possible to save memory bandwidth necessary for temporal prediction and inter-layer prediction by half. The proposed method is implemented on top of JCTVC-K0038. It is reportedly shown that BL + EL (actual) BD-rate losses are only 1.4%, 2.2% and 1.7% for RA 2x, 1.5x and SNR scalabilities, respectively, when compared to JCTVC-K0038. It is reportedly shown that the proposed method still provides BL + EL (actual) BD-rate gains of 15.3%, 23.6% and 19.6% for RA 2x, 1.5x and SNR scalabilities, respectively, over Simulcast.

# Introduction

The multi-loop approach is effective yet simple for the scalable extension of HEVC from the architecture point of view. At decoder side, all intermediate layers are fully reconstructed for inter-layer prediction of next layers to decode a target layer. Because of strong correlation between a base layer and enhancement layers, the coding efficiency can be significantly improved with a simple inter-layer prediction method [1]. However, it increases memory bandwidth at decoder side since all intermediate layers are reconstructed via motion compensation. To provide a way to trade off coding efficiency and memory bandwidth in multi-loop HEVC scalable extension, it is proposed to signal two flags in NAL unit header to indicate whether a picture is used as a reference for next layers and a temporal reference for other pictures that referred by next layers, respectively. In intermediate layers, the pictures not being used as reference layer can be discarded in decoder and only the pictures being used as temporal references for other pictures need to be stored at decoder side. In this contribution, this signaling is done based on hierarchical structure. Experimental results confirm that the proposed method provides varying BD-rate gains depending on the memory bandwidth consumed.

# Hierarchical inter-layer prediction

Figure 1 illustrates prediction structure in base layer for CTC Random Access case, where the dashed line shows reference picture. Please note that, for temporal level 2 and 3, reference pictures are shown only for one picture for simplicity. Assuming that inter-layer prediction is enabled for all access units, all pictures in base layer should be fully reconstructed in decoder.

Figure 1. Prediction structure in base layer for CTC Random Access case (GOP size: 8)

In this contribution, it is proposed to signal in NAL unit header two flags; nal_ilp_flag, which indicates whether a NAL unit is used as a reference for next layers, and nal_ilp_ref_flag, which indicates whether a NAL units is used as a temporal reference for other NAL units having nal_ilp_flag equal to 1. Table 1 shows modified NAL unit header syntax on top of one in [1].

Table 1. NAL unit header syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| temporal_id | u(3) |
| nal_ilp_ref_flag | u(1) |
| nal_ilp_flag | u(1) |
| dependency_id | u(3) |
| quality_id | u(4) |
| reserved_one_4bits | u(4) |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

Figure 2 shows the case where the pictures in 2 lowest levels (i.e. level 0 and level 1) are used for inter-layer prediction of enhancement layer by the proposed syntax elements. These pictures in level 0 and level 1 having nal_ilp_flag equal to 1 are named ILP picture. Moreover, the pictures in level 0 are used as temporal references for other ILP pictures in the same layer. These pictures in level 0 having nal_ilp_ref_flag equal to 1 are named ILP-REF picture. The constraint that ILP picture should not have non-ILP-REF picture as reference for temporal prediction is imposed. With the proposed syntax elements and the constraint, decoder can reduce memory bandwidth by discarding non-ILP picture without decoding. In Figure 2, the pictures having both nal_ilp_ref_flag and nal_ilp_flag equal to 1 are colored with red and the pictures having nal_ilp_ref_flag equal to 1 and nal_ilp_flag equal to 0 are colored with blue. It should be noted that the dotted pictures can be discarded in decoder without decoding.

It should be noted that DPB management is not affected by nal_ilp_ref_flag in encoder. DPB is managed by nal_ref_flag and RPS without any change. However, for ILP pictures, initial reference picture list is constructed considering nal_ilp_ref_flag of the pictures in DPB. For non-ILP pictures (i.e. pictures in level 2 and 3 in Figure 2), there is no change in reference picture list initialization.

Figure 3 illustrates the another example where the pictures in level 0, 1 and 2 are ILP pictures and the pictures in level 0 and 1 are ILP-REF pictures.

Figure 2. Prediction structure in base layer for CTC Random Access case (GOP size: 8) with 2-level ILP. Pictures in level 0 and 1 are used for inter-layer prediction of enhancement layer. And pictures in level 0 are used as temporal references for other pictures with nal_ilp_flag equal to 1. Dotted pictures in level 2 and level 3 can be discarded in decoder without decoding.

Figure 3. Prediction structure in base layer for CTC Random Access case (GOP size: 8) with 3-level ILP. Pictures in level 0, 1 and 2 are used for inter-layer prediction of enhancement layer. And pictures in level 0 and 1 are used as temporal references for other pictures with nal_ilp_flag equal to 1. Dotted pictures in Level 3 3 can be discarded in decoder without decoding.

Finally, the required DBP size for base layer in decoder is 2 and 3 for Figure 2 and Figure 3, respectively. If the DPB sizes for intermediate layers are signaled in high-level header such as VPS, it is also possible to reduce DPB size in decoder.

# Experimental results

The proposed method are tested using CTC for RA 2x, 1.5x and SNR scalability in Category 1 [1]. All-level ILP without the proposed syntax element proposed in [1], 2-level and 3-level ILP using the proposed syntax elements are compared with anchor (i.e. Simulcast). For all test conditions, the same BL and EL QPs as in anchor were used.

Table 2 shows BD-rate gain over Simulcast when inter-layer prediction is used for all pictures in enhancement layer. Table 3 shows BD-rate gain over Simulcast when inter-layer prediction is used for the pictures in level 0 and 1 in enhancement layer (i.e. 2-level ILP). It should be noted that nal_ilp_flag is set to 1 only for the pictures in level 0 and 1 in base layer as illustrated in Figure 2. Table 4 shows BD-rate gain over Simulcast when inter-layer prediction is used for the pictures in level 0, 1 and 2 in enhancement layer (i.e. 3-level ILP).

With 2-level ILP, luma BD-rate gain over Simulcast with EL-only actual rate decreased by 6.1%, 10.3% and 7.1% for RA 2x, 1.5x and SNR, respectively, and luma BD-rate gain over Simulcast with BL + EL actual rate decreased by 3.5%, 5.5% and 4.0%, respectively. However, since only 2 pictures out of 8 pictures in GOP needs to be decoded in base layer, memory bandwidth necessary for temporal prediction and inter-layer prediction also decreases roughly by 75%.

With 3-level ILP, luma BD-rate gain over Simulcast with EL-only actual rate decreased only by 2.4%, 3.8% and 3.0% for RA 2x, 1.5x and SNR, respectively, and luma BD-rate gain over Simulcast with BL + EL actual rate decreased only by 1.4%, 2.2% and 1.7% for RA 2x, 1.5x and SNR, respectively. Moreover, since only 4 pictures out of 8 pictures in GOP needs to be decoded in base layer, memory bandwidth necessary for temporal prediction and inter-layer prediction also decreases roughly by half.

Table 2. BD-rate gain over Simulcast with EL-only actual rate and BL + EL actual rate for RA 2x, when ILP is used for all pictures in enhancement layer.

- EL-only actual rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -30.3% | -16.2% | -16.6% |  |  |  |
| Class B | -24.3% | -14.7% | -12.7% | -46.6% | -36.1% | -33.9% |
| Overall | -27.3% | -15.5% | -14.6% | -46.6% | -36.1% | -33.9% |
|  | -27.3% | -15.7% | -14.8% | -46.9% | -36.5% | -34.5% |

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -38.2% | -26.7% | -26.4% |
| Class B | -32.6% | -20.2% | -15.5% |
| Overall | -35.4% | -23.4% | -21.0% |
|  | -35.4% | -24.1% | -21.7% |

- BL + EL actual rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -18.3% | -6.6% | -7.0% |  |  |  |
| Class B | -15.1% | -7.1% | -5.3% | -25.8% | -17.2% | -15.4% |
| Overall | -16.7% | -6.9% | -6.1% | -25.8% | -17.2% | -15.4% |
|  | -16.7% | -7.0% | -6.2% | -25.8% | -17.3% | -15.5% |

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -22.7% | -13.6% | -13.2% |
| Class B | -19.8% | -9.5% | -5.7% |
| Overall | -21.3% | -11.5% | -9.5% |
|  | -21.3% | -11.6% | -9.5% |

Table 3. BD-rate gain over Simulcast with EL-only actual rate and BL + EL actual rate for RA 1.5x, when ILP is used for the pictures in level 0 and 1 in enhancement layer (2-level ILP).

- EL-only actual rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -23.2% | -12.9% | -12.8% |  |  |  |
| Class B | -19.1% | -12.0% | -10.5% | -36.3% | -27.7% | -26.1% |
| Overall | -21.2% | -12.4% | -11.7% | -36.3% | -27.7% | -26.1% |
|  | -21.2% | -12.5% | -11.7% | -36.3% | -27.8% | -26.2% |

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -29.6% | -22.0% | -21.5% |
| Class B | -26.7% | -17.6% | -14.3% |
| Overall | -28.1% | -19.8% | -17.9% |
|  | -28.2% | -19.8% | -17.9% |

- BL + EL actual rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -14.4% | -5.9% | -5.8% |  |  |  |
| Class B | -12.1% | -6.1% | -4.9% | -20.3% | -13.6% | -12.2% |
| Overall | -13.2% | -6.0% | -5.4% | -20.3% | -13.6% | -12.2% |
|  | -13.2% | -6.1% | -5.4% | -20.3% | -13.6% | -12.3% |

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -18.0% | -11.9% | -11.3% |
| Class B | -16.5% | -8.9% | -6.2% |
| Overall | -17.3% | -10.4% | -8.8% |
|  | -17.3% | -10.3% | -8.7% |

Table 4. BD-rate gain over Simulcast with EL-only actual rate and BL + EL actual rate for RA 1.5x, when ILP is used for the pictures in level 0, 1 and 2 in enhancement layer (3-level ILP).

- EL-only actual rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -27.5% | -14.7% | -14.9% |  |  |  |
| Class B | -22.2% | -14.1% | -12.3% | -42.8% | -33.2% | -31.3% |
| Overall | -24.9% | -14.4% | -13.6% | -42.8% | -33.2% | -31.3% |
|  | -24.8% | -14.6% | -13.7% | -42.8% | -33.4% | -31.6% |

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -34.6% | -24.9% | -24.4% |
| Class B | -30.2% | -19.6% | -15.4% |
| Overall | -32.4% | -22.2% | -19.9% |
|  | -32.4% | -22.4% | -20.2% |

- BL + EL actual rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -16.8% | -6.2% | -6.4% |  |  |  |
| Class B | -13.8% | -7.1% | -5.6% | -23.6% | -16.1% | -14.5% |
| Overall | -15.3% | -6.7% | -6.0% | -23.6% | -16.1% | -14.5% |
|  | -15.3% | -6.7% | -6.1% | -23.6% | -16.1% | -14.5% |

|  | Random Access HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -20.8% | -13.0% | -12.5% |
| Class B | -18.4% | -9.5% | -6.1% |
| Overall | -19.6% | -11.3% | -9.3% |
|  | -19.6% | -11.2% | -9.3% |

# Conclusion

For multi-loop HEVC scalable extension, it is proposed to signal two flags in NAL unit header to indicate whether a picture is used as a reference for next layers and a temporal reference for other pictures that referred by next layers, respectively. It was shown that trade-off between memory bandwidth and coding efficiency can be easily made using the proposed method. It should be also emphasized that the DBP size in intermediate layers can be reduced if the required DPB sizes for intermediated layers are signaled in high-level header such as VPS.

# References

- D.-K. Kwon, M. Budagavi and M. Zhou, “Description of scalable video coding technology proposal by Texas Instruments Inc.”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-K0038, October 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).