| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0265 |
| --- | --- |

| Title: | About an order of SAO syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina, A. Alshin, J.H. Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This contribution is about changing the order of SAO syntax elements which allows reduction for conditions check both in specification text and s/w. There is no performance change due to suggested modification.

# Introduction

During the 10th JCTVC meeting in Stockholm several SAO syntax modifications were adopted at the same time. Among them independent SAO on/off slice level flag for Luma and Chroma [1], SAO merge flag sharing between all 3 color components [2], SAO type sharing between 2 Chroma color components [3] and elegant SAO type signalling with fixed length code [4].

According to the SAO specification for each CTB we need to encode 1 merge left flag, 1 merge up flag, 2 SAO types, 2 EO classes (edge offset types) or 3 band positions and 4 offsets for each 3 color components. As a result SAO syntax description in specification text has a lot of condition checks.

In this contribution we suggest to change the order of SAO syntax elements and so reduce the number of condition checks.

# Suggested modification explanation

Current specification for SAO syntax and suggested changes are summarized in Table 1. Before suggested modification SAO types (sao_type_idx_luma and sao_type_idx_chroma) and EO classes (sao_eo_class_luma and sao_eo_class_chroma) parsing was inside loop across color components (for( cIdx = 0; cIdx < 3; cIdx++ )…). We have placed parsing SAO types and EO classes parsing before this loop.

Removed specification text (highlighted in grey) contains 8 3 = 24 condition checks.

New text (highlighted in yellow) has 4 condition checks. In total 20 conditions checks are removed by suggested modification.

Table 1. Sample adaptive offset syntax

| sao( rx, ry ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = ( CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbsY] ) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbsY ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| if( slice_sao_luma_flag) { |  |
| sao_type_idx_luma | ae(v) |
| if( SaoTypeIdx[ 0 ][ rx ][ ry ] == 2 ) { |  |
| sao_eo_class_luma | ae(v) |
| } |  |
| } |  |
| if( slice_sao_chroma_flag) { |  |
| sao_type_idx_chroma | ae(v) |
| if( SaoTypeIdx[ 1 ][ rx ][ ry ] == 2 ) { |  |
| sao_eo_class_chroma | ae(v) |
| } |  |
| } |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( ( slice_sao_luma_flag && cIdx = = 0 ) ¦ ¦ ( slice_sao_chroma_flag && cIdx > 0 ) ) { |  |
| if( cIdx = = 0 ) |  |
| sao_type_idx_luma | ae(v) |
| if( cIdx = = 1 ) |  |
| sao_type_idx_chroma | ae(v) |
| if( SaoTypeIdx[ cIdx ][ rx ][ ry ] != 0 ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( SaoTypeIdx[ cIdx ][ rx ][ ry ] = = 1 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } else { |  |
| if( cIdx = = 0 ) |  |
| sao_eo_class_luma | ae(v) |
| if( cIdx = = 1 ) |  |
| sao_eo_class_chroma | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Performance impact

Suggested modification has no performance impact since the same amount of syntax elements is encoded in different order. The summary for performance test results is shown in Table 2. In this test we used common test conditions defined in [5]. Variation of encoding/decoding time is under noise level.

Table 2. Performance test summary for proposed modification.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 98% |  |  |

Author would like to thank Motorola for the cross-check.

# Conclusion

Samsung suggests putting SAO type and EO class parsing before the color components loop. This modification simplifies SAO parameters parsing since in total 20 conditions check per each CTB will be removed without any harm for the performance.

# References

- Minhua Zhou, “AHG6: Independent luma and chroma SAO on/off control at slice level,” Joint Collaborative Team on Video Coding, JCTVC-J0087, July, 2012

- Koohyar Minoo, David Baylon (Motorola Mobility), “AHG5, AHG6: Coding of SAO merge left and merge up flags,” Joint Collaborative Team on Video Coding, JCTVC-J0355, July, 2012

- E. Alshina, A. Alshin, J.H. Park, G. Laroche, C. Gisquet, and P. Onno, “AHG6: On SAO type sharing between U and V components,” Joint Collaborative Team on Video Coding, JCTVC-J0045, July, 2012

- J. Xu, A. Tabatabai (Sony), “AHG6: on SAO signalling,” Joint Collaborative Team on Video Coding, JCTVC-J0268, July, 2012.

- F. Bossen “HM 8 Common Test Conditions and Software Reference Configurations”, Joint Collaborative Team on Video Coding, JCTVC-J1100, July, 2012

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).