| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0267 |
| --- | --- |

| Title: | CE1: Cross-check for test 6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

Test 6 in de-blocking filter CE is verified in this report. Objective results match with proponent’s data. Subjective quality was investigated during this cross-check. We have found no critical effect on visual quality from disabling Intra boundary smoothing for 32x32 PUs.

# Introduction

Verified test is based on JCTVC-10th (Stockholm) meeting contribution from Ericsson [1]. That contribution included several modifications:

- Intra Boundary Smoothing restriction (for 3232 Intra PUs)

- Strong Filter decision and strong filter coefficients modification

- Bs modification on CU boundary

CE1-test6 studies only the Intra Boundary Smoothing restriction.

At the same time Samsung is a cross-checker for CE1 –test 2 which includes both 3 modification suggested by Ericsson in [1]. Visual quality comparison between anchor (HM8.0), verified here CE1-test 6 and CE1-test 2, CE1-test 4 is included into [2].

# Verification tests results

Objective performance was tested using common test conditions [3] using code provide by proponent. Provided s/w implementation matches with the description in the contribution document [4]. Performance test results under common test conditions are summarized in Table 1.

Table 1. Performance test results summary (common test conditions).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | -0.3% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.2% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | 0.3% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class D | -0.1% | 0.3% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class E | -0.1% | -0.1% | 0.3% | -0.1% | -0.2% | 0.0% |
| Class F | 0.1% | 0.1% | -0.1% | -0.1% | -0.2% | -1.0% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.2% | 0.0% | -0.3% | -0.1% |
| Class D | 0.0% | 0.5% | 0.2% | 0.0% | -0.4% | 0.3% |
| Class E | 0.0% | 0.3% | -0.7% | 0.0% | -0.7% | -0.3% |
| Class F | 0.0% | 0.7% | 0.9% | 0.0% | 0.1% | -0.5% |
| Overall | 0.0% | 0.3% | 0.2% | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Additional test results

According to CE1 description [4] additional test sequences were used for subjective visual quality evaluation. We would like to report objective performance for additional test sequences in Table 2.

Common test conditions (ctc) [5] were used in this test except QP settings. According to [4] QP=32,37 and 41 need to be tested. Additionally QP=27 was tested to estimate BD-rate gain.

Table 2. Objective performance for additional video sequences.

|  | RA-main |  |  | LDB-main |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| DucksTakeOff | -0.1% | 0.0% | 0.2% | 0.0% | 0.1% | -0.3% |
| ChinaSpeed | 0.0% | 0.0% | 0.2% | 0.1% | -0.1% | 0.0% |
| RedKayak | 0.0% | 0.2% | 0.2% | 0.0% | -0.1% | 0.2% |
| WestWindsEasy | 0.0% | 0.3% | -0.8% | 0.0% | -0.2% | 2.3% |
| RiverBed | -0.1% | -0.1% | 0.1% | 0.0% | -0.3% | -0.2% |

According to these test data verified algorithm shows noise level objective performance for additional test sequences and QP=27,32,37,41.

All objective performance data reported by tester in [4] for additional test sequences matched with our simulations.

# Visual test results.

According to [4] Visual quality should be evaluated for following test cases:

- DucksTakeOff, 25fps, RA-main, QP=32,37,41

- RedKayak,570fr, RA-main, QP=32,37,41

- Riverbed, LDB-main, QP=32,37,41

- WestWindEasy, LDB-main, QP=32,37,41

- ChinaSpeed, LDB-main, QP=32,37,41

Our cross-check for CE1-test 2 [] contains visual quality discussion for CE1-test6 as well.

In general we have found no difference between anchor and CE1-test 6. We have to note that Sao was enabled in all tests in CE1. Visual quality problems due to Intra boundary smoothing were reported in test where SAO was disabled and artefacts were visible if only Luma component is displayed (Cr=Cb=128). We have not performed such kind of tests (not requested in CE1). So CE1 test conditions were not designed for potential artefact coursed by Intra boundary smoothing observation.

# Conclusion

Samsung confirms objective performance reported by Sony in CE1 test 6. We have found no examples when verified algorithm harms or helps visual quality.

# References

- A. Norkin, “AHG6: Reduction of block artifacts in HEVC for large blocks”, Joint Collaborative Team on Video Coding, JCTVC-J0286, July, 2012.

- E. Alshina “CE1: Cross-check for test 2”, Joint Collaborative Team on Video Coding, JCTVC-K0266, October, 2012

- F. Bossen “HM 8 Common Test Conditions and Software Reference Configurations”, Joint Collaborative Team on Video Coding, JCTVC-J1100, July, 2012.

- S. Lu, O. Nakagami, T. Suzuki (Sony), “CE1: On deblocking filter”, Joint Collaborative Team on Video Coding, JCTVC-J0149, October, 2012

- A. Norkin, T. Suzuki, M. Budagavi, G. Van der Auwera (CE coordinators) “Description of Core Experiment 1: Deblocking filter”, Joint Collaborative Team on Video Coding, JCTVC-J1101, July, 2012

- E. Alshina, “CE1: Cross-check for test 2”, Joint Collaborative Team on Video Coding, JCTVC-J0265, October, 2012