| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0268 |
| --- | --- |

| Title: | AHG6: SAO offset coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | In Suk Chong, Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 ichong@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In HM8.0, SAO offsets are coded using truncated unary binarization with all bins being bypass-coded. The maximum offset value depends on the internal bit-depth: ranges 0~7/0~15/0~31 are allowed for 8/9/10 or higher bit-depth, respectively. Therefore, the worst-case number of bypass-coded bins per offset is 7/15/31 bins with TU binarization. Considering that a total of 12 offsets are sent per LCU, the worst-case is allegedly too large in the current design. We propose a different binarization to reduce the worst number of bins to 5/7/9 for bit-depth 8/9/10 or higher, respectively, coding all bins in bypass and without coding performance loss on average.

# Technical Description

In the current HM8.0, sao_offset is coded with truncated unary binarization with all bins bypass-coded. In this document, it is proposed to reduce those worst-case number of bins using the binarization TU+FL (shown in table 1 for the cases 8 bit, 9 bit, and 10 bit-depth and above), which was already proposed in [1].

## 8 bit bit-depth case, maxSymbol = 7

| Offset value | Prefix | Suffix | Suffix range |
| --- | --- | --- | --- |
|  | Truncated unary | Fixed Length |  |
| 0 | 0 | - | - |
| 1 | 10 | - | - |
| 2-3 | 110 | X | 0-1 |
| 4-7 | 111 | XX | 0-3 |

## 9 bit bit-depth case, maxSymbol = 15

| Offset value | Prefix | Suffix | Suffix range |
| --- | --- | --- | --- |
|  | Truncated unary | Fixed Length |  |
| 0 | 0 | - | - |
| 1 | 10 | - | - |
| 2-3 | 110 | X | 0-1 |
| 4-7 | 1110 | XX | 0-3 |
| 8-15 | 1111 | XXX | 0-7 |

## 10 bit or higher bit-depth cases, maxSymbol = 31

| Offset value | Prefix | Suffix | Suffix range |
| --- | --- | --- | --- |
|  | Truncated unary | Fixed Length |  |
| 0 | 0 | - | - |
| 1 | 10 | - | - |
| 2-3 | 110 | X | 0-1 |
| 4-7 | 1110 | XX | 0-3 |
| 8-15 | 11110 | XXX | 0-7 |
| 16-31 | 11111 | XXXX | 0-15 |

Table 1. Proposed SAO offset binarization

In the proposed SAO offset binarization, the worst-case number of bins are 5/7/9 for 8/9/>=10 bit-depth, repectively. Table 2 summarizes the worst-case scenario.

| Worst-case bypass bins |  |  |
| --- | --- | --- |
| Bit-depth | HM8.0 | Proposal |
| 8 | 7 | 5 |
| 9 | 15 | 7 |
| 10 and higher | 31 | 9 |

Table 2. Comparison of the worst-case number of bins

# Simulation results

The proposed methods are implemented in HM8.0 and simulated under the common test conditions. Performance shows no loss for Main configuration and 0.1% gain for HE10 configurations. Source code is included in this package.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.0% | -0.1% | -0.2% | -0.2% |
| Enc Time[%] | 102% |  |  | 90% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.2% |
| Class B | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | -0.4% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.1% | -0.1% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.2% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.4% | -0.1% | -0.1% | 0.1% | -0.4% |
| Class E | 0.0% | 0.1% | 0.7% | 0.0% | 0.3% | 0.3% |
| Overall | 0.0% | 0.2% | 0.2% | -0.1% | 0.1% | -0.1% |
|  | 0.0% | 0.2% | 0.1% | -0.1% | 0.1% | -0.1% |
| Class F | 0.1% | 0.2% | -0.3% | -0.3% | -0.2% | -0.8% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 3. results for proposed binarization

# Conclusions

A modification in the SAO offset coding significantly decreases the worst-case number of bins in that syntax element without coding loss. We propose to adopt the proposal into HEVC.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- I. S. Chong, J. Sole, M. Karczewicz, “AHG6/AHG5: SAO offset coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0106, July, 2012.