| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0269 |
| --- | --- |

| Title: | Non-CE1: Suppression of blocking artifacts at large TU boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is reported that blocking artifacts at large TU boundaries can be suppressed by applying the largest values of beta_offset_div2 and tc_offset_div2 to the maximum TU boundaries and disabling intra-prediction boundary filtering at 32x32 blocks. To apply beta and tc offsets only to the largest TU boundary, it is proposed to signal in PPS and slice header a new syntax element max_tu_offset_flag to specify whether the signaled beta and tc offsets are applied to all block boundaries or maximum TU boundaries only. With beta_offset_div2 and tc_offset_div2 being equal to 6, the blocking artifacts at large TU boundaries are reported to be suppressed by the proposed method. However, when compared to HM-8.0 anchor, it is reported that BD-rate loss is 0.5% ~ 1.0% (AI-Main: 0.6%, RA-Main: 0.5%, LB-Main: 0.8%, LP-Main: 0.5%, AI-HE10: 0.6%, RA-HE10: 0.5%, LB-HE10: 1.0%, and LP-HE10: 0.6%) for the maximum TU size 32.

# Introduction

It was reported in [1] and [2] that blocking artifacts exist at large TU boundaries even after deblocking filtering. It is proposed to combine one of the suggested changes in [3] with the beta and tc offset signaling method for maximum TU boundaries in this contribution. It is shown that the proposed method suppresses large TU artifacts at the cost of marginal BD-rate loss.

# Proposed method

In [3], intra-prediction boundary filtering in DC, horizontal and vertical modes sometimes was found to result in blocking artifacts especially at large block boundaries and therefore intra prediction boundary filtering was disabled at 32x32 intra blocks. In this contribution it is proposed to similarly disable intra-prediction boundary filtering. In addition, it is proposed to signal a flag so that beta and tc offsets can be applied only to maximum TU boundaries. For this purpose, a new syntax element max_tu_offset_flag is introduced in PPS and slice header as shown in Table 1 and Table 2, respectively.

Table 1. Picture parameter set syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| max_tu_offset_flag | u(1) |
| } |  |
| } |  |
| …. |  |
| } |  |

Table 2. Slice header syntax

| Slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| max_tu_offset_flag | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

max_tu_offset_flag equal to 1 specifies that the signaled beta_offset_div2 and tc_offset_div2 are applied only to maximum TU boundaries. That is, beta_offset_div2 and tc_offset_div2 are set to 0 for other than maximum TU boundaries. Otherwise, these offsets are applied to all block boundaries. The blocking artifacts at large TU boundaries can be removed by using positive offset values and max_tu_offset_flag equal to 1.

# Experimental Results

The proposed method is implemented in HM-8.0 reference software and its objective quality is evaluated based on the common test condition [5]. The visual qualities are also compared using Type1 and Typ2 sequences suggested in [4]. For the proposed method, deblocking filter configurations were set as follows:

DeblockingFilterControlPresent  : 1           # Dbl control params present (0=not present, 1=present)

LoopFilterOffsetInPPS         : 1           # Dbl params in PPS or SliceHeader (0=SliceHeader, 1=PPS)

LoopFilterDisable            : 0           # Disable deblocking filter in slice header (0=Filter, 1=No Filter)

LoopFilterBetaOffset_div2     : 6           # Range: -6 ~ 6

LoopFilterTcOffset_div2       : 6           # Range: -6 ~ 6

MaxTuOffsetFlag             : 1           # Maximum TU offset flag

3.1 Objective Quality Evaluation

For the maximum TU size 32x32, the BD-rate results by the proposed method compared to HM-8.0 anchor are listed in Table 3. For comparison, the BD-rate results by the deblocking filter with beta_offset_div2 and tc_offset_div2 equal to 6 are listed in Table 4. It can be observed that the proposed method decreases BD-rate only by 0.5% ~ 1.0% on average while applying maximum values of beta and tc offsets result in BD-rate loss of 4.0 % ~ 6.0 %. Both methods are similar in that they suppress blocking artifacts by filtering more block boundaries. However, considering that blocking artifacts are more visible and BD-rate difference, the proposed method should be preferred.

Table 3. BD-rate results by the proposed method compared to HM-8.0 anchor. The maximum TU size is 32x32.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.4% | 0.8% | 1.0% | 1.5% | 0.9% | 1.0% |
| Class B | 0.6% | 0.4% | 0.4% | 0.7% | 0.4% | 0.4% |
| Class C | 0.2% | 0.3% | 0.2% | 0.2% | 0.3% | 0.2% |
| Class D | 0.2% | 0.1% | 0.1% | 0.2% | 0.2% | 0.1% |
| Class E | 0.6% | 0.1% | 0.0% | 0.6% | 0.2% | 0.0% |
| Overall | 0.6% | 0.4% | 0.4% | 0.6% | 0.4% | 0.4% |
|  | 0.6% | 0.4% | 0.4% | 0.6% | 0.4% | 0.4% |
| Class F | 0.7% | 0.3% | 0.4% | 0.8% | 0.2% | 0.6% |
| Enc Time[%] | 105% |  |  | 101% |  |  |
| Dec Time[%] | 108% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.7% | 0.2% | 0.4% | 0.6% | 0.3% | 0.3% |
| Class B | 0.5% | 0.3% | 0.4% | 0.6% | 0.3% | 0.5% |
| Class C | 0.5% | 0.3% | 0.4% | 0.6% | 0.2% | 0.1% |
| Class D | 0.2% | -0.1% | 0.2% | 0.2% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.5% | 0.2% | 0.4% | 0.5% | 0.2% | 0.2% |
|  | 0.5% | 0.2% | 0.4% | 0.5% | 0.2% | 0.2% |
| Class F | 0.7% | 0.1% | 0.3% | 0.8% | 0.5% | 0.6% |
| Enc Time[%] | 104% |  |  | 106% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.7% | 0.5% | 0.3% | 0.9% | 0.5% | 0.8% |
| Class C | 0.8% | 0.0% | 0.1% | 0.9% | 0.2% | 0.3% |
| Class D | 0.3% | 0.2% | -0.3% | 0.3% | -0.1% | -0.2% |
| Class E | 1.5% | 1.0% | 1.2% | 2.0% | 0.7% | 0.9% |
| Overall | 0.8% | 0.4% | 0.3% | 1.0% | 0.3% | 0.4% |
|  | 0.8% | 0.4% | 0.3% | 1.0% | 0.3% | 0.4% |
| Class F | 1.0% | 0.6% | 0.4% | 0.9% | 0.6% | -0.5% |
| Enc Time[%] | 97% |  |  | 103% |  |  |
| Dec Time[%] | 97% |  |  | 104% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.3% | 0.6% | 0.3% | 0.3% | 0.1% |
| Class C | 0.6% | 0.1% | 0.2% | 0.7% | 0.0% | 0.1% |
| Class D | 0.2% | 0.3% | -0.2% | 0.3% | -0.5% | 0.5% |
| Class E | 1.1% | 1.1% | -0.2% | 1.6% | 0.2% | 0.5% |
| Overall | 0.5% | 0.4% | 0.1% | 0.6% | 0.0% | 0.3% |
|  | 0.5% | 0.4% | 0.2% | 0.6% | 0.0% | 0.3% |
| Class F | 0.8% | 0.7% | 1.1% | 0.9% | 0.3% | 0.1% |
| Enc Time[%] | 105% |  |  | 110% |  |  |
| Dec Time[%] | 110% |  |  | 110% |  |  |

Table 4. BD-rate results of the deblocking filter with beta_offset_div2 and tc_offset_div2 equal to 6 compared to HM-8.0 anchor. The maximum TU size is 32x32.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 4.1% | 2.9% | 2.9% | 4.1% | 2.9% | 2.9% |
| Class B | 5.3% | 3.7% | 3.6% | 5.3% | 3.6% | 3.6% |
| Class C | 4.5% | 5.2% | 5.1% | 4.6% | 5.2% | 5.0% |
| Class D | 4.2% | 4.7% | 5.3% | 4.2% | 4.7% | 5.3% |
| Class E | 3.9% | 2.0% | 0.6% | 3.9% | 2.0% | 0.6% |
| Overall | 4.5% | 3.8% | 3.7% | 4.5% | 3.8% | 3.6% |
|  | 4.4% | 3.8% | 3.7% | 4.4% | 3.8% | 3.7% |
| Class F | 3.9% | 6.8% | 8.2% | 3.9% | 6.8% | 8.2% |
| Enc Time[%] | 105% |  |  | 99% |  |  |
| Dec Time[%] | 107% |  |  | 98% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 2.5% | 1.9% | 2.0% | 2.6% | 2.0% | 2.1% |
| Class B | 4.8% | 2.9% | 3.0% | 5.0% | 2.9% | 3.1% |
| Class C | 4.3% | 4.4% | 4.4% | 4.4% | 4.5% | 4.3% |
| Class D | 4.5% | 3.9% | 4.4% | 4.5% | 4.0% | 4.5% |
| Class E |  |  |  |  |  |  |
| Overall | 4.1% | 3.3% | 3.4% | 4.2% | 3.3% | 3.4% |
|  | 4.1% | 3.3% | 3.4% | 4.2% | 3.3% | 3.4% |
| Class F | 5.5% | 7.8% | 9.2% | 5.8% | 8.1% | 9.1% |
| Enc Time[%] | 105% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 5.4% | 1.8% | 1.7% | 5.8% | 1.9% | 2.4% |
| Class C | 4.6% | 1.7% | 1.9% | 4.7% | 2.1% | 2.3% |
| Class D | 5.3% | 2.2% | 1.6% | 5.5% | 1.9% | 2.2% |
| Class E | 7.7% | 3.8% | 3.5% | 8.7% | 4.6% | 3.4% |
| Overall | 5.6% | 2.3% | 2.1% | 6.0% | 2.4% | 2.5% |
|  | 5.6% | 2.3% | 2.1% | 6.0% | 2.5% | 2.4% |
| Class F | 6.3% | 5.2% | 5.9% | 6.4% | 5.2% | 5.5% |
| Enc Time[%] | 97% |  |  | 104% |  |  |
| Dec Time[%] | 96% |  |  | 105% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 3.3% | 1.0% | 1.1% | 3.2% | 1.4% | 1.0% |
| Class C | 3.6% | 1.7% | 1.7% | 3.5% | 1.7% | 1.8% |
| Class D | 4.3% | 1.8% | 1.5% | 4.4% | 1.7% | 2.5% |
| Class E | 5.3% | 3.1% | 1.7% | 5.7% | 2.6% | 2.2% |
| Overall | 4.0% | 1.8% | 1.5% | 4.1% | 1.8% | 1.8% |
|  | 3.9% | 1.8% | 1.4% | 4.0% | 1.8% | 1.8% |
| Class F | 5.7% | 4.9% | 6.5% | 5.8% | 5.6% | 5.6% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 106% |  |  | 102% |  |  |

More objective quality results for other sequences and for maximum TU size 16x16 are available in the spreadsheets.

3.2 Subjective Quality Evaluation

Visual qualities of the proposed method are compared with visual qualities by HM-8.0 anchor and CE-1 Test 8 in the CE1 description [4], which are based on [3]. Visual qualities of all Type 1 and Type 2 sequences suggested in [4] were evaluated. Some of Type 1 sequences are shown in this section.

Figures 1 shows visual qualities of WestWindEasy sequence. CE-1 Test 8 and the proposed method improved visual qualities slightly compared to HM-8.0 anchor. The proposed method provided similar to or slightly better qualities than CE-1 Test 8 does. Figure 2 shows visual qualities of Riverbed sequences. CE-1 Test 8 and the proposed method were clearly better than HM-8.0 anchor. However, CE-1 Test 8 provided slightly better visual quality than the proposed method does for Riverbed sequence. Finally, Figure 3 shows visual qualities of DucksTakeOff sequence. It was observed that CE-1 Test 8 couldn’t remove quite a few blocking artifacts at 32x32 TU boundaries. However, these artifacts were removed by the proposed method.

It is important to note that CE1-1 Test 8 cannot remove completely blocking artifacts at large TU boundaries in some cases. It is still necessary to signal non-zero beta and tc offsets. However, applying beta and tc offset results in significant BD-rate losses. In this regard, the proposed method is useful.

(a) HM-8.0 Anchor

(b) CE-1 Test 8

(c) Proposed

Figure 1. HM-8.0 Anchor, CE-1 Test 8 [4] vs. Proposed Algorithm 1 - WestWindEasy, LB-Main @ Frame 47, QP 32.

(a) HM-8.0 Anchor

(b) CE-1 Test 8

(c) Proposed

Figure 2. HM-8.0 Anchor, CE-1 Test 8 [4] vs. Proposed Algorithm 1 - Riverbed, LB-Main @ Frame 27, QP 37.

(a) HM-8.0 Anchor

(b) CE-1 Test 8

(c) Proposed

Figure 3. HM-8.0 Anchor, CE-1 Test 8 [4] vs. Proposed Algorithm 1 - DucksTakeOff, LB-Main @ Frame 182, QP 37.

# Conclusion

It was proposed to signal a new syntax element max_tu_offset_flag to specify whether the signaled beta and tc offsets are applied to all block boundaries or maximum TU boundaries. By applying offsets to maximum TU boundaries using the proposed syntax element, the blocking artifacts at maximum TU boundaries could be suppressed at the cost of marginal BD-rate loss. It was also proposed to disable intra-prediction boundary filtering at 32x32 intra blocks.

It was shown that some artifacts at large TU boundaries can be removed only by increasing beta and tc offset. Therefore, the proposed method is advantageous in that it does not introduce significant BD-rate loss and does not require any changes in filtering operation itself. Therefore, it is recommended to adopt the proposed method.

# DIS Text Changes

#### Picture parameter set RBSP syntax

Changes are shown in Table 1.

#### 7.3.5.1 General slice header syntax

Changes are shown in Table 2.

#### 7.4.2.4 Picture parameter set RBSP semantics

beta_offset_div2 and tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied for pictures referring to the picture parameter set unless the default deblocking parameter offsets are overriden by the deblocking parameter offsets present in the slice header for pictures referring to the picture parameter set. The values of beta_offset_div2 and tc_offset_div2 shall both be in the range of −6 to 6, inclusive.

max_tu_offset_flag equal to 1 specifies that the signaled beta_offset_div2 and tc_offset_div2 are applied only to maximum TU boundaries. If max_tu_offset_flag is equal to 1, beta_offset_div2_nonMaxTu and tc_offset_div2_nonMaxTu are set to 0, beta_offset_div2_maxTu is set to beta_offset_div2 and tc_offset_div2_maxTu is set to tc_offset_div2. Otherwise, beta_offset_div2_nonMaxTu and beta_offset_div2_maxTu are set to beta_offset_div2 and tc_offset_div2_nonMaxTu and tc_offset_div2_maxTu is set to tc_offset_div2.

#### 7.4.5.1 General slice header semantics

beta_offset_div2 and tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the current slice. The values of beta_offset_div2 and tc_offset_div2 shall be in the range of −6 to 6, inclusive.

max_tu_offset_flag equal to 1 specifies that the signaled beta_offset_div2 and tc_offset_div2 are applied only to maximum TU boundaries. If max_tu_offset_flag is equal to 1, beta_offset_div2_nonMaxTu and tc_offset_div2_nonMaxTu are set to 0, beta_offset_div2_maxTu is set to beta_offset_div2 and tc_offset_div2_maxTu is set to tc_offset_div2. Otherwise, beta_offset_div2_nonMaxTu and beta_offset_div2_maxTu are set to beta_offset_div2 and tc_offset_div2_nonMaxTu and tc_offset_div2_maxTu is set to tc_offset_div2.

##### 8.4.4.2.5 Specification of Intra_DC (1) prediction mode

Inputs to this process are:

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the chroma component of the current block.

Output of this process are:

–	predicted samples predSamples[ x ][ y ], with x, y =0..nT−1.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1, are derived as the following ordered steps:

- A variable dcVal is derived as:

dcVal = x'=0nS−1p[x'][−1]+y'=0nS−1p[−1][ y']+nS>>(k+1)	(8-1)
where k=Log2(nT)

- Depending on the chroma component index cIdx, the following applies.

- If cIdx is equal to 0 and Log2(nT) is less than 5, the following applies.

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2*dcVal + p[ 0 ][ −1 ] + 2 ) >> 2	(8-2)
predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3*dcVal + 2 ) >> 2, with x = 1..nT−1	(8-3)
predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3*dcVal + 2 ) >> 2, with y = 1..nT−1	(8-4)
predSamples[ x ][ y ] = dcVal, with x, y = 1..nT−1	(8-5)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as

predSamples[ x ][ y ] = dcVal, with x, y = 0..nT−1	(8-6)

##### 8.4.4.2.6 Specification of Intra_Angular (2..34) prediction mode

Inputs to this process are:

–	intra prediction mode intraPredMode,

–	neighbouring samples p[ x ][ y ], with x, y = −1..2*nT−1,

–	a variable nT specifying the transform block size,

–	a variable cIdx specifying the chroma component of the current block.

…

2.	The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1 are derived as follows.

  - The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( y + 1 )*intraPredAngle ) >> 5		(8-7)

iFact = ( ( y + 1 )*intraPredAngle ) && 31		(8-8)

  - Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ x+iIdx+1 ] + iFact*ref[ x+iIdx+2] + 16 ) >> 5	(8-9)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ref[ x+iIdx+1 ]		(8-10)

  - When intraPartMode is equal to Intra_Angular (26), cIdx is equal to 0 and Log2(nT) is less than 5, the following filtering applies with x = 0, y = 0..nT−1.

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] – p[ −1 ][ −1 ] ) >> 1 ) )	(8-11)

2.	The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nT−1 are derived as follows.

- The index variable iIdx and the multiplication factor iFact are derived by

iIdx = ( ( x + 1 )*intraPredAngle ) >> 5		(8-12)

iFact = ( ( x + 1 )*intraPredAngle ) && 31		(8-13)

- Depending on the value of iFact, the following applies.

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ( ( 32 – iFact )*ref[ y+iIdx+1 ] + iFact*ref[ y+iIdx+2] + 16 ) >> 5	(8-14)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived by

predSamples[ x ][ y ] = ref[ y+iIdx+1 ]		(8-15)

- When intraPartMode is equal to Intra_Angular (10), cIdx is equal to 0 and Log2(nT) is less than 5, the following filtering applies with x = 0..nT−1, y = 0.

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] – p[ −1 ][ −1 ] ) >> 1 )	(8-16)

#### 8.7.2.3 Derivation process of boundary filtering strength

Inputs of this process are:

–	max_tu_offset_flag,

–	a luma picture sample array recPictureL,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a two-dimensional array of size (nS)x(nS), edgeFlags.

Output of this process is:

- a two-dimensional array of size (nS)x(nS), bS specifying the boundary filtering strength.

–	a two-dimensional array of size (nS)x(nS), maxTuBoundary specifying the maximum transform boundary.

The boundary filtering strength array bS for the current coding unit is derived as follows.

The variables xDi, yDj, xN and yN are derived as follows.

- If edgeType is equal to EDGE_VER, xDi is set equal to ( i << 3 ), yDj is set equal to ( j << 2 ), xN is set equal to (1 << ( log2CbSize − 3 )) − 1 and yN is set equal to (1 << ( log2CbSize − 2 )) − 1.

- Otherwise (edgeType is equal to EDGE_HOR), xDi is set equal to ( i << 2 ), yDj is set equal to ( j << 3 ), xN is set equal to (1 << ( log2CbSize − 2 )) − 1 and yN is set equal to (1 << ( log2CbSize − 3 )) − 1.

For xDi with i=0..xN, the following applies.

For yDj with j=0..yN, the following applies.

- If edgeFlags[ xDi ][ yDj ] is equal to 1, the sample values are derived as follows.

- If edgeType is equal to EDGE_VER, sample p0 = recPictureL[ xC + xDi − 1 ][ yC + yDj ] and q0 = recPictureL[ xC + xDi ][ yC + yDj ].

- Otherwise (edgeType is equal to EDGE_HOR), sample p0 = recPictureL[ xC + xDi ][ yC + yDj − 1 ] and q0 = recPictureL[ xC + xDi ][ yC + yDj ].

Depending on p0 and q0, the variable maxTuBoundary[ xDi ][ yDj ] is derived as follows.

  - The variables log2TrafoSizeQ and log2TrafoSizeP are set equal to the log2TrafoSize values of the transform units which include the sample q0 and p0, respectively.

- If max_tu_offset_flag is equal to 0, maxTuBoundary[ xDi ][ yDj ] is set equal to 0.

  - Otherwise, if the block edge is also a transform block edge and one of the following conditions is true then the variable maxTuBoundary [ xDi ][ yDj ] is set equal to 1.

    - If the sample p0 is in the luma coding block of a coding unit that is coded with intra prediction mode or contains non-zero transform coefficient level and log2TrafoSizeP is equal to Log2MaxTrafoSize

    - The sample q0 is in the luma coding block of a coding unit coded that is coded with intra prediction mode or contains non-zero coefficient level and log2TrafoSizeQ is equal to Log2MaxTrafoSize

- Otherwise, maxTuBoundary [ xDi ][ yDj ] is set equal to 0.

Depending on p0 and q0, the variable bS[ xDi ][ yDj ] is derived as follows.

- If the sample p0 or q0 is in the luma coding block of a coding unit coded with intra prediction mode, the variable bS[ xDi ][ yDj ] is set equal to 2.

- Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a luma transform block which contains non-zero transform coefficient level.

- Otherwise, the following applies.

……

##### 8.7.2.4.3	Decision process for luma block edge

Inputs of this process are:

–	a luma picture sample array recPictureL,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma block relative to the top left sample of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a variable bS specifying the boundary filtering strength.

–	a variable maxTuBoundary specifying the 32x32 transform boundary.

Outputs of this process are:

–	variables dE, dEp, dEq containing decisions,

–	variables β, tC.

If maxTuBoundary

If edgeType is equal to EDGE_VER, the sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = recPictureL[ xC + xB +i ][ yC + yB + k ]		(8-17)

pi,k = recPictureL[ xC + xB – i − 1 ][ yC + yB + k ]	(8-18)

Otherwise (edgeType is equal to EDGE_HOR), the sample values pi,k and qi,k with i = 0..3 and k = 0, 3 are derived as follows:

qi,k = recPictureL[ xC + xB +k ][ yC + yB + i ]		(8-19)

pi,k = recPictureL[ xC + xB +k ][ yC + yB – i − 1 ]		(8-20)

The variables QPQ and QPP are set equal to the QPY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

A variable qPL is derived as follows:

qPL = ( ( QPQ + QPP + 1 ) >> 1 )	(8-21)

The value of the variable β′ is determined as specified in Error: Reference source not found based on the luma quantization parameter Q derived as:

if maxTuBoundary is equal to 1,

Q = Clip3( 0, 51, qPL + ( beta_offset_div2_maxTu << 1 ) )	(8-22)

Otherwise,

Q = Clip3( 0, 51, qPL + ( beta_offset_div2_nonMaxTu << 1 ) )	(8-23)

where beta_offset_div2_maxTu and beta_offset_div2_nonMaxTu are the value of the variable beta_offset_div2_maxTu and beta_offset_div2_nonMaxTu for the slice that contains sample q0,0.

The variable β is derived as:

β = β′*(1<<(BitDepthY − 8))	(8-24)

The value of the variable tC′ is determined as specified as Error: Reference source not found based on the luma quantization parameter Q derived as:

if maxTuBoundary is equal to 1,

Q = Clip3( 0, 53, qPL + 2*( bS − 1) + ( tc_offset_div2_maxTu << 1) )	(8-25)

Otherwise,

Q = Clip3( 0, 53, qPL + 2*( bS − 1) + ( tc_offset_div2_nonMaxTu << 1) )	(8-26)

where tc_offset_div2_maxTu and tc_offset_div2_nonMaxTu are the value of the variable tc_offset_div2_maxTu and tc_offset_div2_nonMaxTu for the slice that contains sample q0,0.

The variable tC is derived as:

tC = tC′*(1<<(BitDepthY − 8))	(8-27)

……

##### 8.7.2.4.5 Filtering process for chroma block edge

Inputs of this process are:

–	a chroma picture sample array s’,

–	a luma location ( xC, yC ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left sample of the current luma block relative to the top left sample of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a variable bS specifying the boundary filtering strength.

–	a variable maxTuBoundary specifying the 32x32 transform boundary.

Output of this process is:

–	the modified chroma picture sample array s’.

If edgeType is equal to EDGE_VER, the values pi and qi with i = 0..1 and k = 0..3 are derived as follows:

qi,k = s’[ xC + xB +i ][ yC + yB + k ]	(8-28)

pi,k = s’[ xC + xB – i − 1 ][ yC + yB + k ]	(8-29)

Otherwise (edgeType is equal to EDGE_HOR), the sample values pi and qi with i = 0..1 and k = 0..3 are derived as follows:

qi,k = s’[ xC + xB +k ][ yC + yB + i ]	(8-30)

pi,k = s’[ xC + xB +k ][ yC + yB – i − 1 ]	(8-31)

The variables QPQ and QPP are set equal to the QPY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable QPC is determined as specified in Error: Reference source not found based on the index qPi derived as:

qPi = ( ( QPQ + QPP + 1 ) >> 1 )	(8-32)

The value of the variable tC′ is determined as specified as Error: Reference source not found based on the chroma quantization parameter Q derived as:

if maxTuBoundary is equal to 1,

Q = Clip3( 0, 53, QPC + 2*(bS − 1) + (tc_offset_div2_maxTu << 1 ) )	(8-33)

Otherwise,

Q = Clip3( 0, 53, QPC + 2*(bS − 1) + (tc_offset_div2_nonMaxTu << 1 ) )	(8-34)

The variable tC is derived as:

tC = tC′*(1<<(BitDepthY − 8))	(8-35)

where tc_offset_div2_maxTu and tc_offset_div2_nonMaxTu are the value of the variable tc_offset_div2_maxTu and tc_offset_div2_nonMaxTu for the slice that contains sample q0,0.

# References

- D.-K. Kwon and M. Budagavi, “Transform size dependent deblocking filter”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0244, May 2012.

- D.-K. Kwon and M. Budagavi, “Suppression of blocking artifacts at 32x32 transform boundaries”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J0096, July 2012.

- A. Norkin, “AHG6: Reduction of block artifacts in HEVC for large blocks”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J0286, July 2012.

- A. Norkin, T. Suzuki, M. Budagavi and G. V. Auwera, “Description of Core Experiment 1: Deblocking filter”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J1101, July 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).