| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0270 |
| --- | --- |

| Title: | Unification of Context Modeling Methods for Large Transform Blocks for Coding Significant Coefficient Flags |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang and Tzu-Der Chuang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, peter.chuang shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the 10th JCT-VC meeting in Stockholm, it was adopted to employ the same set of the context templates used by the 16x16 and 32x32 transform blocks (TBs) for entropy coding significant coefficient flags in the 8x8 TBs. This contribution proposes the modified context modeling methods for the 8x8 TBs in order to further unify the context modeling schemes for the 8x8, 16x16 and 32x32 TBs. The proposed modifications remove the need to reference the scan order index for context selection and further reduce the number of the modeling contexts for the 8x8 TBs. Simulation results reportedly show that the proposed method with context reduction by 6 increases overall average Y BD rate by 0.03%, 0.02%, 0.02%, 0.03%, 0.00%, and -0.02% for AI Main, RA Main, LB Main, AI HE10, RA HE10, and LB HE10, respectively, under the common test conditions.

# Introduction

In the 10th JCT-VC meeting in Stockholm, the proposal JCTVC-J0256 [1] was adopted to remove 8x2 and 2x8 coefficient group (CGs) associated with the 8x8 transform block (TB). It was also adopted to employ the same modeling method and template set used by the 16x16 and 32x32 TBs for context selection and modeling. This method employs one of the templates, as illustrated in Figure 1, to map the positions of the transform coefficients within a 4x4 sub-block to the context indices of a context subset consisting of three contexts. Selection of the template is determined by the coded_sub_block_flag values of the bottom and right neighboring sub-blocks of the current subblock. For the Luma component, a TB is further divided into the low-frequency sub-block set (containing only the top-left sub-block) and the high-frequency sub-block set and each sub-block set is assigned separate context sets. The 16x16 and 32x32 TBs share their contexts.

However, it was adopted to use the separate context subsets for coding the 8x8 TBs. The 8x8 Luma TBs further allocate additional context subsets for the TBs in horizontal or vertical scan order. As a result, we need 6 contexts for entropy coding significant coefficient flags  in the 16x16 and 32x32 Luma TBs and 12 contexts in the 8x8 Luma TBs in High Efficiency Video Coding Test Model version 8.0 (HM-8.0), as summarized in Table 1.

Figure 1: Templates employed for context selection for coding significant_coeff_flag. The template selection is determined by (bottom coded_sub_block_flag, right coded_sub_block_flag).

# Proposed Method

This proposal attempts to further unify the context modeling schemes for the 8x8 TBs and the 16x6 and 32x32 TBs in order to lower the complexity for entropy coding. In HM-8.0, the 8x8 Luma TB is the only TB type that employs a separate set of contexts for coding TBs corresponding to a horizontal or vertical scan order. It thus increases the number of the modeling contexts and needs additional logic and test for determining the scan order index for context selection for entropy coding the 8x8 TB.

This contribution proposes to employ the same set of contexts for all the 8x8 Luma TBs in diagonal and non-diagonal scans orders, disregarding the scan index order for the current TB. That is, in our Method I we only employ 2 context subsets for the 8x8 Luma TBs, one for the lowest-frequency sub-block and the other for the three high-frequency sub-blocks. In this way, we can reduce the total number of the contexts by 6 and remove the need to reference the scan order index for context selection operation, allowing further unification of context modeling methods for 8x8, 16x16, and 32x32 TBs.

In our Method II, it is tested to further reduce the total number of the contexts by having the high-frequency sub-blocks from the 8x8, 16x16 and 32x32 Luna TBs share the same set of contexts and having the sub-blocks from the 8x8 and 16x16 Chroma TBs share the same set of contexts. In this way, we can reduce the total number of the modeling contexts for entropy coding significant coefficient flags by 12. Both proposed methods just re-use the related parameters in HM-8.0 for the initialization of the proposed CABAC models.

Table 1: Summary of the numbers of the contexts employed for different TB types by HM-8.0 and the proposed methods (* partially shared with the 8x8 TBs for Method II)

|  | Luma Component |  |  |  | Chroma Component |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DC | 4x4 | 8x8 | 16x16& 32x32* | DC | 4x4 | 8x8 | 16x16& 32x32* | Total |
| HM-8.0 | 1 | 8 | 12 | 6 | 1 | 8 | 3 | 3 | 42 |
| Method I | 1 | 8 | 6 | 6 | 1 | 8 | 3 | 3 | 36 |
| Method II | 1 | 8 | 3 | 6 | 1 | 8 | 0 | 3 | 30 |

# Simulation Results

The proposed algorithm has been implemented using HM-8.0 as software base and experimentally evaluated according to the common test conditions defined in JCTVC-J1100 [2]. The experimental results are provided in the included spread sheets and summarized in Tables 2 - 5.

Please note our previous results, reported in the spread sheet file ‘JCTVC-K270-red12.xls’, for our proposed method with context reduction by 12 were generated with a minor software bug related to the RDOQ bit estimation. The related results have been updated with correct data in this version.

Table 2: Summary of the BD-rate results for the proposal with context reduction by 6

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.02% | -0.01% | -0.05% | 0.02% | -0.01% | 0.01% |
| Class B | 0.02% | -0.02% | -0.02% | 0.02% | -0.03% | -0.01% |
| Class C | 0.03% | 0.03% | 0.02% | 0.03% | 0.04% | -0.04% |
| Class D | 0.02% | 0.03% | 0.00% | 0.02% | 0.00% | 0.01% |
| Class E | 0.09% | -0.03% | -0.06% | 0.10% | -0.07% | -0.05% |
| Overall | 0.03% | 0.00% | -0.02% | 0.03% | -0.01% | -0.01% |
|  | 0.03% | 0.00% | -0.02% | 0.03% | -0.01% | -0.03% |
| Class F | 0.00% | 0.01% | 0.02% | 0.04% | 0.05% | -0.02% |
| Enc Time[%] | 100.26% |  |  | 100.15% |  |  |
| Dec Time[%] | 100.68% |  |  | 106.39% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.04% | -0.04% | -0.06% | -0.02% | -0.11% | -0.27% |
| Class B | 0.01% | 0.00% | -0.02% | 0.01% | 0.10% | 0.04% |
| Class C | 0.00% | 0.05% | 0.03% | 0.03% | 0.00% | -0.13% |
| Class D | 0.03% | 0.06% | -0.12% | -0.03% | -0.05% | -0.37% |
| Class E |  |  |  |  |  |  |
| Overall | 0.02% | 0.02% | -0.04% | 0.00% | -0.01% | -0.17% |
|  | 0.02% | 0.01% | -0.04% | 0.00% | 0.00% | -0.17% |
| Class F | -0.02% | -0.05% | -0.08% | 0.00% | 0.14% | 0.09% |
| Enc Time[%] | 99.62% |  |  | 99.96% |  |  |
| Dec Time[%] | 100.96% |  |  | 100.79% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.03% | 0.19% | 0.05% | -0.01% | -0.16% | 0.23% |
| Class C | 0.04% | 0.05% | -0.10% | -0.01% | 0.01% | 0.45% |
| Class D | -0.05% | 0.67% | -0.25% | 0.03% | -0.31% | -0.55% |
| Class E | 0.06% | -0.72% | 1.01% | -0.12% | 0.72% | -0.49% |
| Overall | 0.02% | 0.11% | 0.12% | -0.02% | 0.01% | -0.05% |
|  | 0.02% | 0.07% | 0.00% | -0.01% | -0.07% | -0.06% |
| Class F | 0.16% | 0.08% | -0.36% | -0.04% | 0.84% | -0.58% |
| Enc Time[%] | 100.19% |  |  | 99.10% |  |  |
| Dec Time[%] | 99.63% |  |  | 98.15% |  |  |

Table 3: Summary of the BD-rate results for the proposal with context reduction by 12

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.05% | 0.01% | -0.01% | 0.05% | 0.01% | 0.01% |
| Class B | 0.05% | -0.09% | -0.06% | 0.05% | -0.06% | -0.04% |
| Class C | 0.05% | 0.05% | 0.03% | 0.05% | 0.02% | -0.02% |
| Class D | 0.03% | 0.04% | -0.01% | 0.04% | 0.04% | 0.01% |
| Class E | 0.11% | -0.05% | -0.08% | 0.11% | -0.05% | -0.07% |
| Overall | 0.06% | -0.01% | -0.03% | 0.06% | -0.01% | -0.02% |
|  | 0.06% | -0.01% | -0.03% | 0.06% | 0.00% | -0.02% |
| Class F | 0.09% | 0.02% | 0.01% | 0.11% | 0.04% | 0.04% |
| Enc Time[%] | 99.07% |  |  | 99.03% |  |  |
| Dec Time[%] | 98.49% |  |  | 98.87% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.04% | -0.06% | -0.11% | -0.02% | 0.12% | 0.07% |
| Class B | 0.01% | -0.13% | 0.14% | 0.01% | -0.05% | 0.05% |
| Class C | 0.01% | 0.01% | 0.13% | 0.02% | 0.14% | -0.07% |
| Class D | 0.04% | 0.09% | 0.10% | -0.04% | 0.08% | -0.28% |
| Class E |  |  |  |  |  |  |
| Overall | 0.02% | -0.03% | 0.07% | -0.01% | 0.06% | -0.05% |
|  | 0.02% | -0.06% | 0.02% | -0.01% | 0.07% | -0.05% |
| Class F | 0.02% | -0.12% | -0.17% | 0.08% | -0.03% | -0.06% |
| Enc Time[%] | 99.09% |  |  | 99.45% |  |  |
| Dec Time[%] | 99.31% |  |  | 99.71% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.04% | -0.11% | -0.17% | 0.04% | -0.16% | 0.04% |
| Class C | 0.09% | -0.07% | -0.02% | -0.01% | -0.04% | 0.10% |
| Class D | -0.03% | 0.73% | -0.21% | 0.04% | 0.01% | -0.35% |
| Class E | 0.02% | -0.06% | 0.93% | -0.16% | 0.67% | -0.43% |
| Overall | 0.03% | 0.12% | 0.06% | -0.01% | 0.07% | -0.13% |
|  | 0.03% | 0.09% | -0.02% | -0.01% | -0.02% | -0.18% |
| Class F | 0.19% | 0.32% | -0.34% | -0.06% | 0.13% | -1.24% |
| Enc Time[%] | 99.85% |  |  | 98.23% |  |  |
| Dec Time[%] | 98.30% |  |  | 95.56% |  |  |

Table 4: Summary of the BD-rate results for the proposal with context reduction by 6 over low QPs (QP = 12, 17, 22, 27)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | -0.01% | 0.04% | 0.03% | 0.00% | 0.04% | 0.03% |
| Class C | 0.00% | 0.02% | 0.01% | 0.00% | 0.00% | 0.00% |
| Class D | -0.01% | 0.00% | 0.00% | -0.01% | 0.01% | 0.00% |
| Class E | 0.01% | 0.02% | 0.00% | 0.02% | -0.01% | 0.01% |
| Overall | 0.00% | 0.02% | 0.01% | 0.00% | 0.01% | 0.01% |
|  | 0.00% | 0.02% | 0.01% | 0.00% | 0.01% | 0.01% |
| Class F | 0.01% | -0.03% | 0.02% | 0.01% | 0.00% | -0.01% |
| Enc Time[%] | 100.18% |  |  | 100.31% |  |  |
| Dec Time[%] | 99.86% |  |  | 104.56% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.04% | -0.05% | -0.04% | 0.02% | 0.04% | -0.08% |
| Class B | 0.02% | 0.03% | 0.00% | 0.00% | 0.00% | 0.04% |
| Class C | 0.01% | 0.05% | 0.02% | 0.02% | 0.02% | -0.02% |
| Class D | 0.01% | -0.02% | 0.09% | -0.01% | 0.05% | -0.03% |
| Class E |  |  |  |  |  |  |
| Overall | 0.02% | 0.01% | 0.02% | 0.00% | 0.03% | -0.02% |
|  | 0.03% | 0.00% | 0.02% | 0.01% | 0.03% | -0.02% |
| Class F | -0.06% | -0.07% | 0.05% | 0.03% | 0.02% | 0.13% |
| Enc Time[%] | 100.10% |  |  | 100.26% |  |  |
| Dec Time[%] | 100.58% |  |  | 107.81% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.01% | -0.05% | 0.00% | -0.01% | -0.06% |
| Class C | 0.00% | -0.01% | 0.06% | 0.01% | -0.01% | -0.01% |
| Class D | 0.00% | 0.11% | -0.06% | 0.01% | -0.18% | -0.08% |
| Class E | 0.08% | 0.13% | 0.11% | -0.04% | 0.07% | -0.24% |
| Overall | 0.01% | 0.05% | 0.00% | 0.00% | -0.04% | -0.09% |
|  | 0.01% | 0.08% | 0.01% | 0.00% | -0.07% | -0.09% |
| Class F | 0.08% | -0.05% | -0.12% | -0.06% | 0.08% | -0.19% |
| Enc Time[%] | 100.48% |  |  | 100.42% |  |  |
| Dec Time[%] | 100.18% |  |  | 104.99% |  |  |

Table 5: Summary of the BD-rate results for the proposal with context reduction by 12 over low QPs (QP = 12, 17, 22, 27)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.02% | 0.00% | 0.00% | 0.02% | 0.00% | 0.00% |
| Class B | -0.05% | -0.15% | -0.06% | -0.06% | -0.15% | -0.06% |
| Class C | 0.00% | 0.02% | 0.02% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | -0.01% | 0.02% | 0.00% | 0.00% | 0.01% |
| Class E | 0.02% | -0.01% | -0.01% | 0.03% | -0.03% | 0.00% |
| Overall | -0.01% | -0.04% | -0.01% | -0.01% | -0.04% | -0.01% |
|  | -0.01% | -0.04% | 0.00% | -0.01% | -0.04% | -0.01% |
| Class F | 0.08% | -0.01% | 0.00% | 0.08% | -0.04% | -0.01% |
| Enc Time[%] | 99.55% |  |  | 99.51% |  |  |
| Dec Time[%] | 99.31% |  |  | 99.01% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.01% | -0.02% | -0.04% | 0.03% | 0.06% | -0.04% |
| Class B | 0.02% | -0.08% | -0.10% | -0.04% | -0.16% | 0.04% |
| Class C | 0.00% | 0.01% | -0.02% | 0.01% | 0.01% | -0.06% |
| Class D | 0.02% | 0.01% | 0.09% | 0.01% | 0.01% | -0.09% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.02% | 0.00% | -0.03% | -0.03% |
|  | 0.03% | -0.02% | -0.03% | 0.01% | -0.02% | -0.04% |
| Class F | 0.02% | -0.07% | -0.05% | 0.04% | -0.10% | 0.04% |
| Enc Time[%] | 99.28% |  |  | 99.85% |  |  |
| Dec Time[%] | 99.38% |  |  | 100.39% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.02% | -0.09% | -0.02% | -0.01% | -0.10% | -0.06% |
| Class C | 0.01% | -0.02% | -0.05% | 0.01% | -0.07% | 0.00% |
| Class D | 0.01% | -0.01% | -0.03% | 0.03% | -0.11% | -0.18% |
| Class E | 0.03% | -0.45% | -0.20% | -0.07% | 0.15% | -0.73% |
| Overall | 0.01% | -0.12% | -0.06% | -0.01% | -0.05% | -0.20% |
|  | 0.00% | -0.15% | -0.08% | 0.00% | -0.05% | -0.23% |
| Class F | 0.00% | 0.02% | -0.07% | -0.03% | -0.13% | -0.40% |
| Enc Time[%] | 100.04% |  |  | 100.10% |  |  |
| Dec Time[%] | 99.32% |  |  | 99.41% |  |  |

# Conclusion

This contribution proposes the modified context modeling methods for entropy coding significant coefficient flags in the 8x8 transform blocks (TBs). The proposed methods reportedly further unify the context modeling schemes for the large TBs and remove the need to reference the scan order index for context selection. The simulation results reportedly show no significant differences in BD-rates and run times using the proposed methods in comparison with HM-8.0. The simulation results also reportedly confirm that we can employ only one subset of 3 contexts, instead of 12 contexts, dedicated to the 8x8 Luma TBs and share the contexts for the large Chroma TBs without important BD rate loss.

# References

- J. Sole, R. Joshi, M. Karczewicz, “Removal of the 8x2/2x8 coefficient groups,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0256, July, 2012.

- F. Bossen, “HM 8 common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-J1100, July, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).