The proposed modifications are based on High efficiency video coding (HEVC) text specification draft 8 (JCTVC-J1003). The inserted parts and removed parts are marked in yellow and pink, respectively.

###### 9.3.3.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan position ( xC, yC ), the scan order index scanIdx, the transform block size log2TrafoSize.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the colour component index cIdx, the transform block size and previsously decoded bins of the syntax element coded_sub_block_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoSize is equal to 2, sigCtx is derived using ctxIdxMap[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap[ (yC << 2) + xC ]	(9-1)

- Otherwise, if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  0		(9-2)

- Otherwise, sigCtx is derived using previous values of coded_sub_block_flag as follows.

- The horizontal and vertical sub-block positions xS and yS are set equal to (xC >> 2) and (yC >> 2), respectively.

- The variable prevCsbf is set equal to 0.

- When xS is less than ( 1 << log2TrafoSize − 2 ) − 1, the following applies.

prevCsbf  +=  coded_sub_block_flag[ xS + 1 ][ yS ]	(9-3)

- When yS is less than ( 1 << log2TrafoSize ) − 1, the following applies.

prevCsbf  +=  (coded_sub_block_flag[ xS ][ yS + 1 ] << 1)	(9-4)

- The inner sub-block positions xP and yP are set equal to ( xC & 3 ) and ( yC & 3 ), respectively.

- The variable sigCtx is derived as follows.

  - If prevCsbf is equal to 0, the the following applies.

sigCtx  =  ( xP + yP = =  0 ) ? 2 : ( xP + yP < 3 ) ? 1: 0	(9-5)

  - Otherwise, if prevCsbf is equal to 1, the following applies.

sigCtx  =  ( yP = =  0 ) ? 2 : ( yP = =  1 ) ? 1: 0	(9-6)

  - Otherwise, if prevCsbf is equal to 2, the following applies.

sigCtx  =  ( xP = =  0 ) ? 2 : ( xP = =  1 ) ? 1: 0	(9-7)

  - Otherwise ( prevCsbf is equal to 3 ), the following applies.

sigCtx  =  2	(9-8)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0, the following applies.

  - When ( xS + yS ) is greater than 0, the following applies.

sigCtx += 3		(9-9)

  - The variable sigCtx is modified as follows.

  - If log2TrafoSize is equal to 3, the following applies.

sigCtx += ( scanIdx = = 0 ) ? 9 : 15	(9-10)

sigCtx += 9                                                       (9-11)

  - Otherwise, the following applies.

sigCtx += 21		(9-12)

sigCtx += 15		(9-13)

  - Otherwise (cIdx is greater than 0), the following applies.

  - If log2TrafoSize is equal to 3, the following applies.

sigCtx += 9		(9-14)

  - Otherwise, the following applies.

sigCtx += 12		(9-15)

The context index increment ctxIdxInc is derived using the colour component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-16)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx		(9-17)

ctxIdxInc  =  21  +  sigCtx		(9-18)

Table 9-1 – Specifcation of ctxIdxMap[ i ]

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ctxIdxMap[ i ] | 0 | 1 | 4 | 5 | 2 | 3 | 4 | 5 | 6 | 6 | 8 | 8 | 7 | 7 | 8 |