| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct 2012 | Document: JCTVC-K0272 |
| --- | --- |

| Title: | Compression performance comparison of HEVC with AVC at cfp target bitrates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianhua Zheng, Yongbing Lin, Xiaozhen Zheng Q20, No. 156, Rd. Beiqing Haidian District, Beijing 100095 China | Tel: Email: | +86-10-60612141 zhengjianhua@huawei.com |
| Source: | HiSilicon Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

In past meetings, HEVC compression performance compared to AVC are presented and evaluated at fixed QP points. However the coding gain evaluated at the fixed QP points in current common test condition can not reflect performance under target rates in real application. As HEVC call for proposal defined five target rate points to evaluate the coding performance in HEVC application bitrate range. This contribution provides the compression performance comparison of HEVC main profile and AVC high profile under the HEVC call for proposal target rate points. HM8.0 and JM 18.4 are used in evaluation. The experimental results show that in high bitrate range, HM8.0 main profile achieves 34.7%, 36.9% and 38.1% bitrate saving for RA/LDB/LDP case. In low bitrate range, HM8.0 main profile achieves 36.3%, 38.6% and 40.4% bitrate saving for RA/LDB/LDP case. The decoding time of HM8.0 main profile are roughly double with comparison to JM18.4.

The test QP points are selected individually for each test materials and provided in this proposal annex to meet the Cfp target bitrates for RA/LDB/LDP case. It is proposed to restrict the bitrates in common test condition and substitute these QP points setting for the fixed QPs to evaluate coding performance within typical application bitrate range.

# Introduction

In HEVC call for proposal[1], target rate points table is defined to check the codec compression performance at the most possible application bitrate for HEVC design. These target rate points are list in Table1[1], see Class A to Class E. It is typical bitrates for HEVC application.

In recent JCT-VC meetings, several contributions are presented to report the HEVC coding gain compared to AVC high profile. These experiments are completed under four fixed QPs. For each video sequence four quantization parameter values are to be used: 22, 27, 32 and 37[2]. However, it is observed a lot of bitrate of test points using these four fixed QPs have exceed the target bitrate range in HEVC call for proposal, e.g. for Nebuta in RA-Main case, bitrate reach up to 216M, for BQTerrace in LDB-MAIN case, bitrate reach up to 52M, which has seriously exceed the bitrate of rate5 in Table1. Objective and subjective comparison between HEVC and AVC at those bitrates may not reflect the real coding performance. This contribution evaluate HEVC and AVC compression performance under HEVC call for proposal bitrates, it may be more useful information for HEVC products and applications.

Table 1  Target rate points

| Class | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| --- | --- | --- | --- | --- | --- |
| A: 2560x1600p30 | 2.5 Mbit/s | 3.5 Mbit/s | 5 Mbit/s | 8 Mbit/s | 14 Mbit/s |
| B1: 1080p24 | 1 Mbit/s | 1.6 Mbit/s | 2.5 Mbit/s | 4 Mbit/s | 6 Mbit/s |
| B2: 1080p50-60 | 2 Mbit/s | 3 Mbit/s | 4.5 Mbit/s | 7 Mbit/s | 10 Mbit/s |
| C: WVGAp30-60 | 384 kbit/s | 512 kbit/s | 768 kbit/s | 1.2 Mbit/s | 2 Mbit/s |
| D: WQVGAp30-60 | 256 kbit/s | 384 kbit/s | 512 kbit/s | 850 kbit/s | 1.5 Mbit/s |
| E: 720p60 | 256 kbit/s | 384 kbit/s | 512 kbit/s | 850 kbit/s | 1.5 Mbit/s |
| F: | 256 kbit/s | 384 kbit/s | 512 kbit/s | 850 kbit/s | 1.5 Mbit/s |

# Performance evaluation

For AVC and HEVC comparison, JM18.4 and HM8.0 are used. HM8.0 is configured using common test condition configuration except that test QPs are selected for individual sequences. JM18.4 is configured using the HM-like configuration available in JM18.4 software. The test platform is equipped with Xeon X5650 12 cores @2.66GHz , 48GB RAM, Windows server 2003(x64).

In this contribution, Table1 is extended to include Class F test materials, and the target bitrates are set as same as ClassE due to the screen contents scenario may be very similar to video conference.

For each test sequences and HEVC main profile test cases in common test condition(RA/LDB/LDP), five QPs are selected for individual test sequence and test case. Coding performance is evaluated at high bitrate range from rate2 to rate5, and at low bitrate range from rate1 to rate4. The QP settings for HM8.0 and JM18.4 in RA/LDB/LDP test cases are provided in annex Table 8 and Table 9, respectively, in which, five qp values are corresponding to the five target rate point in Table 1.

The performance is reported as average BD-rate saving. The average YUV PSNR (the right-most column of the summary Tables) was calculated as

PSNRavg=6×PSNRY+PSNRU+PSNRV8

# Experimental results

The summary results of performance in high bitrate range are shown in the Tables 2, 3, and 4. The summary results of performance in low bitrate range are shown in the Tables 5, 6, and 7. Details can be found in the attached excel files.

## AVC v.s. HEVC in high bitrate range

Table 2  Results for Random Access (JM18.4 v.s. HM8.0), high bitrate range

|  | Random Access Main |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | Average |
| Class A | -42.1% | -41.4% | -43.6% | -42.6% |
| Class B | -39.6% | -36.4% | -34.4% | -39.0% |
| Class C | -30.9% | -31.5% | -31.1% | -31.1% |
| Class D | -27.3% | -31.5% | -32.2% | -28.2% |
| Class E |  |  |  |  |
| Class F | -30.6% | -33.1% | -35.3% | -31.5% |
| Overall | -34.4% | -34.8% | -35.3% | -34.7% |
| Enc Time[%] | 118% |  |  |  |
| Dec Time[%] | 181% |  |  |  |

Table 3  Results for Low delay B (JM18.4 v.s. HM8.0),high bitrate range

|  | Low delay B Main |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | Average |
| Class A |  |  |  |  |
| Class B | -42.1% | -35.0% | -35.2% | -41.0% |
| Class C | -35.0% | -37.4% | -36.9% | -35.5% |
| Class D | -30.8% | -33.5% | -34.7% | -31.3% |
| Class E | -42.1% | -31.6% | -32.0% | -40.3% |
| Class F | -35.2% | -39.6% | -42.1% | -36.4% |
| Overall | -37.0% | -35.6% | -36.4% | -36.9% |
| Enc Time[%] | 80% |  |  |  |
| Dec Time[%] | 180% |  |  |  |

Table 4  Results for Low delay P (JM18.4 v.s. HM8.0), high bitrate range

|  | Low delay P Main |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | Average |
| Class A |  |  |  |  |
| Class B | -38.6% | -40.2% | -41.0% | -39.1% |
| Class C | -35.6% | -42.5% | -41.8% | -36.9% |
| Class D | -30.5% | -40.5% | -40.8% | -32.2% |
| Class E | -42.6% | -44.6% | -44.9% | -43.2% |
| Class F | -38.7% | -47.0% | -48.2% | -40.5% |
| Overall | -37.0% | -42.7% | -43.1% | -38.1% |
| Enc Time[%] | 132% |  |  |  |
| Dec Time[%] | 206% |  |  |  |

## AVC v.s. HEVC in low bitrate range

Table 5  Results for Random Access (JM18.4 v.s. HM8.0), low bitrate range

|  | Random Access Main |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | Average |
| Class A | -45.7% | -27.8% | -29.9% | -46.5% |
| Class B | -40.8% | -34.6% | -33.0% | -39.9% |
| Class C | -32.0% | -37.0% | -37.3% | -33.0% |
| Class D | -28.1% | -30.7% | -32.8% | -28.9% |
| Class E |  |  |  |  |
| Class F | -29.8% | -42.7% | -43.0% | -32.3% |
| Overall | -35.6% | -34.6% | -35.1% | -36.3% |
| Enc Time[%] | 110% |  |  |  |
| Dec Time[%] | 181% |  |  |  |

Table 6  Results for Low delay B (JM18.4 v.s. HM8.0),low bitrate range

|  | Low delay B Main |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | Average |
| Class A |  |  |  |  |
| Class B | -43.9% | -33.9% | -34.9% | -42.6% |
| Class C | -37.2% | -44.4% | -44.4% | -38.6% |
| Class D | -31.4% | -32.1% | -35.5% | -32.0% |
| Class E | -43.1% | -33.5% | -34.7% | -41.5% |
| Class F | -35.8% | -47.1% | -47.1% | -38.3% |
| Overall | -38.3% | -38.2% | -39.3% | -38.6% |
| Enc Time[%] | 77% |  |  |  |
| Dec Time[%] | 177% |  |  |  |

Table 7  Results for Low delay P (JM18.4 v.s. HM8.0), low bitrate range

|  | Low delay P Main |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | Average |
| Class A |  |  |  |  |
| Class B | -41.6% | -39.1% | -39.8% | -41.5% |
| Class C | -37.6% | -50.0% | -48.9% | -39.6% |
| Class D | -32.6% | -37.4% | -40.2% | -33.9% |
| Class E | -45.8% | -45.5% | -46.0% | -45.8% |
| Class F | -39.2% | -34.3% | -34.4% | -42.3% |
| Overall | -39.2% | -40.9% | -41.5% | -40.4% |
| Enc Time[%] | 124% |  |  |  |
| Dec Time[%] | 212% |  |  |  |

## AVC v.s. HEVC summary

Table 8  Summary results with Class F

| AVC v.s. HEVC | high bitrate range |  |  |  | low bitrate range |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA | LDB | LDP |  | RA | LDB | LDP |
| Avg. bitrate saving | 34.7% | 36.9% | 38.1% |  | 36.3% | 38.6% | 40.4% |
| Encoding Time | 118% | 80% | 132% |  | 110% | 77% | 124% |
| Decoding Time | 181% | 180% | 206% |  | 181% | 177% | 212% |

Table 9 Summary results without Class F

| AVC v.s. HEVC | high bitrate range |  |  |  | low bitrate range |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA | LDB | LDP |  | RA | LDB | LDP |
| Avg. bitrate saving | 35.4% | 37.1% | 37.6% |  | 37.2% | 38.7% | 39.9% |
| Encoding Time | 119% | 81% | 137% |  | 111% | 78% | 129% |
| Decoding Time | 183% | 185% | 203% |  | 183% | 182% | 209% |

# Conclusions

This contribution provides the compression performance comparison of HEVC main profile and AVC high profile under the HEVC call for proposal target bitrates. The results show that,

For high bitrate range QPs setting,

For random access case, HM 8.0 achieves 34.7% bitrate saving and encoding/decoding time increasing 118%/181% , and

For low delay B case, HM 8.0 achieves 36.9% bitrate saving and encoding/decoding time increasing 80%/180%, and

For low delay P case, HM 8.0 achieves 38.1% bitrate saving and encoding/decoding time increasing 132%/206%.

For low bitrate range QPs Setting,

For random access case, HM 8.0 achieves 36.3% bitrate saving and encoding/decoding time increasing 110%/181% , and

For low delay B case, HM 8.0 achieves 38.6% bitrate saving and encoding/decoding time increasing 77%/177%, and

For low delay P case, HM 8.0 achieves 40.4% bitrate saving and encoding/decoding time increasing 124%/212%.

The test QP points for each test materials in RA/LB/LP test case are provided in Table 8 in the annex. It is proposed to restrict the bitrates in common test condition and use individual QP points for each test material and take Table 8 as a substitute for the current four fixed QP points in common test condition.

# References

- MPEG output document, N11113, “Joint Call for Proposals on Video Compression Technology” January 2010, Kyoto, JP

- JCTVC-I1100, “Common HM test conditions and software reference configurations,” Geneva, CH, 27 April – 7 May 2012.

# Annex

Table 8  HM8.0 QP settings for Cfp target rate points

| Class | Sequence name | Random access | Low-delayB | Low-delay P |
| --- | --- | --- | --- | --- |
| A | Traffic | 22,25,27,30,32 |  |  |
| A | PeopleOnStreet | 28,32,36,39,42 |  |  |
| A | Nebuta | 34,36,38,40,41 |  |  |
| A | SteamLocomotive | 24,26,28,30,32 |  |  |
| B | Kimono | 21,23,26,29,32 | 21,24,27,30,33 | 21,24,27,30,33 |
| B | ParkScene | 24,26,28,32,35 | 23,26,28,31,34 | 24,26,29,31,34 |
| B | Cactus | 24,26,28,31,34 | 24,26,28,31,34 | 25,26,29,31,34 |
| B | BQTerrace | 26,27,29,31,33 | 26,27,29,30,32 | 27,28,29,31,33 |
| B | BasketballDrive | 24,26,29,31,34 | 25,27,30,32,35 | 25,27,30,33,36 |
| C | RaceHorses | 27,30,33,36,38 | 28,31,34,36,38 | 28,31,34,36,38 |
| C | BQMall | 26,30,33,36,38 | 26,30,33,36,38 | 27,30,33,36,39 |
| C | PartyScene | 30,33,36,39,41 | 30,33,36,38,40 | 31,34,36,38,40 |
| C | BasketballDrill | 26,29,32,36,38 | 26,29,32,35,38 | 26,30,33,36,38 |
| D | RaceHorses | 20,24,28,30,33 | 21,25,28,30,33 | 21,25,29,31,33 |
| D | BQSquare | 22,25,28,30,33 | 24,27,29,31,33 | 25,27,30,31,33 |
| D | BlowingBubbles | 23,26,30,32,34 | 23,27,30,32,34 | 24,27,30,32,34 |
| D | BasketballPass | 22,26,30,32,35 | 23,27,31,33,36 | 23,27,31,33,36 |
| E | FourPeople |  | 24,27,31,33,36 | 24,27,31,33,36 |
| E | Johnny |  | 23,25,27,28,30 | 23,25,27,28,30 |
| E | KristenAndSara |  | 23,27,29,31,34 | 23,26,29,31,34 |
| F | BaskeballDrillText | 28,32,36,39,42 | 28,32,36,38,41 | 29,33,36,39,42 |
| F | ChinaSpeed | 31,35,39,41,44 | 31,35,38,40,42 | 31,35,38,40,42 |
| F | SlideEditing | 20,30,39,43,46 | 4,9,15,19,26 | 4,9,15,19,26 |
| F | SlideShow | 15,22,27,30,35 | 13,19,25,28,32 | 13,20,25,28,32 |

Table 9  JM18.4 QP settings for Cfp target rate points

| Class | Sequence name | Random access | Low-delayB | Low-delay P |
| --- | --- | --- | --- | --- |
| A | Traffic | 21,24,28,30,33 |  |  |
| A | PeopleOnStreet | 28,33,37,40,43 |  |  |
| A | Nebuta | 33,35,37,39,42 |  |  |
| A | SteamLocomotive | 23,26,29,31,34 |  |  |
| B | Kimono | 21,23,27,30,33 | 21,24,27,31,35 | 24,27,31,34,38 |
| B | ParkScene | 23,26,28,32,35 | 23,25,28,31,34 | 26,29,31,34,37 |
| B | Cactus | 24,26,29,32,35 | 24,26,29,32,34 | 27,29,32,35,38 |
| B | BQTerrace | 25,26,28,30,33 | 25,27,29,31,32 | 29,31,32,35,37 |
| B | BasketballDrive | 24,26,29,32,36 | 25,27,31,34,37 | 28,31,34,37,41 |
| C | RaceHorses | 27,30,34,36,39 | 27,31,34,36,39 | 31,34,37,40,42 |
| C | BQMall | 26,29,33,36,39 | 26,30,34,37,40 | 30,34,37,40,43 |
| C | PartyScene | 29,32,36,38,40 | 30,33,36,38,40 | 34,37,40,42,44 |
| C | BasketballDrill | 26,30,33,36,39 | 26,30,33,36,39 | 30,33,37,39,42 |
| D | RaceHorses | 20,24,28,30,33 | 21,25,28,30,33 | 24,28,32,34,36 |
| D | BQSquare | 22,25,28,30,32 | 24,27,29,31,33 | 28,31,34,35,37 |
| D | BlowingBubbles | 22,25,29,31,33 | 23,27,29,31,33 | 26,30,33,35,37 |
| D | BasketballPass | 22,26,29,32,35 | 23,27,30,33,36 | 26,30,34,36,39 |
| E | FourPeople |  | 23,27,31,33,37 | 26,30,34,36,40 |
| E | Johnny |  | 21,24,26,29,31 | 24,27,29,32,34 |
| E | KristenAndSara |  | 23,26,30,32,36 | 26,29,33,35,38 |
| F | BaskeballDrillText | 28,33,36,40,43 | 29,33,37,39,43 | 32,36,40,43,46 |
| F | ChinaSpeed | 31,35,39,41,44 | 31,35,38,40,43 | 35,39,42,44,46 |
| F | SlideEditing | 23,32,39,43,46 | 4,11,18,22,28 | 7,13,19,24,30 |
| F | SlideShow | 16,23,28,32,36 | 13,20,25,28,32 | 16,23,28,32,36 |