| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0273 |
| --- | --- |

| Title: | AHG6: ALF in HM80 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Tomoo Yamakage, Takayuki Itoh, Takeshi Chujoh TOSHIBA 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | Marta Karczewicz +1-858-658-5673 {ichong, martak} @qualcomm.com Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai, chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Tomoo Yamakage +81-44-549-2152 {tomoo.yamakage, takayuki2.itoh, takeshi.chujoh} @toshiba.co.jp |
| Source: | Qualcomm Inc., MediaTek Inc., TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this contribution, we first evaluate ALF in HM80. Compared with ALF-off, the ALF coding gain is 1.3%/2.4%/1.1%/2.2% for Main-AI/RA/LB/LP and 1.4%/2.8%/2.1%/3.3% for HE10-AI/RA/LB/LP. Furthermore, we evaluate ALF in JCTVC-J0390 on top of HM80. Compared with ALF-off, the ALF coding gain is 1.4%/2.8%/2.1%/3.5% for Main-AI/RA/LB/LP and 1.5%/3.1%/2.5%/4.1% for HE10-AI/RA/LB/LP. For key technical area sequences (i.e., 720p or higher resolution), the ALF coding gain is 3.9% (Y) / 5.4% (Cb) /4.8% (Cr). We propose to keep ALF in the reference software (i.e., HM90) with improvements made in the JCTVC-J0390 to help development of HEVC extensions and 2nd version HEVC profiles.

# Introduction

In the Stockholm meeting, ALF was impoved in the aspect of both performance and complexity even though ALF was not adopted as one of tools in the Main profile. But in this meeting many SVC proposals include ALF (i.e., 11 out of 20), and in the other profiles (i.e., 4:2:2, 4:4:4, or 10bit or higher bitdepth profiles) ALF may be considered. In this proposal, we evaluate the performance of ALF in HM80 as well as improved ALF which was proposed in JCTVC-J0390 [1].

# ALF modification and clean ups in JCTVC-J0390

Several changes were introduced in JCTVC-J0390 to improve and clean up ALF, which already were verified in the Stockholm meeting. Those should be useful for the development of other extensions of HEVC such as SVC and other 2nd version profiles in the HEVC. The following items are the list of modifications.

- non-normative encoder-only improvements

- bug-fixes (i.e., ticket #574) and clean ups

- Reduction of the filter coefficient precision related changes

  - from 9-bit to 7-bit

  - Sum of positive non-center filter coefficients times 510 plus center filter coefficient times 255 shall be in the range of [0, 215-1-32).

  - Sum of negative filter coefficients times 510 shall be in the range of [-215, 0).

- Reduction of filter shape from cross9x7+square3x3 to cross7x7+square3x3

- Not applying filtering across slice/tile boundaries

Non-normative changes in item 1) mostly improves the coding gain of ALF, and normative changes in item 3)-5) make ALF more implementation friendly. For example, with changes in 3), the multiplier output can be represented within 16 bits, and the 16-bit accumulation is also doable, which will make ALF more software-implementation friendly.

# Simulation results

Simulations were conducted according to the common test conditions (CTC) defined in JCTVC-J1100 [2] with ALF-off as the anchor. Results of using the current picture to derive filter coefficients are shown. Table 1 shows the BD-rates of HM80 ALF using CTC sequences.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.3% | -1.6% | -1.6% | -2.4% | -1.9% | -1.9% |
| Class B | -1.3% | -2.3% | -2.3% | -1.4% | -2.5% | -2.7% |
| Class C | -0.9% | -2.5% | -3.1% | -1.1% | -3.1% | -3.7% |
| Class D | -0.3% | -2.3% | -2.7% | -0.4% | -2.8% | -3.2% |
| Class E | -1.9% | -7.8% | -8.6% | -2.0% | -8.2% | -9.0% |
| Overall | -1.3% | -3.0% | -3.3% | -1.4% | -3.4% | -3.8% |
|  | -1.3% | -3.0% | -3.3% | -1.4% | -3.4% | -3.8% |
| Class F | -0.5% | -1.1% | -1.0% | -0.6% | -1.4% | -1.3% |
| Enc Time[%] | 103% |  |  | 109% |  |  |
| Dec Time[%] | 113% |  |  | 113% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -3.8% | -4.0% | -5.0% | -4.1% | -5.4% | -6.3% |
| Class B | -2.1% | -3.1% | -1.9% | -2.7% | -4.6% | -3.2% |
| Class C | -1.6% | -1.6% | -1.7% | -2.0% | -2.2% | -2.2% |
| Class D | -2.3% | -0.7% | -0.6% | -2.6% | -1.2% | -1.0% |
| Class E |  |  |  |  |  |  |
| Overall | -2.4% | -2.4% | -2.3% | -2.8% | -3.4% | -3.2% |
|  | -2.4% | -2.4% | -2.3% | -2.8% | -3.4% | -3.3% |
| Class F | -0.7% | -1.2% | -1.0% | -0.8% | -1.6% | -1.3% |
| Enc Time[%] | 101% |  |  | 107% |  |  |
| Dec Time[%] | 111% |  |  | 110% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.5% | -2.7% | -2.4% | -2.2% | -5.6% | -4.7% |
| Class C | -1.7% | -2.4% | -2.4% | -2.4% | -3.9% | -3.8% |
| Class D | -1.4% | -0.1% | -0.9% | -1.7% | -1.4% | -1.6% |
| Class E | 0.5% | -5.8% | -6.0% | -2.0% | -7.2% | -8.3% |
| Overall | -1.1% | -2.6% | -2.7% | -2.1% | -4.4% | -4.4% |
|  | -1.1% | -2.5% | -2.7% | -2.1% | -4.5% | -4.5% |
| Class F | -1.9% | -1.5% | -1.1% | -2.7% | -2.8% | -3.0% |
| Enc Time[%] | 102% |  |  | 106% |  |  |
| Dec Time[%] | 110% |  |  | 108% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -4.7% | -5.3% | -3.8% | -5.9% | -8.6% | -7.7% |
| Class C | -1.5% | -2.3% | -2.5% | -2.4% | -3.6% | -4.1% |
| Class D | -0.3% | -0.3% | -1.3% | -0.7% | -1.8% | -1.0% |
| Class E | -1.4% | -7.1% | -8.3% | -3.6% | -11.0% | -11.3% |
| Overall | -2.2% | -3.6% | -3.7% | -3.3% | -6.1% | -5.8% |
|  | -2.2% | -3.6% | -3.7% | -3.3% | -6.1% | -5.7% |
| Class F | -1.3% | -1.3% | -0.7% | -1.9% | -1.8% | -1.4% |
| Enc Time[%] | 95% |  |  | 106% |  |  |
| Dec Time[%] | 111% |  |  | 109% |  |  |

Table 1 BD-rates of HM80 ALF

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.3% | -1.6% | -1.5% | -2.4% | -1.8% | -1.6% |
| Class B | -1.4% | -2.4% | -2.4% | -1.5% | -2.6% | -2.7% |
| Class C | -0.9% | -2.7% | -3.3% | -1.0% | -3.1% | -3.8% |
| Class D | -0.4% | -2.4% | -2.7% | -0.5% | -2.9% | -3.2% |
| Class E | -2.2% | -7.7% | -8.5% | -2.4% | -8.1% | -9.0% |
| Overall | -1.4% | -3.1% | -3.4% | -1.5% | -3.4% | -3.7% |
|  | -1.4% | -3.0% | -3.3% | -1.5% | -3.3% | -3.7% |
| Class F | -0.5% | -1.0% | -1.0% | -0.5% | -1.2% | -1.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 113% |  |  | 113% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -4.2% | -3.0% | -3.9% | -4.3% | -3.9% | -4.8% |
| Class B | -2.7% | -3.0% | -1.6% | -3.1% | -4.2% | -2.6% |
| Class C | -1.8% | -1.9% | -1.9% | -2.1% | -2.2% | -2.3% |
| Class D | -2.6% | -1.5% | -1.5% | -2.9% | -1.7% | -1.8% |
| Class E |  |  |  |  |  |  |
| Overall | -2.8% | -2.4% | -2.2% | -3.1% | -3.0% | -2.9% |
|  | -2.8% | -2.4% | -2.2% | -3.1% | -3.0% | -2.9% |
| Class F | -0.7% | -1.1% | -1.0% | -0.8% | -1.4% | -1.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 111% |  |  | 110% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -2.3% | -2.8% | -2.1% | -2.6% | -5.3% | -4.0% |
| Class C | -2.0% | -2.7% | -2.4% | -2.5% | -3.9% | -4.0% |
| Class D | -1.7% | -0.9% | -2.2% | -1.8% | -2.9% | -2.6% |
| Class E | -2.3% | -6.1% | -5.9% | -2.9% | -6.7% | -7.2% |
| Overall | -2.1% | -2.9% | -2.9% | -2.5% | -4.6% | -4.3% |
|  | -2.1% | -2.9% | -2.8% | -2.4% | -4.5% | -4.3% |
| Class F | -2.0% | -2.4% | -2.0% | -2.8% | -3.0% | -3.3% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 110% |  |  | 108% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -5.7% | -5.3% | -4.1% | -6.6% | -8.6% | -7.5% |
| Class C | -2.2% | -2.7% | -2.7% | -2.8% | -4.3% | -4.5% |
| Class D | -0.9% | -0.6% | -2.2% | -1.1% | -2.8% | -2.3% |
| Class E | -4.6% | -7.9% | -8.3% | -5.4% | -11.4% | -10.4% |
| Overall | -3.4% | -4.0% | -4.0% | -4.1% | -6.6% | -6.0% |
|  | -3.4% | -3.9% | -3.9% | -4.1% | -6.5% | -5.8% |
| Class F | -1.7% | -1.9% | -1.2% | -2.2% | -2.8% | -2.2% |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | 111% |  |  | 109% |  |  |

Table 2 BD-rates of ALF in JCTVC-0390

Table 2 shows the BD-rates of ALF in JCTVC-0390 on top of HM80. By using modifications in JCTVC-0390, coding gain is improved by 0.1%/0.4%/1.0%/1.3% for Main-AI/RA/LB/LP and 0.1%/0.3%/0.4%/0.8% for HE10-AI/RA/LB/LP.

And the encoding time increase is 1-3%, while the decoding time increase is 10%. Note that we also provide SIMD instructions based filtering engine, which may speed up ALF by ~ 4 times (i.e., decoding time increase was shown to be reduced from 10% to 2.3% [1]).

# Conclusions

In this contribution, we show the gain of ALF in HM80 as well as the gain of improved ALF proposed in JCTVC-J0390, which includes cleanups, simplifications, and non-normatively improvement of ALF. These changes improve ALF performance (i.e., 0.1%/0.4%/1.0%/1.3% for Main-AI/RA/LB/LP and 0.1%/0.3%/0.4%/0.8% for HE10-AI/RA/LB/LP), while making ALF more implementation friendly. We propose to keep ALF in the reference software (i.e., HM90) with improvements made in the JCTVC-J0390 to help development of HEVC extensions and 2nd version HEVC profiles.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, T. Yamakage, T. Itoh, T. Chujoh, I. S. Chong, M. Karczewicz, “AHG6: Further cleanups and simplifications for the ALF in JCTVC-J0048,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0390, July, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, Document of Joint Collaborative Team on Video Coding, JCTVC-J1100, July, 2012.