| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0274 |
| --- | --- |

| Title: | VPS extension design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

JCTVC-K0274 proposes syntax and semantics for the vps_extension syntax structure for scalable, multiview and 3D extensions. The proposal includes two parts:

- Indication of the mapping of the 6-bit layer_id (reserved_zero_6bits in HEVC v1) to scalability dimension identifiers. This part of the proposal was initiated by LG Electronics and discussed over email and is therefore similar to JCTVC-K0169. It is asserted that the only essential difference between JCTVC-K0274 and JCTVC-K0169 is the presence of nuh_layer_id[ i ] syntax element in the VPS extension proposal of JCTVC-K0274, which reportedly enables more flexible allocation of layer_id values – e.g., layer_id can be a combination of bit-fields of other scalability identifiers.

- Indication of a profile-tier-level combination for operation points.

# Background

The proposed indication of profile-tier-level combination for operation points uses an operation point index, which is proposed in JCTVC-K0180. Thus, this section briefly summarizes the part of JCTVC-K0180 that proposes indicating operation points in the VPS syntax.

In JCTVC-K0180 we propose that operation points are specified as a separate loop in VPS rather than as part of the loop for the HRD parameters. The listed operation points are indexed in their appearance order, and the index is used to refer to a particular operation point from other structures. The change enables

- including HRD parameters for a different number of operation points than the total list of operation points, and

- referring to operation points specified in VPS even if no HRD parameters were provided for each operation point.

The VPS syntax proposed in JCTVC-K0180 is as follows (with changes indicated with red font color):

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| max_num_layers_minus1 //reserved_zero_6bits in the base spec | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| next_essential_info_byte_offset //reserved_zero_12bits in the base spec | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| num_op_points_minus1 | ue(v) |
| for( i = 1; i <= num_op_points_minus1; i++ ) { |  |
| operation_point( i ) |  |
| } |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_op_point_idx | ue(v) |
| if( i > 0 ) |  |
| operation_point( i ) |  |
| hrd_parameters( hrd_op_point_idxi = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposal

## Syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| num_dimensions_minus1 | ue(v) |
| for( i = 0; i <= num_dimensions_minus1; i++ ) { |  |
| dimension_type[ i ] | u(4) |
| dimension_len[ i ] | u(8) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(v) |
| nuh_layer_id[ i ] | u(6) |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| num_profile_level | ue(v) |
| for( i = 1; i <= num_profile_level + 1; i++ ) { |  |
| prfl_lvl_num_op_points_minus1[ i ] | ue(v) |
| for( j = 0; j <= prfl_lvl_num_op_points_minus1[ i ]; j ++ ) |  |
| prfl_lvl_op_point_idx[ i ][ j ] | ue(v) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |

## Semantics

vps_extension_byte_alignment_reserved_one_bit shall be equal to 1.

num_dimensions_minus1 plus 1 specifies the number of dimension types.

dimension_type[ i ] specifies the i-th scalability dimension type, as specified in the following table:

| dimension_type | dimension_id[ dimension_type ] |
| --- | --- |
| 0 | view order idx |
| 1 | depth flag |
| 2 | dependency ID |
| 3 | quality ID |
| 4..15 | reserved |

The value of dimension_type[ i ] shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this Recommendation | International Standard. Other values for dimension_type[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore values of dimension_type[ i ] that are not in the range of 0 to 3, inclusive.

dimension_len[ i ] specifies the number of bits of the i-th scalability dimension identifier within the combined VPS layer identifier.

vps_layer_id[ i ] specifies the values of DimensionId[ dimType ] for values of dimType in the range of 0 to num_dimensions_minus1, inclusive, as follows:

vpsLayerId = vps_layer_id[ i ]
for( j = num_dimensions_minus1; j >= 0; j-- ) {
	DimensionId[ j ] = vpsLayerId & ( ( 1 << dimension_len[ j ] ) – 1 )
	vpsLayerId = vpsLayerId >> dimension_len[ j ]
}

NOTE: In other words, vps_layer_id[ i ] is a bit-field of identifiers dimension_id[ dimension_type[ num_dimensions_minus1 ] ] .. dimension_id[ dimension_type[ 0 ] ] from most significant bit to least significant bit, where dimension_len[ i ] indicates the length of each dimension_id field in the bit-field.

nuh_layer_id[ i ] specifies that when the value of layer_id in the NAL unit header is equal to nuh_layer_id[ i ], values of DimensionId[ i ] are in effect in the decoding process.

num_direct_ref_layers[ i ] specifies the number of layers the i-th layer directly depends on.

ref_layer_id[ i ][ j ] identifies the j-th layer the i-th layer directly depends on.

ref_layer_id[ i ][ j ] identifies the j-th layer the i-th layer directly depends on.

num_profile_level specifies the number of profile_tier_level( ) syntax structures present in the VPS extension.

prfl_lvl_num_op_points_minus1[ i ] specifies the number of operation points for which the i-th profile_tier_level( ) syntax structure present in the VPS extension applies to.

prfl_lvl_op_point_idx[ i ][ j ], where j is in the range 0 to prfl_lvl_num_op_points_minus1[ i ], inclusive, specifies the operation points for which the i-th profile_tier_level( ) syntax structure present in the VPS extension applies to.

# Relation of JCTVC-K0274 to JCTVC-K0169

The syntax to indicate the mapping of layer_id to scalability dimensions was proposed by LG Electronics and refined over an email discussion. Hence, from this aspect JCTVC-K0274 is similar to JCTVC-K0169. It is asserted that the only essential difference is the nuh_layer_id[ i ] syntax element, which is present in the syntax proposed in JCTVC-K0274 and absent in the syntax of JCTVC-K0169.

In JCTVC-K0274, nuh_layer_id[ i ] indicates the layer_id value in the NAL unit header for the combination of scalability dimension identifiers derived from vps_layer_id[ i ]. This allows flexible allocation of layer_id values, e.g. layer_id can be a combination of bit-fields of other scalability identifiers, while it also enables “tight” allocation of layer_id values.

In JCTVC-K0169, it seems that the loop counter i in “for (i = 1; i <= vps_max_layers_minus1; i++)” indicates the NAL unit header layer_id value corresponding to vps_layer_id[ i ], i.e. layer_id values are always “tightly” allocated and layer_id specified as a combination of bit-fields of scalability identifiers is not generally possible.

The use of nuh_layer_id[ i ] is explained below with two examples.

In the first example, the following integer-bit partitioning of layer_id to other scalability identifiers is desired by the encoder: 1 unused bit (MSB of layer_id), 3 bits for dependency_id, 2 bits (LSBs) for quality_id. The bitstream has fewer layers/dimension than the integer-bit partitioning would allow: 5 spatial/CGS layers (i.e. dependency_id is in the range of 0 to 4, inclusive), 3 MGS layers (i.e., quality_id is in the range of 0 to 2, inclusive). As the total number for scalability identifiers is smaller than or equal to 6 (i.e. the sum of dimension_len[ i ] with all values of i <= 6), nuh_layer_id[ i ] is set equal to vps_layer_id[ i ]. This is illustrated below with pseudo-code:

| video_parameter_set_rbsp( ) { | Value |
| --- | --- |
| … |  |
| max_num_layers_minus1 | 14 |
| … |  |

| vps_extension( ) { | Value |
| --- | --- |
| … |  |
| num_dimensions_minus1 | 1 |
| // for( i = 0; i <= num_dimensions_minus1; i++ ) { |  |
| dimension_type[ 0 ] | 2 |
| dimension_len[ 0 ] | 4 |
| dimension_type[ 1 ] | 3 |
| dimension_len[ 1 ] | 2 |
| // for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ 1 ] // dependency_id = 0, quality_id = 1 | 000001b |
| nuh_layer_id[ 1 ] | 000001b |
| … |  |
| vps_layer_id[ 2 ] // // dependency_id = 0, quality_id = 2 | 000010b |
| nuh_layer_id[ 2 ] | 000010b |
| … |  |
| vps_layer_id[ 3 ] // // dependency_id = 1, quality_id = 0 | 000100b |
| nuh_layer_id[ 3 ] | 000100b |
| … |  |

In the second example,

- There are 3 views in the bitstream (which is a common test condition in JCT-3V)

- There are both texture views and depth views (a common test condition in JCT-3V)

- Texture view components have two spatial enhancement layers (assuming that multiview and scalable coding would be combined)

- Each spatial layer has three quality layers

- There are no spatial or quality enhancement for the depth views

Thus, there are:

- For texture: 3 views x 3 spatial scalability layers x 3 quality layers = 27 layer representations

- For depth: 3 views

- Altogether: 30 layer representations

In the bit partitioning approach, the number of bits needed would be:

- View_id: 2 bits

- Depth_flag: 1 bit

- Dependency_id: 2 bits

- Quality_id: 2 bits

- Altogether: 7 bits, which is the length of vps_layer_id[ i ] in this example.

In the example, the depth_flag occupies the MSB of the layer_id in the NAL unit header, while view_id, dependency_id and quality_id share the values of the 5 LSBs of layer_id. Other layer_id value allocations would have been possible too (and are generally up to the encoder to be determined).

The pseudo-code for the second example is as follows:

| video_parameter_set_rbsp( ) { | Value |
| --- | --- |
| … |  |
| max_num_layers_minus1 | 29 |
| … |  |

| vps_extension( ) { | Value |
| --- | --- |
| … |  |
| num_dimensions_minus1 | 3 |
| // for( i = 0; i <= num_dimensions_minus1; i++ ) { |  |
| dimension_type[ 0 ] | 0 // voidx |
| dimension_len[ 0 ] | 2 |
| dimension_type[ 1 ] | 1 // depth_flag |
| dimension_len[ 1 ] | 1 |
| dimension_type[ 2 ] | 2 // dependency_id |
| dimension_len[ 2 ] | 2 |
| dimension_type[ 3 ] | 3 // quality_id |
| dimension_len[ 3 ] | 2 |
| // for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ 1 ] | 00-0-00-01b |
| nuh_layer_id[ 1 ] | 1 |
| … |  |
| vps_layer_id[ 2 ] | 00-0-00-10b |
| nuh_layer_id[ 2 ] | 2 |
| … |  |
| vps_layer_id[ 3 ] | 00-0-01-00b |
| nuh_layer_id[ 3 ] | 3 |
| … |  |
| vps_layer_id[ 4 ] | 00-0-01-01b |
| nuh_layer_id[ 4 ] | 4 |
| … |  |
| vps_layer_id[ 5 ] | 00-0-01-10b |
| nuh_layer_id[ 5 ] | 5 |
| … |  |
| vps_layer_id[ 9 ] | 01-0-00-00b |
| nuh_layer_id[ 9 ] | 9 |
| … |  |
| vps_layer_id[ 17 ] | 01-0-10-10b |
| nuh_layer_id[ 17 ] | 17 |
| … |  |
| vps_layer_id[ 18 ] | 10-0-00-00b |
| nuh_layer_id[ 18 ] | 18 |
| … |  |
| vps_layer_id[ 26 ] | 10-0-10-10b |
| nuh_layer_id[ 26 ] | 26 |
| … |  |
| vps_layer_id[ 27 ] | 00-1-00-00b |
| nuh_layer_id[ 27 ] | 32 |
| … |  |
| vps_layer_id[ 28 ] | 01-1-00-00b |
| nuh_layer_id[ 28 ] | 33 |
| … |  |
| vps_layer_id[ 29 ] | 10-1-00-00b |
| nuh_layer_id[ 29 ] | 34 |
| … |  |

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).