| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0275 |
| --- | --- |

| Title: | Cross check of Suppression of blocking artifacts at large TU boundaries(JCTVC-K0269) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shuo Lu Ohji Nakagami Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Shuo.Lu@jp.sony.com ohji.nakagami@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document reports the cress-check results of JCTVC-K0269. It is confirmed that source code exactly reflects the proposal. BD-rate matches with what provided by the proposer too.

# Introduction

JCTVC-K0269 is proposed to reduce blocking artifacts at large TU boundaries by applying the largest values of beta_offset_div2 and tc_offset_div2 to the maximum TU boundaries and disabling intra-prediction boundary filtering at 32x32 blocks.

The following changes on syntax are introduced:

max_tu_offset_flag is added to enable/disable the proposed method.

Table 1. Picture parameter set syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| max_tu_offset_flag | u(1) |
| } |  |
| } |  |
| …. |  |
| } |  |

Table 2. Slice header syntax

| Slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| max_tu_offset_flag | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

# Results

The tests are run with the following configuration:

HM8.0:

DeblockingFilterControlPresent  : 0           # Dbl control params present (0=not present, 1=present)

LoopFilterOffsetInPPS         : 0           # Dbl params in PPS or SliceHeader (0=SliceHeader, 1=PPS)

LoopFilterDisable            : 0           # Disable deblocking filter in slice header (0=Filter, 1=No Filter)

LoopFilterBetaOffset_div2     : 0           # Range: -6 ~ 6

LoopFilterTcOffset_div2       : 0           # Range: -6 ~ 6

HM8.0_with_MaxOffset:

DeblockingFilterControlPresent  : 1           # Dbl control params present (0=not present, 1=present)

LoopFilterOffsetInPPS         : 1           # Dbl params in PPS or SliceHeader (0=SliceHeader, 1=PPS)

LoopFilterDisable            : 0           # Disable deblocking filter in slice header (0=Filter, 1=No Filter)

LoopFilterBetaOffset_div2     : 6           # Range: -6 ~ 6

LoopFilterTcOffset_div2       : 6           # Range: -6 ~ 6

Proposal:

DeblockingFilterControlPresent  : 1           # Dbl control params present (0=not present, 1=present)

LoopFilterOffsetInPPS         : 1           # Dbl params in PPS or SliceHeader (0=SliceHeader, 1=PPS)

LoopFilterDisable            : 0           # Disable deblocking filter in slice header (0=Filter, 1=No Filter)

LoopFilterBetaOffset_div2     : 6           # Range: -6 ~ 6

LoopFilterTcOffset_div2       : 6           # Range: -6 ~ 6

MaxTuOffsetFlag             : 1           # Maximum TU offset flag

## Objective results:

Table 3. Proposal vs. HM8.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.4% | 0.8% | 1.0% | 1.5% | 0.9% | 1.0% |
| Class B | 0.6% | 0.4% | 0.4% | 0.7% | 0.4% | 0.4% |
| Class C | 0.2% | 0.3% | 0.2% | 0.2% | 0.3% | 0.2% |
| Class D | 0.2% | 0.1% | 0.1% | 0.2% | 0.2% | 0.1% |
| Class E | 0.6% | 0.1% | 0.0% | 0.6% | 0.2% | 0.0% |
| Class F | 0.7% | 0.3% | 0.4% | 0.8% | 0.2% | 0.6% |
| Overall | 0.6% | 0.3% | 0.4% | 0.6% | 0.4% | 0.4% |
|  | 0.6% | 0.4% | 0.4% | 0.6% | 0.4% | 0.4% |
| Enc Time[%] | 107% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 97% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.7% | 0.2% | 0.4% | 0.6% | 0.3% | 0.3% |
| Class B | 0.5% | 0.3% | 0.4% | 0.6% | 0.3% | 0.5% |
| Class C | 0.5% | 0.3% | 0.4% | 0.6% | 0.2% | 0.1% |
| Class D | 0.2% | -0.1% | 0.2% | 0.2% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.7% | 0.1% | 0.3% | 0.8% | 0.5% | 0.6% |
| Overall | 0.5% | 0.2% | 0.4% | 0.5% | 0.2% | 0.2% |
|  | 0.5% | 0.2% | 0.4% | 0.5% | 0.2% | 0.2% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 94% |  |  | 88% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.7% | 0.5% | 0.3% | 0.9% | 0.5% | 0.8% |
| Class C | 0.8% | 0.0% | 0.1% | 0.9% | 0.2% | 0.3% |
| Class D | 0.3% | 0.2% | -0.3% | 0.3% | -0.1% | -0.2% |
| Class E | 1.5% | 1.0% | 1.2% | 2.0% | 0.7% | 0.9% |
| Class F | 1.0% | 0.6% | 0.4% | 0.9% | 0.6% | -0.5% |
| Overall | 0.8% | 0.4% | 0.3% | 0.9% | 0.4% | 0.2% |
|  | 0.8% | 0.4% | 0.3% | 0.9% | 0.3% | 0.3% |
| Enc Time[%] | 108% |  |  | 103% |  |  |
| Dec Time[%] | 95% |  |  | 83% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.3% | 0.6% | 0.3% | 0.3% | 0.1% |
| Class C | 0.6% | 0.1% | 0.2% | 0.7% | 0.0% | 0.1% |
| Class D | 0.2% | 0.3% | -0.2% | 0.3% | -0.5% | 0.5% |
| Class E | 1.1% | 1.1% | -0.2% | 1.6% | 0.2% | 0.5% |
| Class F | 0.8% | 0.7% | 1.1% | 0.9% | 0.3% | 0.1% |
| Overall | 0.5% | 0.5% | 0.3% | 0.7% | 0.1% | 0.2% |
|  | 0.5% | 0.4% | 0.3% | 0.7% | 0.1% | 0.3% |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 92% |  |  | 83% |  |  |

Table 4.  HM8.0_with_MaxOffset vs. HM8.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 4.1% | 2.9% | 2.9% | 4.1% | 2.9% | 2.9% |
| Class B | 5.3% | 3.7% | 3.6% | 5.3% | 3.6% | 3.6% |
| Class C | 4.5% | 5.2% | 5.1% | 4.6% | 5.2% | 5.0% |
| Class D | 4.2% | 4.7% | 5.3% | 4.2% | 4.7% | 5.3% |
| Class E | 3.9% | 2.0% | 0.6% | 3.9% | 2.0% | 0.6% |
| Class F | 3.9% | 6.8% | 8.2% | 3.9% | 6.8% | 8.2% |
| Overall | 4.4% | 4.3% | 4.4% | 4.5% | 3.8% | 3.6% |
|  | 4.3% | 4.3% | 4.4% | 4.4% | 3.8% | 3.7% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 94% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 2.5% | 1.9% | 2.0% | 2.6% | 2.0% | 2.0% |
| Class B | 4.8% | 2.9% | 3.0% | 5.0% | 2.9% | 3.1% |
| Class C | 4.3% | 4.4% | 4.4% | 4.4% | 4.5% | 4.3% |
| Class D | 4.5% | 3.9% | 4.4% | 4.5% | 4.0% | 4.5% |
| Class E |  |  |  |  |  |  |
| Class F | 5.5% | 7.8% | 9.2% | 5.8% | 8.1% | 9.1% |
| Overall | 4.1% | 3.3% | 3.4% | 4.2% | 3.3% | 3.4% |
|  | 4.1% | 3.3% | 3.4% | 4.2% | 3.3% | 3.4% |
| Enc Time[%] | 105% |  |  | 102% |  |  |
| Dec Time[%] | 93% |  |  | 86% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 5.4% | 1.8% | 1.7% | 5.8% | 1.9% | 2.4% |
| Class C | 4.6% | 1.7% | 1.9% | 4.7% | 2.1% | 2.3% |
| Class D | 5.3% | 2.2% | 1.6% | 5.5% | 1.9% | 2.2% |
| Class E | 7.7% | 3.8% | 3.5% | 8.7% | 4.6% | 3.4% |
| Class F | 6.3% | 5.2% | 5.9% | 6.4% | 5.2% | 5.5% |
| Overall | 5.7% | 2.9% | 2.8% | 6.1% | 3.0% | 3.1% |
|  | 5.7% | 2.9% | 2.9% | 6.1% | 3.0% | 3.1% |
| Enc Time[%] | 103% |  |  | 102% |  |  |
| Dec Time[%] | 90% |  |  | 82% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 3.3% | 1.0% | 1.1% | 3.2% | 1.4% | 1.0% |
| Class C | 3.6% | 1.7% | 1.7% | 3.5% | 1.7% | 1.8% |
| Class D | 4.3% | 1.8% | 1.5% | 4.4% | 1.7% | 2.5% |
| Class E | 5.3% | 3.1% | 1.7% | 5.7% | 2.6% | 2.2% |
| Class F | 5.7% | 4.9% | 6.5% | 5.8% | 5.6% | 5.6% |
| Overall | 4.3% | 2.4% | 2.5% | 4.4% | 2.5% | 2.6% |
|  | 4.3% | 2.4% | 2.4% | 4.3% | 2.5% | 2.6% |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 90% |  |  | 83% |  |  |

Table 5.  Proposal of type-1 sequences (LB-Main)

|  | sequence | QP | kbps | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- |
| K0269 | riverbed | 32 | 5854.2968 | 34.6336 | 38.9586 | 41.3974 |
|  |  | 37 | 2870.9864 | 31.7327 | 38.1326 | 40.896 |
|  |  | 41 | 1566.296 | 29.5737 | 37.6952 | 40.607 |
|  | DucksTakeOff | 32 | 12334.0416 | 30.3299 | 32.234 | 36.4689 |
|  |  | 37 | 5104.1096 | 27.5133 | 31.2302 | 35.9606 |
|  |  | 41 | 2510.3712 | 25.6471 | 30.6315 | 35.675 |
|  | RedKayak | 32 | 4595.7192 | 36.421 | 41.9048 | 44.8785 |
|  |  | 37 | 2123.5756 | 33.4837 | 40.9827 | 44.0272 |
|  |  | 41 | 1052.8611 | 31.3472 | 40.3621 | 43.3255 |
|  | WestWindEasy | 32 | 3754.8848 | 36.2124 | 44.7091 | 47.4805 |
|  |  | 37 | 1675.0928 | 33.0148 | 43.6389 | 46.6726 |
|  |  | 41 | 854.208 | 30.7521 | 43.0374 | 45.9835 |
|  | F.ChinaSpeed | 32 | 1273.0483 | 35.2675 | 40.2292 | 40.186 |
|  |  | 37 | 613.5739 | 31.9465 | 38.6979 | 38.3985 |
|  |  | 41 | 317.3035 | 29.3795 | 37.7641 | 36.8312 |

Table 6.  Proposal of type-1 sequences (RA-Main)

|  | sequence | QP | kbps | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- |
| K0269 | riverbed | 32 | 4772.0664 | 33.5564 | 38.6444 | 41.2726 |
|  |  | 37 | 2356.9936 | 30.835 | 37.9975 | 40.8822 |
|  |  | 41 | 1277.9232 | 28.7807 | 37.5746 | 40.6373 |
|  | DucksTakeOff | 32 | 9215.9048 | 29.549 | 32.4595 | 36.7275 |
|  |  | 37 | 3954.7872 | 27.2624 | 31.5372 | 36.2683 |
|  |  | 41 | 1995.9352 | 25.6902 | 30.9555 | 35.973 |
|  | RedKayak | 32 | 3712.9082 | 35.6361 | 41.8184 | 44.9149 |
|  |  | 37 | 1717.4615 | 32.9438 | 41.0096 | 44.1134 |
|  |  | 41 | 854.4733 | 30.9692 | 40.4265 | 43.494 |
|  | WestWindEasy | 32 | 3164.1336 | 35.3308 | 44.8991 | 47.6579 |
|  |  | 37 | 1487.6856 | 32.5121 | 43.8805 | 46.8012 |
|  |  | 41 | 794.144 | 30.4745 | 43.2012 | 46.2002 |
|  | F.ChinaSpeed | 32 | 1323.9782 | 35.0352 | 40.7274 | 40.6025 |
|  |  | 37 | 682.2317 | 31.8807 | 39.2387 | 39.0555 |
|  |  | 41 | 396.229 | 29.5413 | 38.2504 | 37.4182 |

## Subjective results:

# Patent rights declaration(s)

Sony does not have any current or pending patent rights relating to the technology described in this contribution.