| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0276 |
| --- | --- |

| Title: | Parameter set design for HEVC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thomas Rusert Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 76 7602660 thomas.rusert@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a parameter set design for multi-layer HEVC extensions, i.e. the 3D video and scalability extensions. The proposal is reportedly based on the HEVC extensions NAL unit header and parameter set design “straw man” as described in JCTVC-J1007. It includes syntax and semantics for VPS extension, syntax and semantics for SPS NAL units that have layer_id greater than 0 and text for parameter set activation. No changes to the base specification are proposed.

In the proposed design, up to one SPS and up to one PPS are active for each value of layer_id.

The specific VPS extension design is claimed to be based on the following design criteria:

- The VPS should include parameters that are useful for bitstream extraction and/or capability exchange.

- Parameters that are not required for bitstream extraction and/or capability exchange should not be present in the VPS.

- Sequence-level parameters that may change in some layers while not affecting the general layer structure should not be present in the VPS. Such parameters can be present in the SPS instead.

The SPS design for SPS NAL units with layer_id greater than 0 is claimed to be based on the following design criteria:

- Parameters present in the VPS should not be duplicated in the SPS (except for those parameters that are required for base layer decoding).

- Sequence-level parameters that are not present in the VPS should be included in the SPS.

# Introduction

This contribution proposes a parameter set design for multi-layer HEVC extensions, i.e. the 3D video and scalability extensions. First, a basic parameter set design for multi-layer HEVC extensions is proposed. Based on that, criteria for the specific design of VPS and SPS are discussed, and syntax and semantics for SPS and VPS are proposed.

The proposal is based on the HEVC extensions NAL unit header and parameter set design “straw man” described in JCTVC-J1007.

In the following the term “layer” denotes a bit stream subset containing NAL units that have layer_id equal to the same value LId. In particular the subset of NAL units that have layer_id equal to 0 is denoted as “base layer”, and a subset of NAL units that have layer_id equal to a value greater than zero is denoted as “enhancement layer”. The base layer is compliant with the HEVC base specification. An enhancement layer may represent a certain coded view (video or depth data) in the context of 3D-HEVC, or a video representation at a certain spatial resolution and quality level in the context of a scalable HEVC extension (or potentially a combination thereof).

# Basic parameter design for HEVC extensions

The following basic parameter set design is proposed for multi-layer HEVC extensions:

- The value of the syntax element layer_id in the NAL unit header of parameter set NAL units is assigned such as to allow for clean bitstream extraction using the HEVC sub-bitstream extraction process [section 10.1 in JCTVC-J1003_d7]. Specifically:

  - layer_id equal to 0 for VPS NAL units

  - layer_id greater than or equal to 0 for SPS and PPS NAL units

- Activation of parameter sets for layers above the base layer is performed in the same way as for the base layer, i.e. a VPS is activated by reference from a SPS which is activated by reference from a PPS which is activated by reference from a slice header of a coded slice.

- As is in the base specification, up to one VPS is active during decoding of a coded video sequence, regardless of whether one or several layers are decoded.

- Up to one SPS and up to one PPS is active for each value of layer_id. A SPS (or PPS) NAL unit containing a sequence parameter set RBSP (picture parameter set RBSP) that is activated for a layer with layer_id equal to LId shall have a layer_id less than or equal to LId.

Furthermore, the following earlier recommendations by the JCT-VC are adhered:

- The base layer is decodable without requiring parsing of the VPS.

- The VPS extension carries layer dependency information.

- VPS parameters should be coded as fixed-length codes as far as possible such as to facilitate low-complex parsing.

# Criteria for the design of VPS and SPS for HEVC extensions

## General constraints

The purpose of the VPS is to serve as an interface for systems to access information about the coded video sequence that is relevant for sub-bitstream extraction and capability exchange. In the HEVC base specification design, the VPS is mandatory to be present in a coded video sequence but not required to be parsed by decoders as it does not contain any information relevant for the decoding process that is not contained in the SPS. It is however anticipated that VPS parsing will be required in the decoding process of multi-layer HEVC extensions.

The following constraints to be considered for the VPS/SPS design are suggested:

- The contents of a VPS should change infrequently

A VPS may contain sequence-level parameters that are relevant for the decoding of one or multiple layers. Thus activation of a new VPS requires that the decoding process is initialized, i.e. an IDR picture is present in all layers. Activation of a new SPS for a specific layer requires that an IDR picture is present for that layer, however it may not require an IDR picture to be present for all layers. As of the coding efficiency loss associated with the use of IDR pictures, if a parameter that is associated with a specific layer may change while other layers are unaffected, that parameter should preferably be contained in the SPS rather than the VPS.

- The amount of data associated with higher layers in the VPS should be small

After sub-bitstream extraction, the VPS is included in all extracted sub-bitstreams (as long as the base layer is part of the sub-bitstream). Any VPS parameters associated with higher layers are useless for the decoding of extracted sub-bitstreams and will thus have a negative impact on the coding efficiency of the sub-bitstreams. This is particularly relevant for applications with frequent in-band transmission of parameter sets (broadcast/multicast).

Re-writing the VPS upon sub-bitstream extraction sounds like a solution to the problem, however changing the VPS may require presence of IDR pictures for all layers which is undesirable in terms of coding efficiency (see constraint 1 above). Furthermore, VPS rewriting is potentially complex and may not be applicable in some applications.

- The size of parameter sets in general, and the VPS in particular, should be small

When transmitted out-of band, the size of parameter sets should be small such as to avoid bitrate overhead and thus delay in e.g. session setup. When the VPS in particular is used for capability exchange, it should be small to avoid bitrate overhead and thus delay in the capability exchange. Thus it is desirable that the amount of VPS parameters that are not needed for the capability exchange is minimized.

## Design criteria for VPS and SPS

Based on the proposed basic design and the general constraints above, the following VPS design criteria are derived:

- The VPS should include parameters that are useful for bitstream extraction and/or capability exchange.

- Parameters that are not required for bitstream extraction and/or capability exchange should not be present in the VPS.

- Sequence-level parameters that may change in some layers while not affecting the general layer structure should not be present in the VPS. Such parameters can be present in the SPS instead.

Complementary, the following SPS design criteria are derived:

- Parameters present in the VPS should not be duplicated in the SPS (except for those parameters that are required for base layer decoding).

- Sequence-level parameters that are not present in the VPS should be included in the SPS. This includes specifically:

  - Parameters that are not required for bitstream extraction and/or capability exchange.

  - Parameters that may change in one or few layers while not affecting the general layer structure.

  - Coding tool parameters.

# Proposed design

Based on the design criteria above, the VPS and SPS designs given in the following sub-sections are proposed. Changes relative to the HEVC base specification are highlighted in yellow. Changes relative to “approach 1” of the HEVC extensions NAL unit and parameter set design “straw man”, JCTVC-J1007, are highlighted in green.

Section 4.1 shows the proposed VPS extension design. Compared to JCTVC-J1007, the following changes have been made:

- The syntax elements num_dimensions_minus1 and dimension_type have been moved up and are no longer part of the loop over the scalability layers, implying that all layers share the same scalability dimension types.

- A new syntax element dimension_id_length_minus1 has been added that specifies the bit length of a scalability dimension type identifier, i.e. the length in bits of the syntax element dimension_id[ i ][ j ].

- A new syntax element vps_layer_id has been added that specifies the value of layer_id for each scalability layer. This allows for a flexible allocation of layer_id values (the layers do not necessarily have to have consecutive values of layer_id). In particular, in certain applications, the values of layer_id can be assigned in such a way that the 6 bits of the layer_id are partitioned into one or more bit fields (e.g. 1 bit for depth_flag and 5 bits for view_id) so that the scalability dimensions can be identified based on layer_id in a simple way (similar as targeted by “approach 2” in JCTVC-J1007).

- A loop over additional operation points has been added. Each operation point is defined as a set of layer_id values, defining a set of target output views or a target video representation. For each defined operation point, profile and level are specified.

Section 4.2 shows the proposed SPS design. The same NAL unit type is used for SPS associated with base and enhancement layers. The base layer SPS design is not changed compared to the SPS design in the HEVC base specification. The value of layer_id distinguishes a base layer SPS from an enhancement layer SPS. Compared to the base layer SPS, the enhancement layer SPS does not include temporal scalability related parameters that are already included in the VPS.

## Video parameter set RBSP syntax and semantics

### Video parameter set RBSP

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| reserved_zero_2bits | u(2) |
| max_num_layers_minus1 //reserved_zero_6bits in the base spec | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_and_level( 1, vps_max_sub_layers_minus1 ) |  |
| next_essential_info_byte_offset //reserved_zero_12bits in the base spec | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| bit_equal_to_one | u(1) |
| vps_extension( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

max_num_layers_minus1 plus 1 specifies the maximum number of layers in the coded video sequences referring to the video parameter set.

next_essential_info_byte_offset specifies the byte offset of the next set of fixed-length coded information in the video parameter set NAL unit, starting from the beginning of the NAL unit.

NOTE 3 –Video parameter set information for non-base layer or view starts from a byte-aligned position of the video parameter set NAL unit, with fixed-length coded information that is essential for session negotiation and/or capability exchange. The byte offset specified by next_essential_info_byte_offset would then help to locate and access those essential information in the video parameter set NAL unit without the need of entropy decoding, which may not be equipped with some network entities that may desire to access only the information in the video parameter set that is essential for session negotiation and/or capability exchange.

bit_equal_to_one shall be equal to 1.

Semantics for other syntax elements are as specified in the HEVC base specification.

### Video parameter set extension

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| // layer specific information |  |
| num_dimensions_minus1 | u(4) |
| for( i = 0; i <= num_dimensions_minus1; i++ ) { |  |
| dimension_type[ i ] | u(4) |
| dimension_id_length_minus1[ i ] | u(3) |
| } |  |
| for( i = 1; i <= max_num_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| vps_layer_id[ i ] | u(6) |
| num_dimensions_minus1[ i ] | u(4) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) { |  |
| dimension_type[ i ][ j ] | u(4) |
| dimension_id[ i ][ j ] | u(8v) |
| } |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| num_additional_operation_points | ue(v) |
| for( i = 0; i < num_additional_operation_points; i++ ) { |  |
| op_point( i ) |  |
| profile_and_level( 1, vps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |

vps_extension_byte_alignment_reserved_one_bit shall be equal to 1.

num_dimensions_minus1 plus 1 specifies the number of dimension types. The value of num_dimensions_minus1 shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this Recommendation | International Standard. Other values for num_dimensions_minus1 are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore values of num_dimensions_minus1 that are not in the range of 0 to 3, inclusive.

dimension_type[ i ] specifies the i-th scalability dimension type, as specified in the following table:

| dimension_type[ i ] | scalability dimension type |
| --- | --- |
| 0 | view order idx |
| 1 | depth flag |
| 2 | dependency ID |
| 3 | quality ID |
| 4..15 | reserved |

The value of dimension_type[ i ] shall not be equal to the value of dimension_type[ j ] for any combination of i greater than or equal to 0 and j greater than i. The value of dimension_type[ i ] shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this Recommendation | International Standard. Other values for dimension_type[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore values of dimension_type[ i ] that are not in the range of 0 to 3, inclusive.

dimension_id_length_minus1[ i ] plus 1 specifies the number of bits used to specify the i-th scalability dimension type.

vps_layer_id[ i ] specifies the value of layer_id associated with the i-th layer.

dimension_id[ i ][ j ] specifies the identifier of the j-th scalability dimension type of the i-th layer. The length in bits of the syntax element dimension_id[ i ][ j ] is given by dimension_id_length_minus1[ j ] plus 1.

num_direct_ref_layers[ i ] specifies the number of layers the i-th layer directly depends on. [Ed.(YK): Add the exact meaning of a layer directly depending on another layer.]

ref_layer_id[ i ][ j ] identifies the j-th layer the i-th layer directly depends on.

num_additional_operation_points specifies the number of additional operation points specified.

## Sequence parameter set RBSP syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_bit | u(1) |
| profile_and_level( 1, sps_max_sub_layers_minus1 ) |  |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| if( layer_id == 0 ) |  |
| for( i = 0; i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| if( layer_id == 0 ) |  |
| sps_temporal_id_nesting_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Semantics are as specified in the HEVC base specification.

## Parameter set activation

- A picture parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more coded pictures. Each picture parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one picture parameter set RBSP is considered active for a layer at any given moment during the operation of the decoding process, and the activation of any particular picture parameter set RBSP for a layer results in the deactivation of the previously-active picture parameter set RBSP for that layer (if any).

- When a picture parameter set RBSP (with a particular value of pic_parameter_set_id) is not active for the layer having layer_id equal to LId and it is referred to by a coded slice NAL unit having layer_id equal to LId (using that value of pic_parameter_set_id), it is activated for the layer having layer_id equal to LId. This picture parameter set RBSP is called the active picture parameter set RBSP for the layer having layer_id equal to LId until it is deactivated by the activation of another picture parameter set RBSP for that layer. A picture parameter set RBSP, with that particular value of pic_parameter_set_id and layer_id less than or equal to LId, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId less than or equal to the TemporalId of the picture parameter set NAL unit, unless the picture parameter set is provided through external means.

- Any picture parameter set NAL unit containing the value of pic_parameter_set_id for the active picture parameter set RBSP for a coded picture shall have the same content as that of the active picture parameter set RBSP for the coded picture unless it follows the last VCL NAL unit of the coded picture and precedes the first VCL NAL unit of another coded picture.

- A sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more SEI NAL units containing a buffering period SEI message. Each sequence parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one sequence parameter set RBSP is considered active for a layer at any given moment during the operation of the decoding process, and the activation of any particular sequence parameter set RBSP for a layer results in the deactivation of the previously-active sequence parameter set RBSP for that layer (if any).

- When a sequence parameter set RBSP (with a particular value of seq_parameter_set_id) is not already active for the layer having layer_id equal to LId and it is referred to by activation of a picture parameter set RBSP for the layer having layer_id equal to LId (using that value of seq_parameter_set_id) or is referred to by an SEI NAL unit containing a buffering period SEI message (using that value of seq_parameter_set_id), it is activated for the layer having layer_id equal to LId. This sequence parameter set RBSP is called the active sequence parameter set RBSP for the layer having layer_id equal to LId until it is deactivated by the activation of another sequence parameter set RBSP for that layer. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id and layer_id less than or equal to LId, shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0, unless the sequence parameter set is provided through external means. An activated sequence parameter set RBSP shall remain active for the entire coded video sequence.

- NOTE 1 – Because an IDR or BLA access unit begins a new coded video sequence and an activated sequence parameter set RBSP must remain active for the entire coded video sequence, a sequence parameter set RBSP can only be activated by a buffering period SEI message when the buffering period SEI message is part of an IDR or BLA access unit.

- Any sequence parameter set NAL unit containing the value of seq_parameter_set_id for the active sequence parameter set RBSP for a layer of a coded video sequence shall have the same content as that of the active sequence parameter set RBSP for that layer of the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing a buffering period SEI message (when present) of another coded video sequence.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).