| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0278-r2 |
| --- | --- |

| Title: | Cross-check of JCTVC-K0188: On the derivation of chroma QPs |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Philippe Salmon, Pierre Andrivon 1, av. De Belle-Fontaine – CS17616 35576 Cesson-Sévigné Cedex - France | Tel: Email: | +33 2 99 27 32 42 philippe.bordes@technicolor.com philippe.salmon@technicolor.com pierre.andrivon@technicolor.com |  |
| Source: | Technicolor |  |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results on the two modifications proposals for the chroma QP derivation.

In the first proposal, the table used in the derivation of chroma QP values from luma QP is replaced by a generic equation which reproduces the table values almost identically. This change enables to remove one table from the specification.

In the second proposal, it is suggested to make this equation more generic by introducing two new control parameters that enable to accurately control the link between luma and chroma QPs. These parameters allows for both mastering at which QP (qp start) the ratio luma-chroma quantization steps change, and what is the inclination degree of the slope change (qp shift). The 3 tests show how one can control the chroma and Luma quantization independently using qpStart and qpShift parameters only.

This document is a cross-check of the tests 1, 2 and 3 of the second proposal. No impact on the encoding and decoding runtime is observed.

# Cross-checking

The software based on HM version 8.0 and the experimental results were provided on October 1st, 2012 by Canon. The software was compiled fine and the cross-checking was performed on 64bit linux environment. The simulation has been conducted under common test conditions [1] for Main profile (MP) conditions and high efficiency (HE10) conditions in the case of random access (RA) and low delay B (LB) coding parameters.

The anchors were generated using the HM8.0. In the software provided by the proponents two added macro were set as follows (values used for test 1):

#define QP_START                       29

#define QP_SHIFT                         1

The provided source code implement the syntax proposed in K0188 in case the signaling is done only at the PPS level.

# Results

## Proposal 2: generic equation (eq.2) with 2 control parameters

As in K0188, we have tested the three different settings for the two control parameters defined in proposal 2, by using the common test conditions. The results (see also attached xls files) are depicted in the tables below. No impact on the encoding and decoding runtime is observed.

Test 1 : cbcr_qp_start = 29 – cbcr_qp_shift = 1

|  | Random Access Main |  |  |  | Random Access HE10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U |  | V |  | YUV |
| Class A | 0.4% | -6.6% | -6.4% | -0.5% | 0.3% | -6.3% |  |  | -6.5% | -0.6% |
| Class B | 0.6% | -4.6% | -3.9% | -0.4% | 0.6% | -4.6% |  | -3.8% |  | -0.4% |
| Class C | 0.6% | -3.9% | -3.9% | -0.4% | 0.6% | -3.8% |  | -3.9% |  | -0.4% |
| Class D | 0.5% | -3.8% | -3.9% | -0.4% | 0.4% | -4.0% |  | -4.0% |  | -0.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.5% | -4.7% | -4.5% | -0.4% | 0.5% | -4.7% |  | -4.5% |  | -0.5% |
|  | 0.5% | -4.5% | -4.3% |  | 0.5% | -4.4% |  | -4.4% |  |  |
| Class F | 0.5% | -2.0% | -2.0% | -0.1% | 0.4% | -1.9% |  | -2.0% |  | -0.1% |
|  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |  |  |  |
|  | Y | U | V | YUV | Y | U |  | V |  | YUV |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.6% | -6.9% | -6.4% | -0.8% | 0.6% | -6.7% |  | -5.8% |  | -0.8% |
| Class C | 0.7% | -5.5% | -5.4% | -0.6% | 0.6% | -5.5% |  | -5.4% |  | -0.7% |
| Class D | 0.4% | -5.5% | -6.0% | -0.7% | 0.5% | -6.4% |  | -6.2% |  | -0.8% |
| Class E | 0.4% | -5.4% | -5.5% | -0.7% | 0.4% | -4.4% |  | -4.4% |  | -0.6% |
| Overall | 0.5% | -5.9% | -5.9% | -0.7% | 0.5% | -5.9% |  | -5.5% |  | -0.7% |
|  | 0.5% | -4.5% | -4.4% |  | 0.5% | -4.6% |  | -4.0% |  |  |
| Class F | 0.7% | -3.6% | -4.1% | -0.3% | 0.5% | -3.4% |  | -4.8% |  | -0.5% |

Test 2 : cbcr_qp_start = 31 – cbcr_qp_shift = 1

|  | Random Access Main |  |  |  | Random Access HE10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U |  | V |  | YUV |
| Class A | -0.1% | 3.9% | 3.9% | 0.4% | -0.1% | 3.9% |  |  | 3.6% | 0.4% |
| Class B | -0.2% | 4.3% | 4.3% | 0.6% | -0.2% | 4.3% |  | 4.3% |  | 0.6% |
| Class C | -0.2% | 3.2% | 3.2% | 0.4% | -0.2% | 3.3% |  | 3.1% |  | 0.4% |
| Class D | -0.2% | 3.0% | 3.0% | 0.4% | -0.2% | 3.0% |  | 2.8% |  | 0.3% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.2% | 3.6% | 3.6% | 0.4% | -0.2% | 3.7% |  | 3.5% |  | 0.4% |
|  | -0.2% | 2.9% | 2.8% |  | -0.2% | 3.0% |  | 2.6% |  |  |
| Class F | -0.3% | 3.0% | 3.2% | 0.3% | -0.4% | 3.1% |  | 2.8% |  | 0.3% |
|  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |  |  |  |
|  | Y | U | V | YUV | Y | U |  | V |  | YUV |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.1% | 1.7% | 1.2% | 0.2% | -0.1% | 1.4% |  | 1.7% |  | 0.2% |
| Class C | -0.1% | 1.3% | 1.3% | 0.1% | -0.1% | 1.2% |  | 1.4% |  | 0.1% |
| Class D | -0.1% | 1.3% | 0.9% | 0.1% | -0.1% | 0.9% |  | 0.6% |  | 0.1% |
| Class E | -0.1% | 3.0% | 4.6% | 0.6% | 0.0% | 3.1% |  | 2.7% |  | 0.5% |
| Overall | -0.1% | 1.7% | 1.8% | 0.3% | -0.1% | 1.6% |  | 1.6% |  | 0.1% |
|  | -0.1% | 1.5% | 1.5% |  | -0.1% | 1.3% |  | 1.3% |  |  |
| Class F | 0.0% | 1.7% | 1.3% | 0.3% | -0.3% | 1.7% |  | -0.4% |  | -0.1% |

Test 3 : cbcr_qp_start = 22 – cbcr_qp_shift = 2

|  | Random Access Main |  |  |  | Random Access HE10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U |  | V | YUV |
| Class A | 0.9% | -17.0% | -16.9% | -1.3% | 0.9% | -16.7% |  | -17.0% | -1.4% |
| Class B | 1.4% | -15.1% | -15.1% | -1.7% | 1.4% | -15.3% |  | -15.2% | -1.7% |
| Class C | 1.5% | -12.5% | -12.3% | -1.1% | 1.5% | -12.5% |  | -12.6% | -1.2% |
| Class D | 1.2% | -12.9% | -12.9% | -1.3% | 1.2% | -13.2% |  | -13.0% | -1.4% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 1.2% | -14.4% | -14.3% | -1.4% | 1.2% | -14.5% |  | -14.5% | -1.4% |
|  | 1.2% | -14.1% | -13.9% |  | 1.2% | -14.1% |  | -14.1% |  |
| Class F | 1.3% | -8.5% | -8.6% | -0.6% | 1.4% | -8.3% |  | -8.7% | -0.5% |
|  | Low delay B Main |  |  |  | Low delay B HE10 |  |  |  |  |
|  | Y | U | V | YUV | Y | U |  | V | YUV |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 1.2% | -16.8% | -17.0% | -1.9% | 1.2% | -16.9% |  | -16.6% | -1.9% |
| Class C | 1.4% | -13.6% | -13.8% | -1.3% | 1.3% | -14.2% |  | -14.2% | -1.5% |
| Class D | 1.1% | -13.6% | -14.3% | -1.4% | 1.2% | -15.4% |  | -15.4% | -1.6% |
| Class E | 0.6% | -14.9% | -14.7% | -2.0% | 0.6% | -14.6% |  | -15.2% | -2.1% |
| Overall | 1.1% | -14.9% | -15.1% | -1.5% | 1.1% | -15.4% |  | -15.5% | -1.7% |
|  | 1.1% | -14.5% | -14.6% |  | 1.1% | -15.1% |  | -15.0% |  |
| Class F | 1.5% | -9.9% | -10.2% | -0.7% | 1.4% | -9.5% |  | -11.4% | -0.9% |

# Conclusion

We cross-checked the modifications for the chroma QP derivation proposed in JCTVC-K0188. We confirm the results presented in JCTVC-J0188 for proposal 2, tests 1, 2 and 3.

# References

- F. Bossen, “Common HM test conditions and software reference configurations”, Document no JCTVC-I1100. May 2012.