| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0279_r1 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC Draft 8 with AVC High Profile and Performance of HM8.0 with Different Delay Characteristics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Gary J. Sullivan Jizheng Xu |  |  |
| Source: | University of Science and Technology of China Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution is a further study of the relative objective (i.e. PSNR-based) compression performance of HEVC Main Profile and AVC High Profile. It builds upon the prior work reported in JCTVC-G399 JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, updating the results by using the latest available reference software (JM-18.4 and HM-8.0). Experimental results (without class F) reportedly show that 1) for the Main Profile all-intra configuration, HM-8.0 can save about 21.9% in bit rate; 2) for the Main Profile random-access configuration, HM-8.0 can save about 34.3% in bit rate; and 3) for the Main Profile low-delay configuration, HM-8.0 can save about 36.7% in bit rate when compared with JM-18.4.

HM-8.0 with different delay characteristics (lowdelay_P, lowdelay_B, randomaccess and hierarchical-B with only one Intra picture) are also tested. Without taking class F into consideration, compared with lowdelay_P, lowdelay_B brings 8.4%’s gain; compared with lowdelay_B, randomaccess brings 13.3%’s gain; compared with randomacccess, hierarchical-B with only one Intra picture brings 8.8%’s gain.

Besides, QP refinement proposed in JCTVC-J0242 was also tested on top of HM-8.0 and JM-18.4. It brings 2.3% (RA) and 1.6% (LD) bit-saving to HM-8.0; and brings 3.0% (RA) and 2.3% (LD) bit-saving to JM-18.4. Thus, when enabling QP refinement, the gap between JM and HM will be further reduced to 33.5% (RA) and 35.8% (LD).

# Experimental setting

The JM software was configured with the same coding structure and referencing structure as HM-8.0. Both the JM anchor and HM test encoding used 8-bit encoding and decoding. The detailed JM configuration is explained below.

The average YUV PSNR (shown in the right-most column of the tables below) was calculated as

PSNRavg=6×PSNRY+PSNRU+PSNRV8

It is noted that these results are based on PSNR measurements, which are an imperfect substitute for subjective quality assessment, and that the subjective performance of HEVC may actually be generally somewhat better than its objective (PSNR) performance may indicate.

## All Intra case

Nothing special was configured.

## Random access case

- Using 4 reference pictures

- Using the same referencing structure as HM-8.0 (by MMCO).

- Using B pictures with two identical lists at level 0.

## Low delay case

- Using 4 reference pictures

- Using a 1+3 referencing structure (by MMCO)

- Using B pictures

# Experimental results

## HM8 vs. JM18.4

The summary results of performance comparison between AVC High Profile and HEVC Main Profile are shown in the Tables 1, 2, and 3. Details can be found in the attached spreadsheets. We do not provide the encoding time and decoding time ratio results, since the codebases are different and with different degrees of optimization; and neither of them is really speed-optimized.

The HEVC Main Profile is specified in [6], and we use the configuration files provided in [7] to generate the HEVC Main Profile bitstream.

Table 1 – Results for All Intra (AVC High Profile vs. HEVC Draft 8 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −23.7% | −21.3% | −20.1% | −23.3% |
| Class B | −22.7% | −22.2% | −21.8% | −22.6% |
| Class C | −19.7% | −20.8% | −21.0% | −20.0% |
| Class D | −16.4% | −16.8% | −17.4% | −16.7% |
| Class E | −28.9% | −27.2% | −27.0% | −28.6% |
| Class F | −28.4% | −24.9% | −25.7% | −28.2% |
| Average | −23.0% | −22.0% | −22.0% | −23.0% |
| Average without F | −22.0% | −21.4% | −21.2% | −21.9% |

Table 2 – Results for Random Access (AVC High Profile vs. HEVC Draft 8 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −36.7% | −37.6% | −39.3% | −36.9% |
| Class B | −39.8% | −37.6% | −37.2% | −39.5% |
| Class C | −30.3% | −32.1% | −32.2% | −30.7% |
| Class D | −27.8% | −31.3% | −33.1% | −28.7% |
| Class E |  |  |  |  |
| Class F | −30.8% | −30.5% | −31.9% | −31.0% |
| Average | −33.4% | −34.0% | −34.9% | −33.7% |
| Average without F | −34.0% | −34.8% | −35.5% | −34.3% |

Table 3 – Results for Low Delay (AVC High Profile vs. HEVC Draft 8 Main Profile)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | −42.2% | −34.5% | −35.5% | −41.2% |
| Class C | −32.6% | −32.2% | −33.0% | −32.7% |
| Class D | −29.7% | −31.4% | −32.8% | −30.2% |
| Class E | −44.0% | −39.3% | −39.7% | −43.2% |
| Class F | −33.7% | −35.1% | −37.4% | −34.6% |
| Average | −36.4% | −34.3% | −35.5% | −36.3% |
| Average without F | −37.0% | −34.1% | −35.0% | −36.7% |

## HM with Different Delay Characteristics

We also compared the performance of HM-8.0 Main Profile with different delay characteristics and settings.

Table 4 –Lowdelay_P vs. Lowdelay_B

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | −11.2% | −10.8% | −9.8% | −11.0% |
| Class C | −6.9% | −6.1% | −6.5% | −6.8% |
| Class D | −6.8% | −6.3% | −6.1% | −6.8% |
| Class E | −8.4% | −7.4% | −7.9% | −8.2% |
| Class F | −3.6% | −3.5% | −3.3% | −3.6% |
| Average | −7.5% | −7.0% | −6.8% | −7.4% |
| Average without F | −8.5% | −7.8% | −7.7% | −8.4% |

Table 5 –Lowdelay_B vs. Randomaccess

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | −6.9% | −27.0% | −28.0% | −10.7% |
| Class C | −10.9% | −25.0% | −23.8% | −13.4% |
| Class D | −14.0% | −27.5% | −28.2% | −16.5% |
| Class E |  |  |  |  |
| Class F | 77.3% | 63.9% | 64.7% | 74.8% |
| Average | 10.3% | −5.3% | −5.3% | 7.4% |
| Average without F | −10.3% | −26.5% | −26.8% | −13.3% |

Table 6 Randomaccess vs. Hierarchical-B with only one Intra picture

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −9.2% | −5.8% | −7.1% | −8.9% |
| Class B | −12.1% | −8.6% | −8.1% | −11.5% |
| Class C | −7.0% | −3.2% | −4.2% | −6.4% |
| Class D | −8.1% | −6.3% | −6.5% | −7.9% |
| Class E |  |  |  |  |
| Class F | −30.5% | −28.4% | −29.3% | −30.2% |
| Average | −13.3% | −10.4% | −0.9% | −12.9% |
| Average without F | −9.3% | −6.1% | −6.6% | −8.8% |

Table 7. Lowdelay_B vs. Hierarchical-B with only one Intra picture

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | −18.6% | −33.7% | −34.1% | −21.3% |
| Class C | −17.2% | −27.6% | −27.4% | −19.1% |
| Class D | −20.8% | −32.5% | −33.0% | −22.9% |
| Class E |  |  |  |  |
| Class F | −10.5% | −17.9% | −16.9% | −11.9% |
| Average | −16.9% | −28.3% | −28.2% | −18.9% |
| Average without F | −18.8% | −31.4% | −31.7% | −21.1% |

## QP Refinement (for both HM and JM)

In JCTVC-J0242, we proposed to refine the QP for different temporal layers according to the used Lagrange multiplier. On HM-8.0, such a method brings some gains, shown in Table 8 and 9.

Table 8. HM-8.0 vs. HM-8.0 with QP refinement (Randomaccess)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −1.7% | −6.0% | −7.1% | −2.2% |
| Class B | −1.5% | −4.8% | −5.4% | −2.2% |
| Class C | −2.1% | −4.8% | −4.6% | −2.6% |
| Class D | −1.8% | −4.9% | −4.9% | −2.3% |
| Class E |  |  |  |  |
| Class F | −2.0% | −3.7% | −3.5% | −2.3% |
| Average | −1.8% | −4.8% | −5.1% | −2.3% |
| Average without F | −1.8% | −5.1% | −5.5% | −2.3% |

Table 9. HM-8.0 vs. HM-8.0 with QP refinement (Lowdelay_B)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | −1.1% | −2.1% | −2.6% | −1.3% |
| Class C | −1.3% | −1.9% | −2.0% | −1.4% |
| Class D | −1.3% | −2.0% | −2.5% | −1.4% |
| Class E | −2.2% | −5.5% | −3.6% | −2.6% |
| Class F | −1.4% | −1.8% | −2.5% | −1.5% |
| Average | −1.4% | −2.5% | −2.6% | −1.6% |
| Average without F | −1.4% | −2.7% | −2.6% | −1.6% |

Such a method can also be applied to JM-18.4, the results are shown in Table 10 and 11.

Table 10. JM-18.4 vs. JM-18.4 with QP refinement (Randomaccess)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −2.2% | −4.3% | −6.8% | −2.6% |
| Class B | −1.5% | −5.6% | −8.0% | −2.3% |
| Class C | −3.6% | −6.1% | −6.1% | −4.1% |
| Class D | −2.6% | −6.3% | −6.5% | −3.2% |
| Class E |  |  |  |  |
| Class F | −3.1% | −6.0% | −6.0% | −3.7% |
| Average | −2.5% | −5.7% | −6.7% | −3.1% |
| Average without F | −2.4% | −5.6% | −6.9% | −3.0% |

Table 11. JM18.4 vs. JM18.4 with QP refinement (Lowdelay)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | −2.1% | −2.6% | −4.2% | −2.3% |
| Class C | −2.6% | −3.2% | −3.2% | −2.7% |
| Class D | −2.4% | −4.6% | −4.5% | −2.7% |
| Class E | −4.0% | −10.8% | −12.3% | −5.5% |
| Class F | −2.9% | −3.5% | −4.2% | −3.1% |
| Average | −2.7% | −4.5% | −5.3% | −3.1% |
| Average without F | −2.1% | −2.6% | −4.2% | −2.3% |

It can be observed that QP refinement brings more gains to JM than HM. Thus, with QP refinement, the gap between JM and HM will be further reduced, shown in Table 12 and 13.

Table 12. JM-18.4 with QP refinement vs. HM-8.0 with QP refinement (Randomaccess)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A | −36.0% | −38.9% | −40.0% | −36.4% |
| Class B | −39.5% | −36.9% | −35.2% | −39.1% |
| Class C | −29.0% | −30.6% | −30.7% | −29.3% |
| Class D | −27.0% | −30.2% | −31.7% | −27.9% |
| Class E |  |  |  |  |
| Class F | −30.1% | −27.8% | −29.0% | −29.9% |
| Average | −32.7% | −33.1% | −33.4% | −32.8% |
| Average without F | −33.3% | −34.3% | −34.5% | −33.5% |

Table 13. JM-18.4 with QP refinement vs. HM-8.0 with QP refinement (Lowdelay)

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Class A |  |  |  |  |
| Class B | −41.6% | −34.4% | −34.8% | −40.7% |
| Class C | −31.7% | −31.4% | −32.3% | −31.8% |
| Class D | −28.9% | −29.9% | −31.8% | −29.3% |
| Class E | −42.8% | −35.2% | −33.6% | −41.4% |
| Class F | −32.6% | −33.8% | −36.2% | −33.5% |
| Average | −35.5% | −32.9% | −33.8% | −35.3% |
| Average without F | −36.2% | −32.7% | −33.2% | −35.8% |

# References

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” Geneva, CH, Nov. 2011.

- JCTVC-H0360, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” San José, CA, USA, Feb. 2012.

- JCTVC-I0409, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-J0236, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” Stockholm, SE, 11–20 July 2012.

- JCTVC-J0242, “Encoding with fixed Lagrange multipliers,” Stockholm, SE, 11–20 July 2012.

- JCTVC-J1003, “High efficiency video coding (HEVC) text specification draft 8,” Stockholm, SE, 11–20 July 2012.

- JCTVC-J1100, “HM 8 Common Test Conditions and Software Reference Configurations,” Stockholm, SE, 11–20 July 2012.