| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0281 |
| --- | --- |

| Title: | CE1: Cross-check of Test 8, Test 9, Test 4, Test 5 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Do-Kyoung Kwon | Tel: Email: Tel: Email: | +1-214-480-1861 madhukar@ti.com +1-214-480-1479 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for deblocking filter CE1 Test 8, Test 9, Test 4 and Test 5. It is reported that there is a match to the BD-Rate results generated by the proponents. Informal visual testing was also done and comments on those are also provided in this contribution.

# Tools evaluated

The following tool combinations from CE1 [1] were evaluated.

Test 8: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Bs modification max TU (b.3.3)

Test 9: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Bs modification max TU (b.3.3) + beta table modification (a.1)

Test 5: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Intra tc_offset = 3 (b.2.2) + beta table modification (a.1)

Test 4: Strong filter decisions (b.1.1) + Strong filter (b.1.2) + Intra smoothing restricted (b.1) + Intra tc_offset = 3 (b.2.2)

Note that Test 9 is Test 8 + beta table modification. Also Test 5 is Test 4 + beta table modification.

# BD-Rate results on common test conditions

Table below shows the BD-Rate results for Test 8 v/s HM-8.0. Numbers more that 0.5% are highlighted in table below.

|  | Class A-E | Class F |
| --- | --- | --- |
| AI-Main | 0.1% | -0.1% |
| RA-Main | 0.0% | 0.0% |
| LB-Main | 0.1% | 0.0% |
| LP-Main | -0.1% | 0.0% |
| AI-HE10 | 0.3% | 0.0% |
| RA-HE10 | 0.1% | 0.1% |
| LB-HE10 | 0.6% | 0.2% |
| LP-HE10 | 0.0% | 0.0% |

# Visual quality on Type 2 sequences

The objective of this testing is to verify that there is NO quality degradation on normal video sequences such as those in CTC.

Test 4 and Test 8 video quality at QP=32 and QP=37 was compared against HM8 on all Type 2 sequences in [1]. No significant changes in quality were observed.

# Visual quality on Type 1 sequences

The objective of this testing is to verify that there is quality improvement on video sequences such as ducks takeoff, riverbed, etc.

“No visible improvement” in the tables in this section mean “hard to see difference clearly” or “negligible improvement” or “one of viewers cannot see difference”. Also the “No” in “Better than anchor” columns means “hard to see difference clearly” or “negligible improvement” or “one of viewers cannot see difference”.

## Test 8 and Test 9

| Test 8 v/s HM-8.0, Max TU size = 32x32 |  |  |  |
| --- | --- | --- | --- |
|  | QP | Comments | Better than anchor (HM-8.0) |
| Ducks Takeoff, 25 fps, LB | 37 | Anchor contains visible block edge artifacts. For test, block edge artifacts are less visible in the beginning of the sequence but quite a few remain visible at end of sequence. | Some improvement |
| Ducks Takeoff, 25 fps, RA | 37 | Anchor contains visible block edge artifacts. For test, most of the block edge artifacts get smoothed | Yes |
| RedKayak, LB | 37 | Both anchor and test have significant blockiness. | No |
| RedKayak, RA | 37 | Both anchor and test have significant blockiness. | No |
| Riverbed | 37 | Anchor contains visible blocking artifacts. For test, a lot of the block edge artifacts get smoothed. However blockiness still exists at left part of video sequence. | Yes |
| WestWindEasy | 37 | Block edges are smoothed in test in first few frames. But blocking still remains very noticable in the center of the video through out the sequence. | No |
| ChinaSpeed | 37 | Blockiness in dark sky region in top center of the sequence for both anchor and test. (NB: In HM-7.0 version of the software, test showed better quality than anchor for Chinaspeed) | No |
| Test 8 v/s HM-8.0, Max TU size = 16x16 |  |  |  |
|  | QP | Comments | Better than anchor (HM-8.0) |
| Ducks Takeoff, 25 fps, LB | 37 | Anchor contains visible block edge artifacts. For test, block edge artifacts are less visible in the beginning of the sequence but quite a few remain visible at end of sequence. | Some improvement |
| Ducks Takeoff, 25 fps, RA | 37 | Anchor contains visible block edge artifacts. For test, most of the block edge artifacts get smoothed | Yes |
| RedKayak, LB | 37 | Both anchor and test have significant blockiness. | No |
| RedKayak, RA | 37 | Both anchor and test have significant blockiness. | No |
| Riverbed | 37 | Anchor contains visible blocking artifacts. For test, a lot of the block edge artifacts get smoothed. However blockiness still exists at left part of video sequence. | Yes |
| WestWindEasy | 37 | Block edges are smoothed a bit in test in first few frames. But blocking still remains very noticable in the center of the video through out the sequence. | No |
| ChinaSpeed | 37 | Blockiness in dark sky region in top center of the sequence for both anchor and test. | No |
| Test 9 (max offset) v/s Test 8, Max TU size = 32x32 |  |  |  |
|  | QP | Comments | Better than anchor (Test 8) |
| Ducks Takeoff, 25 fps, LB | 37 | Block edge artifacts that are visible at the end of sequence in Test 8 get smoothed in Test 9. | Yes |
| Ducks Takeoff, 25 fps, RA | 37 | A few of the block edge artifacts that remain at the end of the sequence in Test 8 get smoothed in Test 9. | Slight improvement |
| RedKayak, LB | 37 | No visible improvement | No |
| RedKayak, RA | 37 | No visible improvement | No |
| Riverbed | 37 | Test 9 is slightly better in the middle part of the frame | Slight improvement |
| WestWindEasy | 37 | No visible improvement | No |
| ChinaSpeed | 37 | Test 9 has slightly more flicker in center of picture | No |
| Test 9 (max offset) v/s HM-8.0 (max offset), Max TU size = 32x32 |  |  |  |
|  | QP | Comments | Better than anchor (HM8, maxoffset) |
| Ducks Takeoff, 25 fps, LB | 37 | No visible improvement | No |
| Ducks Takeoff, 25 fps, RA | 37 | No visible improvement | No |
| RedKayak, LB | 37 | No visible improvement | No |
| RedKayak, RA | 37 | No visible improvement | No |
| Riverbed | 37 | Test 9 is slightly better in the middle part of the frame | Slight improvement |
| WestWindEasy | 37 | No visible improvement | No |
| ChinaSpeed | 37 | Test 9 has slightly more flicker in center of picture | No |

## Test 4 and Test 5

| Test 4 v/s HM-8.0, Max TU size = 32x32 |  |  |  |
| --- | --- | --- | --- |
|  | QP | Comments | Better than anchor (HM-8.0) |
| Ducks Takeoff, 25 fps, LB | 37 | Anchor contains visible block edge artifacts. For test, block edge artifacts are less visible in the beginning of the sequence but quite a few remain visible at end of sequence. | Some improvement |
| Ducks Takeoff, 25 fps, RA | 37 | Anchor contains visible block edge artifacts. For test, most of the block edge artifacts get smoothed | Yes |
| RedKayak, LB | 37 | Both anchor and test have significant blockiness. | No |
| RedKayak, RA | 37 | Both anchor and test have significant blockiness. | No |
| Riverbed | 37 | Anchor contains visible blocking artifacts. For test, a lot of the block edge artifacts get smoothed. However blockiness still exists at left part of video sequence. | Yes |
| WestWindEasy | 37 | Block edges are smoothed in test in first few frames. But blocking still remains very noticable in the center of the video through out the sequence. | No |
| ChinaSpeed | 37 | Blockiness in dark sky region in top center of the sequence for both anchor and test. | No |
| Test 5 (max offset) v/s Test 4, Max TU size = 32x32 |  |  |  |
|  | QP | Comments | Better than anchor (Test 4) |
| Ducks Takeoff, 25 fps, LB | 37 | Block edge artifacts that are visible at the end of sequence in Test 4 get smoothed in Test 5. | Yes |
| Ducks Takeoff, 25 fps, RA | 37 | A few of the block edge artifacts that remain at the end of the sequence in Test 4 get smoothed in Test 5. | Slight improvement |
| RedKayak, LB | 37 | No visible improvement | No |
| RedKayak, RA | 37 | No visible improvement | No |
| Riverbed | 37 | Test 5 is slightly better in the middle part of the frame | Slight improvement |
| WestWindEasy | 37 | No visible improvement | No |
| ChinaSpeed | 37 | Test 5 has slightly more flicker in center of picture | No |
| Test 5 (max offset) v/s HM-8.0 (max offset), Max TU size = 32x32 |  |  |  |
|  | QP | Comments | Better than anchor (HM8, maxoffset) |
| Ducks Takeoff, 25 fps, LB | 37 | No visible improvement | No |
| Ducks Takeoff, 25 fps, RA | 37 | No visible improvement | No |
| RedKayak, LB | 37 | No visible improvement | No |
| RedKayak, RA | 37 | No visible improvement | No |
| Riverbed | 37 | Test 5 is slightly better in the middle part of the frame | Slight improvement |
| WestWindEasy | 37 | No visible improvement | No |
| ChinaSpeed | 37 | Test 5 has slightly more flicker in center of picture | No |

# Conclusions

Based on the visual testing done, Test 8 and Test 4 provide similar quality. The quality improvements are mainly in ducks take-off and riverbed sequences.

Based on the visual testing done, Test 9 and Test 5 show slight improvement in video quality over Test 8 and Test 4 respective in ducks take-off and riverbed sequences. Chinaspeed shows slightly higher flicker in sky region. Test 5 and Test 9 also show slight improvement in video quality over HM-8.0 (max-offset) in riverbed.

It is recommended that informal visual testing be done at meeting using more observers.

# References

[1]	A. Norkin, T. Suzuki, M. Budagavi, G. Van der Auwera (CE coordinators) “Description of Core Experiment 1: Deblocking filter”, Joint Collaborative Team on Video Coding, JCTVC-J1101, July, 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).