| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0283 |
| --- | --- |

| Title: | Video Parameter Set extension design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Skupin, V. George, T. Schierl Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 314 23881 robert.skupin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This proposal presents a VPS extension design that features signalling the partitioning of the nuh_reserved_zero_6bits/layer_id in the NAL unit header in a scalable or multiview video coding setup. Identified extensibility shortcomings of the straw man design proposed in JCTVC-J1007 are corrected.

# Introduction

During the last 10th JCT-VC meeting in Stockholm, a BoG on high level syntax for extension planning met to discuss the VPS related contributions, especially the ones that targeted extending the VPS syntax to support scalability signalling. The BoG recommendations [1] have been formed into an output document [2] that suggests two possible straw man design approaches for a VPS extension: one that either signals a mapping of layer_id values in the NAL unit header to scalability identifiers and another one that uses partitioning of the layer_id field into dedicated scalability identifiers. This proposal belongs to the second approach category, i.e. the signalling of a partitioning of the reserved_zero_6bits in the layer_id of the NAL unit header (anticipated to be used as a layer identifier in future extensions) in order to signal individual scalability identifiers.

During the BoG discussions, it was noted that the extensibility of the exemplary design for the partitioning approach may be a limiting factor. This is due to the mapping table used for defining combinations of scalability dimensions. As a solution to this shortcoming of the straw man design, this document proposes a modified signalling of scalability combinations. The signalling does not limit the amount of combinations but the amount of scalability types while combinations of scalability types are explicitly signalled in the VPS extension data.

# Proposed syntax

The VPS syntax has to be extended to include the vps_extension() syntax structure.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_zero_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| profile_and_level( 1, vps_max_sub_layers_minus1 ) |  |
| vps_reserved_zero_12bits | u(12) |
| for( i = 0; i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| if( i > 0 ) |  |
| op_point( i ) |  |
| hrd_parameters( i = = 0, vps_max_sub_layers_minus1 ) |  |
| } |  |
| reserved_one_bit | u(1) |
| vps_extension( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The vps_extension() systax structure is extended to support the signalling of any combination of up to 16 available scalability dimensions.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| num_scalability_dimensions | u(4) |
| for( i = 0; i < num_scalability_dimensions; i++ ) { |  |
| scalability_type[ i ] | u(4) |
| dimension_length[ i ] | u(3) |
| } |  |
| for( i = 0; i <= max_num_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

vps_extension_byte_alignment_reserved_one_bit shall be equal to 1.

num_scalability_dimensions specifies the number of scalability types and dimension bit lengths to be signalled through nuh_reserved_zero_6bits in the NAL unit header.

scalability_type[ i ] specifies the scalability dimension type of the i-th scalability dimension used in the coded video sequence according to the following table.

Table 1: Scalability type

| scalability_type[ i ] value | Semantic meaning |
| --- | --- |
| 0 | Multiview scalability |
| 1 | Depth flag |
| 2 | Spatial scalability |
| 3 | Quality scalability |
| 4..15 | reserved |

dimension_length[ i ] indicates the length in bits of nuh_reserved_zero_6bits that is used to signal the i-th scalability dimension according to Table 1 beginning at the MSB for i = 0 or the end of the (i-1)-th signalled scalability dimension for i > 0.

vps_layer_id[ i ] identifies the value of nuh_reserved_zero_6bits of the layer for with coding dependencies are signalled with num_direct_ref_layers[ i ] and ref_layer_id[ i ].

num_direct_ref_layers[ i ] specifies the number of layers the layer with nuh_reserved_zero_6bits equal to vps_layer_id[ i ] directly depends on to be decoded.

ref_layer_id[ i ][ j ] identifies the j-th layer the i-th layer directly depends on to be decoded.

- NOTE – In all the above semantics it is assumed that future extensions of HEVC will use the nuh_reserved_zero_6bits as layer_id to identify layers in a scalable setup.

# References

- Y.-K. Wang, "Report of BoG on high-level syntax for extension planning", JCTVC-J0574, July 2012.

- JCT-VC, "Solutions considered for NAL unit header and video parameter set for HEVC extensions", JCTVC-J1007, July 2012.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).