| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0284 |
| --- | --- |

| Title: | Cross-check of JCTVC-K0168 on Short Slice Header |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Félix Henry Gordon Clare Orange Labs 4, rue Clos Courtel 4 range Labs 35510 Cesson Sevigne, France4 | Email: | felix.henry@orange.com gordon.clare@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution reports cross-check results of JCTVC-K0168 on Short Slice Header. The software provided by the proponents matches the technology description in JCTVC-K0168. The experiment results generated by the cross-checker match those provided by the proponents for the sequences tested (class C & D only, MAIN profiles), although there were decoder mismatches as discussed in the section “experimental results”.

# Introduction

JCTVC-K0168 [2] proposes a modified slice header called Short Slice Header. A flag indicates if the Slice Header is short or not. If it is short, some syntax elements are not read in the slice header, they are inferred from the previous regular slice, and some other parameters have individual flag specifying whether they should be read or inferred. The first slice of a coded picture must be a regular slice. There are constraints set on the syntax elements of regular slice headers when a picture contains both multiple regular slices headers and short slices header.

# Experimental results

There are two experiments in JCTVC-K0168 contribution.

Some decoder mismatches and crashes were discovered during the cross-check in both experiments, occurring for profiles that were not “all intra. ”. This was apparently due to the fact that the slice header syntax element “cabac_init_flag” was not being transmitted and copied correctly, resulting in the decoder decoding slices with a badly initialized value of that element.  The correction of this problem should have a negligible performance impact, although this has not been verified.

Timing information is not available, as the cross check was performed in a heterogeneous compute environment.

## First experiment

The anchor uses HM8 with common test conditions and reference configurations specified in [1], with 1500 byte slice mode activated. The measure is the software of the proponent with the same test conditions, and where the first slice in a picture is a regular one and the next slices have short slice headers. The results are summarized in table 1 and match the ones published by the proponent, although only class C&D MAIN profiles have been cross-checked.

Table 1

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0,1% | -0,1% | -0,1% |
| Class D | -0,1% | -0,1% | -0,1% |
| Class E | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0,2% | -0,2% | -0,2% |
| Class D | -0,1% | -0,1% | -0,1% |
| Class E |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0,1% | -0,2% | -0,2% |
| Class D | -0,1% | -0,1% | -0,1% |
| Class E | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

## Second experiment

The anchor uses HM8 with common test conditions and reference configurations specified in [1], with class D-size tiles and one slice per tile. The measure is the software of the proponent with the same test conditions, and where the first slice in a picture is a regular one and the next slices have short slice headers. The results are summarized in table 2 and match the ones published by the proponent, although only class C&D MAIN profiles have been cross-checked.

Table 2

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% |
| Class E | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class A | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0,3% | -0,3% | -0,3% |
| Class D | 0,0% | 0,0% | 0,0% |
| Class E |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0,3% | -0,2% | -0,2% |
| Class D | 0,0% | 0,0% | 0,0% |
| Class E | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-J1100, Stockholm, Sweden, July 2012.

[2] Hendry, B. Jeon (LG), “Short Slice Header”, JCT-VC Document, JCTVC-K0168, Shanghai, CN, Oct. 2012.