| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Shanghai, CN, 10-19 October, 2012 | Document: JCTVC-K0285 WG11 Doc: m26633 |
| --- | --- |

| Title: | In Support of a Still Image Profile of HEVC v1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Dai, Madhu Krihnan, Pankaj Topiwala 5840 Banneker Rd., #230 Columbia, MD 21044 USA | Tel: Email: | +1-410-730-6922 pankajtva@gmail.com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

The emerging HEVC standard currently has one profile planned: Main. Additional profiles may be added in the various extensions that are currently being pursued. But one additional profile appears within reach right away – a still image profile. This document presents evidence in support of such a profile. It is suggested to consider creating a still image profile, but one that is directly compatible with Main profile.

# A Still Image Profile of HEVC v1

HEVC version 1 currently has one planned profile: Main. It also has various configuration presets in the reference software associated with this project (called HM; currently at version HM 8.0). Among these preset configurations, we provide evidence that one set of tools already stands out sufficiently to merit declaring a profile – the All Intra configuration, which, when applied to a single image, can provide a still image coding profile.

Effectively, all motion estimation, reference pictures, etc., would be dropped. We would, however, still have the overhead of working with a sequence (and sequence parameter sets, etc), only our sequence would have only one frame. While the overhead structure could be simplified to eliminate the unnecessary overhead, such a course would then create a profile and a bitstream structure that would not be compatible with Main. Such a scenario, while potentially useful in image coding in and of itself, is undesirable from other points of view, and not advocated herein.

In summary, we advocate leaving the syntax intact in Main Profile, use the configuration for All Intra, and limit the application to sequences with only one frame. Compatibility with Main then requires that no additional syntax be added, but perhaps only an SEI message to indicate a single-frame sequence. A specific mechanism for communicating this is not presented here, and can be safely entrusted to a breakout group as needed. We focus instead on presenting evidence that creating such a profile is worthwhile. For convenience, we will tentatively refer to such a profile as HEVC Still.

# Evidence in Support of a Still Image Profile: HEVC Still

The dominant still image coding standard in use today (in terms of deployment) is baseline JPEG, while the dominant standard in terms of lossy coding performance JPEG2000. As the strength of a new still image coding standard in this case would rest on coding efficiency (rather than coding complexity), we focus on indicating that HEVC Still is very competitive with JPEG2000. We caution that our evidence is quite preliminary, and limited to just a few images. However, the consistency of the results is what is remarkable, and encouraging.

We first remark that since HEVC is an 8-bit, YUV 4:2:0 codec, its application, at the outset, is limited to such data only.  While image data is mainly in the RGB space, we note that the conversion loss between these formats does not appear to be very consequential for typical consumer image coding applications (such as cameras, mobile, and web). So for our purposes, we begin and end with an 8-bit YUV 4:2:0 still image. For the JPEG2000 coding, we lift up to RGB, code/decode, and return to YUV 4:2:0.

This matter would certainly be an issue for high-quality, professional applications.  In that context, a fully high bit-depth, 4:4:4 video coding standard would be a more appropriate standard from which to base a professional still image coding standard. It is anticipated that when a professional extension of HEVC is established, that the matter of extracting a still image coding profile from it would be revisited at that time.

2.1. Comparison Testing with JPEG2000.

We performed some comparison testing between HEVC All Intra and JPEG2000 (we used the Kakadu software, v6.4). For convenience, without a known image test set, we tested on our existing video sequences that we use for high-resolution high bitdepth coding, and coded just the first frame in each.

An example result is presented below, for the first frame of the sequence CrowdRun. It can be seen that the HEVC HM 8.0 systematically beats the J2K codec. Here the x-axis is bits per pixel (bpp), and the y-axis is average PSNR, where, in YUV 4:2:0, the average is taken with the weighting ratio of 4:1:1 (e.g, Y = 2/3, and U,V are each 1/6), per our existing calculational tools.  The accompanying spreadsheet has additional data points, all consistent with the result below.

2.2. Settings

For the HM 8.0 (ahg7 version), we used the Intra HE10 default settings, for 8-bit input, chroma setting YUv420.

For JPEG2000, we used Kakadu v6.4

Single-layer mode
One tile per frame (no tiling)
9/7-tap biorthogonal Daubechies wavelet filters
5 levels of wavelet decomposition
Code-block size of 64x64 wavelet coefficients
EBCOT encoding scheme
R-D optimization for a given target bit rate.

Figure 1.  HM 8 vs JPEG2000 in still image coding (here on the first frame of the Crowdrun sequence). The x-axis is bpp, and the y-axis is average PSNR (where YUV420 is weighted 4:1:1).

# Conclusion

Preliminary evidence collected in testing HEVC v1 as a still image coder, versus JPEG2000, indicates that HEVC is highly competitive, and merits consideration for creating a still image profile.

# Patent rights declaration(s)

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).