| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0288 |
| --- | --- |

| Title: | Comments on Entropy Slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Brian Heng Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA Peisong Chen Broadcom Corporation 16340 West Bernardo Dr. San Diego, CA 92127 USA | Email: Email: Email: | wwan@broadcom.com bheng@broadcom.com peisongc@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

The current HEVC draft text specifies the concept of entropy slices but they are not included in the Main Profile and seem to be the only feature that has this status.  This contribution notes that entropy slices fail to break a number of cross-slice dependencies in CABAC decoding state.  As such, it is not possible to independently CABAC decode entropy slices. Because they no longer accomplish their intended purpose, this contribution recommends removing entropy slices from the HEVC specification.

# Discussion

The concept of entropy slices is specified in the current HEVC draft text [1] but not included in the Main Profile definition.  This seems to be the only feature that has this status which has been brought to the attention of companies at the US National Body Meeting with agreement among the US National Body to suggest that entropy slices be either added in the Main Profile or removed from the final standard for consistency.

The main advantage of entropy slices as originally proposed in JCTVC-B111 [2] was to provide parallel entropy decoding.  Entropy slices were intended to be completely independent from any other entropy slice, while still allowing cross-slice prediction for reconstruction. In this way, a decoder could be designed using multiple CABAC decoding engines to process entropy slices in parallel, without sacrificing as much coding efficiency as completely independent slices (slices whose entropy decoding and reconstruction are independent).

As currently described in the WD text and implemented in the HM software, entropy slices are identical to dependent slices, except that the CABAC context probability models are reset at the start of each entropy slice.  All other forms of cross-slice prediction are still used for both dependent slices and entropy slices. This includes using CTBs in neighboring slices for CABAC context determination and for intra mode prediction.

For example, the coefficient scan order for 8x8 and 4x4 intra coded transform units is a function of the intra prediction mode.  The coefficient scan order directly impacts the CABAC parsing process, so intra modes must be determined during CABAC decoding.  Therefore, entropy slices do not break intra mode prediction dependencies, and thus, entropy slices are not independently CABAC decodable.

# Proposed Solution

Given that entropy slices fail to accomplish their original goal of providing independent entropy decoding and since the HEVC standard provides other possible options for parallel decoding (such as wavefront parallel processing), it is recommended to remove entropy slices from the HEVC specification.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-J1003_d7, July 2012.

- K. Misra, J. Zhao, A. Segall, “Entropy Slices for Parallel Entropy Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-B111, July 2010.

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes

7.3.2.3	Picture parameter set RBSP syntax

| dependent_slice_enabled_flag | u(1) |
| --- | --- |
| tiles_enabled_flag | u(1) |
| entropy_coding_sync_enabled_flag | u(1) |
| entropy_slice_enabled_flag | u(1) |

7.4.2.3	Picture parameter set RBSP semantics

dependent_slice_enabled_flag equal to 1 specifies the presence of the syntax element dependent_slice_flag in the slice header for coded pictures referring to the picture parameter set. dependent_slice_enabled_flag equal to 0 specifies the absence of the syntax element dependent_slice_flag in the slice header for coded pictures referring to the picture parameter set. When entropy_slice_enabled_flag is equal to 1, the value of dependent_slice_enabled_flag shall be equal to 1.

…

entropy_slice_enabled_flag equal to 1 specifies that the CABAC decoding of coding tree blocks in a dependent slice is independent of the state of the previously decoded slice. entropy_slice_enabled_flag equal to 0 specifies that the CABAC decoding of coding blocks in a dependent slice depends on the state of the previously decoded slice. When dependent_slice_enabled_flag is equal to 0, the value of entropy_slice_enabled_flag shall be equal to 0.

NOTE – When entropy_slice_enabled_flag and dependent_slice_flag are both equal to 1 for a slice, the slice is an entropy slice.

9.3	CABAC parsing process for slice data

- The arithmetic decoding engine is initialized as follows.

  - If CtbAddrRS is equal to slice_address, entropy_slice_enabled_flag is equal to 0 and dependent_slice_flag is equal to 1, the following applies.

…

- When entropy_slice_enabled_flag is equal to 0 and dependent_slice_flag is equal to 1, or when entropy_coding_sync_enabled_flag is equal to 1, the memorization process is applied as follows.

  - When entropy_coding_sync_enabled_flag is equal to 1 and CtbAddrRS % PicWidthInCtbsY is equal to 2, the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxWPP and TableMPSValWPP as output.

  - When entropy_slice_enabled_flag is equal to 0, dependent_slice_flag is equal to 1, and end_of_slice_flag is equal to 1, the memorization process of the CABAC parsing process is invoked as specified in subclause Error: Reference source not found with TableStateIdxDS and TableMPSValDS as output.

A.3.2	Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Sequence parameter sets shall have seq_parameter_set_id in the range of 0 to 15, inclusive.

- Log2CtbSizeY shall be in the range from 4 to 6, inclusive.

- Picture parameter sets shall have tiles_enabled_flag && entropy_coding_sync_enabled_flag equal to 0.

- Picture parameter sets shall have entropy_slice_enabled_flag equal to 0.