| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0292 |
| --- | --- |

| Title: | AHG9: Scaling list for transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

During the 9th JCTVC meeting at Geneva, transform skip was adopted and the default matrix for transform skip was proposed by JCTVC-I0408. It proposed to use flat matrix for 4x4 transform when transform_skip_enabled flag is set to 1. The proposal was adopted, and transform_skip_enabled_flag was moved from SPS to PPS during the discussion, because there could be a sequence mixed by both natural video and synthetic video. However this change introduced a bug that the default scaling list in SPS can’t be derived because it depends on PPS. This contribution proposes solutions to fix this problem.

# Scaling list for transform skip

The default value of scaling list is specified in 7.4.5 as follows.

“

When scaling_list_pred_matrix_id_delta is equal to 0, i.e. refMatrixId is equal to matrixId, the scaling list is inferred from the default scaling list as follows.

–	If transform_skip_enabled_flag is equal to 1 and sizeId is equal to 0, ScalingList[ sizeId ][ matrixId ][ i ] is set equal to 16 for i = 0..15

–	Otherwise, ScalingList[ sizeId ][ matrixId ] is specified in Error: Reference source not found and Error: Reference source not found.

“

Scaling list can send in both SPS and PPS. The above definition is problem for SPS, because transform_skip_enabled_flag is exist in PPS, and the default value at SPS can’t be derived. To fix this, the following options can be considered.

## Turn off scaling list when transform_skip_enabled_flag is equal to 1

Change the above as follows.

“

When scaling_list_pred_matrix_id_delta is equal to 0, i.e. refMatrixId is equal to matrixId, the scaling list is inferred from the default scaling list as follows.

–

ScalingList[ sizeId ][ matrixId ] is specified in Error: Reference source not found and Error: Reference source not found.

“

Then this option is to change decoding process in 8.6.3 as follows.

“

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1, the following applies.

–	The scaling factor m[ x ][ y ] is derived as follows.

–	If scaling_list_enable_flag is equal to 0, or transform_skip_enabled_flag is equal to 1,

m[ x ][ y ] = 16		(8-1)

–	Otherwise (scaling_list_enable_flag is equal to 1 and transform_skip_enabled_flag is equal to 0),

m[ x ][ y ] = ScalingFactor[ sizeId ][ refMatrixId ][ x ][ y ]	(8-2)

Where the variables sizeId and refMatrixId are specified in Error: Reference source not found and Equation 7-4, respectively.

–	The scaled transform coefficient d[ x ][ y ] is derived as follows.

d[ x ][ y ] = Clip3( −32768, 32767, ( ( TransCoeffLevel[ xT ][ yT ][ cIdx ] * m[ x ][ y ] * 
					levelScale[ qP%6 ] << (qP/6)) + 	(8-3)
				(1 << (bdShift − 1 )) ) >> bdShift )

“

## Introduce intermediate parameter

In this option, intermediate parameter is introduced. The semantics in PPS in 7.4.2.3 is changed as follows.

“

transform_skip_enabled_flag equal to 1 specifies that transform_skip_flag may be present in the residual coding syntax. transform_skip_enabled_flag equal to 0 specifies that transform_skip_flag is not present in the residual coding syntax. When transform_skip_enabled_flag is equal to 1, is4x4FlatMatrix is set to 1. Otherwise the value of is4x4FlatMatrix is set to 0. The default value of is4x4FlatMatrix is 0.

“

Then the semantics in 7.4.5 is changed as follows.

“

scaling_list_pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When scaling_list_pred_mode_flag is equal to 0, scaling_list_pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following:

refMatrixId = matrixId − scaling_list_pred_matrix_id_delta	(7-4)

where matrixId is specified in Error: Reference source not found. The variable refMatrixId shall be greater than or equal to 0. When scaling_list_pred_matrix_id_delta is equal to 0, i.e. refMatrixId is equal to matrixId, the scaling list is inferred from the default scaling list as follows.

–	If is4x4FlatMatrix is equal to 1 and sizeId is equal to 0, ScalingList[ sizeId ][ matrixId ][ i ] is set equal to 16 for i = 0..15

–	Otherwise, ScalingList[ sizeId ][ matrixId ] is specified in Error: Reference source not found and Error: Reference source not found.

“

# Move transform_skip_enabled_flag

The problem was introduced because the location of transform_skip_enabled_flag was moved from SPS to PPS. The problem can be fixed by moving transform_skip_enabled_flag from PPS to SPS as JCTVC-I0408 proposed.

# Summary

Two options to fix the scaling list for transform skip are proposed in this contribution. The problem should be fixed during the discussion at Shanghai meeting.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).