| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0298 |
| --- | --- |

| Title: | Cross-check on Mirroring of Coefficients for Transform Skipping (JCTVC-K0294) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba JAPAN | Tel: Email: | +81 43 299 8526 tsukuba.takeshi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This documents reports the cross-check results of the proposal JCTVC-K0294 on Mirroring of Coefficients for Transform Skipping. In JCTVC-K0294, it is proposed to reorder coefficients in residual blocks depending on the scanning order when transform skip is used. The simulation results performed for this cross-verification exactly match those provided by the proponents.

# Verification

In JCTVC-K0294 [1], it is proposed to reorder coefficients in residual blocks depending on the scanning order when transform skip is used. According to [1], if the diagonal scan is used, residual samples are transposed across the anti-diagonal. Otherwise (horizontal scan or vertical scan), residual samples are rotated across the main diagonal.

The verification is carried under the common test conditions [2] by setting the following macro in the provided code:

#define TS_COEFF_FLIP                    1  ///< flip transform coefficients

The modifications introduced by the TS_COEF_FLIP macro are listed in Table 1. These modifications match the method described in [1].

Table 1: Modifications related to the TS_COEFF_FLIP macro

| File | Modification |
| --- | --- |
| TComTrQuant.h/cpp | The transformNxN, invtransformNxN, xTransfromSkip and xITransformSkip function are modified to take as input parameter the scanning order flag “diag”, which indicates whether the current scan is diagonal or not. In the xTransformSkip and xITransformSkip function, there are changes related to the proposal, reordering of coefficients in residual blocks depending on the scanning order. |
| TDecCu.cpp | The xIntraRecLumaBlk and xIntraRecChromaBlk function are modified to retrieve the scanning order flag. |
| TEncSearch.cpp | The xIntraCodingLumaBlk and xIntraCodingChromaBlk function are modified to retrieve the scanning order flag. |

# Experimental Results

Table 2 shows the summary of the experimental results. It is confirmed that the BD-rate results exactly match those provided by the proponents. Given the changes in the source code related to this method, the running time are not expected to differ from those of the anchor. The observed changes in the reported results may be due to some noise in our cluster.

Table 2: Performance on common condition (QP=22, 27, 32, 37)

# Conclusion

The cross-checkers results exactly match those of the proponents.

# Reference

- R. Weerakkody, M. Mrak, A. Gabriellini, “Mirroring of Coefficients for Transform Skipping,” JCTVC-K0294, Shanghai, Oct. 2012.

- F. Bossen, “HM 8 Common Test Conditions and Software Reference Configurations,” JCTVC-J1100, Stockholm, July 2012.