| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0302_r1 |
| --- | --- |

| Title: | AHG7: On processing 4:2:2 chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) | Andrea Gabriellini Marta Mrak | andrea@rd.bbc.co.uk marta.mrak@bbc.co.uk |  |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document presents the results of an experiment aimed at investigating the efficiency of coding 4:2:2 chroma formats using AHG7 software (branch HM-8.0-dev-ahg7). The modified codec takes a 4:2:2 input and it upconverts it to 4:4:4 before starting processing it. Thus the bitstream generates a 4:4:4 signal. Both locally decoded output (on the encoder side) and the decoder’s output are downconverted to 4:2:2 before writing to file to obtain a sequence the same chroma format as the input. The aim is to avoid having to tackle the potential issues generated by the 4:2:2 format, where non square chroma blocks must be processed. The experiment was limited to intra and low delay test points. The experiment provides a benchmark for the potential compression and complexity penalty associated with processing 4:2:2 sequences as 4:4:4 for further study.

# Introduction

The processing of 4:2:2 chroma format introduces some coding challenges in the form of non-square chroma component blocks. This shape requires adjusting of a number of compression methods, including angular intra prediction and support for non-square blocks during transform, quantization and entropy coding. It is possible to avoid these potential issues by upconverting the input data to 4:4:4 and downconverting the reconstructed (or decoded) data to 4:2:2. Since we are processing 50% more data we expect an increase in complexity. It is also expected that the coding efficiency of this approach may suffer compared to straight 4:2:2 coding; this experiment aims to evaluate the extent of both changes.

# The experiment

The software available to AHG7 (HM-8.0-dev-ahg7) is built on top of HM 8.0 and capable of handling several chroma formats, including 4:2:0, 4:2:2, and 4:4:4. When 4:2:2 coding is selected non-square chroma transform blocks are used. For this experiment a modified version of the AHG7 software has been used; the modified software reads 4:2:2 in and it upconverts it to 4:4:4. All subsequent processing happens at 4:4:4. The upconverting filter is a 4 tap filter (1, 3, 3, 1): even chroma components columns are identical to the original, odd columns are derived by applying the filter.

Finally the encoder’s locally reconstructed output and the decoder’s output are downconverted to 4:2:2 using a three tap filter (1, 2, 1).

The luma component is not affected by the filtering at any stage.

For this experiment four test points, i_main, i_he10, ld_main, and ld_he10, were run according to [1]. Also, the eight 4:2:2 sequences selected by AHG7 were used for the test.

# Results

Simulations were run for this test, upconverting 4:2:2 formats to 4:4:4 so that all compression tools (prediction, transform, etc) operate on 4:4:4 format. Results for two sets of experiments are provided. The first experiment was affected by a typo in the up converting filter which influenced results for the main configuration.

The experiments uses the same configuration used for the generation of the reference 4:2:2 sequences. Related results are reported in Table 1 and 2, with the rounding offset of 8 used for upscaling. The results show an increased complexity for both encoder and decoder, as expected. The larger increase is for the decoder where it reaches 28%. The results also show a decrease in compression efficiency when compared to direct coding of 4:2:2. The reduced compression efficiency of the luma component is likely the result of the increased bitrate since the PSNR values are largely unchanged. The chroma components are compressed less efficiently and this is confirmed by the PSNR values associated with these components.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | 5.1% | -0.5% | 2.9% | 4.8% | -3.4% | -3.1% |
| Parkscene | 2.9% | 9.5% | 17.4% | 2.7% | 7.2% | 14.1% |
| Birds-in-cage | 3.5% | 11.2% | 10.3% | 2.7% | 4.7% | -4.5% |
| Ducks-and-legs | 0.3% | 25.3% | 18.4% | 0.2% | 23.8% | 16.2% |
| Traffic | 4.0% | 7.8% | 9.6% | 3.8% | 4.9% | 4.2% |
| Crowd-run | 1.5% | 25.4% | 22.3% | 1.4% | 22.6% | 19.1% |
| Old-town | -1.2% | 21.2% | 10.2% | -1.6% | 18.6% | 4.7% |
| Seeking | 0.8% | 18.6% | 13.3% | 0.6% | 15.7% | 10.1% |
| Overall | 2.1% | 14.8% | 13.0% | 1.8% | 11.7% | 7.6% |
| Enc Time[%] | 115% |  |  | 116% |  |  |
| Dec Time[%] | 128% |  |  | 127% |  |  |

Table 1: 4:2:2 upconverted to 4:4:4 vs 4:2:2 anchors, all intra test points, rounding offset 8.

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | 10.6% | -18.7% | -8.9% | 10.6% | -25.4% | -16.6% |
| Parkscene | 10.8% | -5.4% | 2.4% | 10.5% | -10.8% | -1.4% |
| Birds-in-cage | 22.8% | 1.3% | 22.2% | 24.8% | -10.8% | -2.5% |
| Ducks-and-legs | 11.6% | 17.8% | 11.4% | 11.3% | 12.8% | 7.5% |
| Traffic | 12.6% | -3.3% | 1.6% | 12.7% | -9.5% | -8.5% |
| Crowd-run | 4.7% | 5.7% | 3.0% | 4.5% | 1.1% | -1.5% |
| Old-town | 5.3% | -3.6% | -11.8% | 4.5% | -12.5% | -21.9% |
| Seeking | 4.9% | -1.9% | -7.0% | 4.7% | -6.1% | -11.0% |
| Overall | 10.4% | -1.0% | 1.6% | 10.5% | -7.7% | -7.0% |
| Enc Time[%] | 109% |  |  | 109% |  |  |
| Dec Time[%] | 125% |  |  | 123% |  |  |

Table 2: 4:2:2 upconverted to 4:4:4 vs 4:2:2 anchors, low delay test points, rounding offset 8.

As a second term of reference the results of this experiment have also been compared to the AVC anchors obtained by running JM 18.4 on the same set of input with 4:2:2 format (8-bit internal processing).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | -20.5% | -27.1% | -30.2% | -24.3% | -33.4% | -40.2% |
| Parkscene | -7.9% | -15.8% | -11.7% | -9.3% | -19.1% | -16.0% |
| Birds-in-cage | -20.3% | -22.3% | -29.8% | -27.0% | -32.8% | -51.6% |
| Ducks-and-legs | -5.6% | -5.6% | -13.8% | -6.7% | -7.5% | -16.4% |
| Traffic | -14.9% | -19.9% | -21.6% | -16.6% | -23.9% | -29.6% |
| Crowd-run | -9.2% | -7.6% | -10.8% | -9.9% | -10.3% | -14.0% |
| Old-town | -10.4% | -13.2% | -22.6% | -12.2% | -18.0% | -30.0% |
| Seeking | -9.7% | -13.3% | -17.1% | -10.7% | -16.7% | -21.0% |
| Overall | -12.3% | -15.6% | -19.7% | -14.6% | -20.2% | -27.3% |
| Enc Time[%] | 79% |  |  | 79% |  |  |
| Dec Time[%] | 182% |  |  | 187% |  |  |

Table 3: 4:2:2 upconverted to 4:4:4 vs JM 18.4 4:2:2 (8-bit processing), all intra test points, rounding offset 8.

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | -42.4% | -41.3% | -34.5% | -45.3% | -50.6% | -47.5% |
| Parkscene | -31.8% | -40.7% | -34.0% | -33.9% | -47.1% | -40.9% |
| Birds-in-cage | -62.2% | -63.9% | -57.4% | -68.4% | -77.6% | -79.2% |
| Ducks-and-legs | -33.2% | -26.0% | -37.6% | -35.2% | -30.5% | -42.2% |
| Traffic | -35.2% | -40.7% | -43.0% | -37.9% | -48.8% | -56.8% |
| Crowd-run | -18.3% | -24.2% | -25.7% | -19.1% | -29.2% | -31.2% |
| Old-town | -48.6% | -55.3% | -55.7% | -51.6% | -67.6% | -69.2% |
| Seeking | -27.2% | -32.9% | -36.1% | -28.5% | -38.8% | -43.3% |
| Overall | -37.4% | -40.6% | -40.5% | -40.0% | -48.8% | -51.3% |
| Enc Time[%] | 77% |  |  | 77% |  |  |
| Dec Time[%] | 136% |  |  | 143% |  |  |

Table 4: 4:2:2 upconverted to 4:4:4 vs JM 18.4 4:2:2 (8-bit processing), all low delay test points, rounding offset 8.

Because of the suboptimal rounding offset of 8 used in the first test set, the experiment was repeated with the rounding offset of 4. Rounding offset of 4 is suitable for used upscaling filter [1, 3, 3, 1]. Results with the rounding offset of 4 are reported in Tables 5 to 8. Corrected rounding offset contributes to better compression on chroma components for Main configurations where 8-bit processing is used.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | 5.1% | -3.4% | -2.5% | 4.8% | -3.6% | -3.6% |
| Parkscene | 2.9% | 7.8% | 15.0% | 2.7% | 7.1% | 14.0% |
| Birds-in-cage | 3.5% | 5.4% | -3.1% | 2.7% | 4.3% | -5.3% |
| Ducks-and-legs | 0.3% | 24.1% | 16.8% | 0.2% | 23.7% | 16.1% |
| Traffic | 3.9% | 5.7% | 5.0% | 3.8% | 4.7% | 3.9% |
| Crowd-run | 1.5% | 23.7% | 20.2% | 1.4% | 22.5% | 18.9% |
| Old-town | -1.3% | 18.1% | 5.7% | -1.6% | 18.3% | 4.4% |
| Seeking | 0.8% | 16.7% | 10.9% | 0.6% | 15.5% | 9.9% |
| Overall | 2.1% | 12.3% | 8.5% | 1.8% | 11.6% | 7.3% |
| Enc Time[%] | 111% |  |  | 112% |  |  |
| Dec Time[%] | 118% |  |  | 119% |  |  |

Table 5: 4:2:2 upconverted to 4:4:4 vs 4:2:2 anchors, all intra test points, rounding offset 4.

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | 10.6% | -23.1% | -15.5% | 10.5% | -25.8% | -17.1% |
| Parkscene | 10.8% | -7.9% | -0.8% | 10.5% | -11.0% | -1.7% |
| Birds-in-cage | 22.9% | -14.4% | -7.4% | 25.1% | -12.3% | -5.3% |
| Ducks-and-legs | 11.5% | 15.5% | 8.7% | 11.3% | 12.5% | 7.4% |
| Traffic | 12.7% | -6.9% | -6.9% | 12.7% | -9.7% | -8.9% |
| Crowd-run | 4.7% | 3.6% | 0.8% | 4.5% | 0.9% | -1.6% |
| Old-town | 5.3% | -11.7% | -21.2% | 4.5% | -13.3% | -22.3% |
| Seeking | 4.9% | -4.5% | -10.0% | 4.7% | -6.3% | -11.3% |
| Overall | 10.4% | -6.2% | -6.5% | 10.5% | -8.1% | -7.6% |
| Enc Time[%] | 109% |  |  | 109% |  |  |
| Dec Time[%] | 123% |  |  | 121% |  |  |

Table 6: 4:2:2 upconverted to 4:4:4 vs 4:2:2 anchors, low delay test points, rounding offset 4.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | -20.4% | -29.4% | -34.0% | -24.3% | -33.6% | -40.5% |
| Parkscene | -7.9% | -16.9% | -13.3% | -9.3% | -19.1% | -16.1% |
| Birds-in-cage | -20.3% | -25.7% | -38.0% | -27.0% | -33.0% | -52.0% |
| Ducks-and-legs | -5.6% | -6.3% | -14.7% | -6.7% | -7.5% | -16.4% |
| Traffic | -14.9% | -21.3% | -24.5% | -16.6% | -24.0% | -29.8% |
| Crowd-run | -9.2% | -8.5% | -12.0% | -9.9% | -10.4% | -14.1% |
| Old-town | -10.5% | -15.2% | -25.6% | -12.2% | -18.1% | -30.2% |
| Seeking | -9.7% | -14.4% | -18.6% | -10.7% | -16.8% | -21.1% |
| Overall | -12.3% | -17.2% | -22.6% | -14.6% | -20.3% | -27.5% |
| Enc Time[%] | 76% |  |  | 76% |  |  |
| Dec Time[%] | 168% |  |  | 175% |  |  |

Table 7: 4:2:2 upconverted to 4:4:4 vs JM 18.4 4:2:2 (8-bit processing), all intra test points, rounding offset 4.

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | -42.4% | -44.3% | -39.1% | -45.3% | -50.8% | -47.8% |
| Parkscene | -31.9% | -42.4% | -36.0% | -33.9% | -47.2% | -41.1% |
| Birds-in-cage | -62.2% | -69.3% | -67.9% | -68.3% | -78.1% | -79.9% |
| Ducks-and-legs | -33.2% | -27.3% | -39.1% | -35.2% | -30.6% | -42.3% |
| Traffic | -35.2% | -43.1% | -48.2% | -37.9% | -48.8% | -57.0% |
| Crowd-run | -18.3% | -25.5% | -27.2% | -19.1% | -29.4% | -31.2% |
| Old-town | -48.6% | -59.9% | -60.8% | -51.6% | -68.0% | -69.4% |
| Seeking | -27.2% | -34.6% | -38.3% | -28.5% | -38.9% | -43.5% |
| Overall | -37.4% | -43.3% | -44.6% | -40.0% | -49.0% | -51.5% |
| Enc Time[%] | 76% |  |  | 77% |  |  |
| Dec Time[%] | 134% |  |  | 141% |  |  |

Table 8: 4:2:2 upconverted to 4:4:4 vs JM 18.4 4:2:2 (8-bit processing), all low delay test points, rounding offset 4.

# Areas for future study

If upconverting 4:2:2 data to 4:4:4 is deemed an interesting solution then keeping the entire coding chain at 4:4:4 resolution may be wasteful of resources, especially memory. It may be useful to investigate whether to use a 4:2:2 version of the signal for motion prediction, where the presence of non-square blocks does not present an issue. This would allow shrinking the reference buffer.

# References

- F. Bossen, “HM 8 Common Test Conditions and Software Reference Configurations”, JCTVC-J1100, 10th meeting, Stockholm, Sweden, July 2012.

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).