| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0306 |
| --- | --- |

| Title: | Cross-check report of JCTVC-K0239 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 yoshitomo.takahashi@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results on JCTVC-K0239.

In JCTVC-K0239 [1], TMVP hook for MV-HEVC is proposed to Ver. 1. The reference index of TMVP on merge mode can be changed from 0 to non-zero value so that the reference pictures of current block and co-located block can belong to the same reference picture types. 

It is reported that the cross-checking results match with the ones provided by the proponent. The cross-checking reports show 0.94% BD-rate improvement in the total of three texture views in HTM-4.0 software and 0.95% BD-rate improvement in the total of three texture views in MV-HEVC software.

It is commented that the proposed change is desirable as a hook for MV-HEVC. However, it is also commented that the search complexity increase for the alternative reference index is not welcomed, especially in the worst case. The cross-checker recommends to adopt JCTVC-K0239 with the following worst case care.

The cross-checker proposes to introduce a restriction that JCTVC-K0239 is adopted only if restricted_ref_pic_lists_flag is equal to 1, in order to reduce the search complexit in the worst case.

# Cross-checking

Two kinds of software and experimental results were provided by the proponent. The simulation has been conducted under common test condition of JCT-3V [2] with the following conditions.

First one is the software and results based on HTM-4.0. The anchor data is created in HTM-4.0 software which is modified so that the reference index of TMVP on merge mode is always set to 0. This modification is adapted when the macro QC_TMVP_MRG_IDX_MODIFY is set to 1 in the software. The proposal is implemented on the modified HTM-4.0 software and adapted when the macro QC_TMVP_MRG_IDX is set to 1. The inter-view motion prediction and inter-view residual prediction are disabled in both anchor and proposal.

Second one is the software and results based on MV-HEVC which being provided to JCT-3V as part of the AhG 5 activity and has not been opened to the public yet. This software aligns with the MV-HEVC WD [3]. The anchor data is created in MV-HEVC software which is modified so that the reference index of TMVP on merge mode is always set to 0. This modification is adapted when the macro QC_TMVP_MRG_IDX_MODIFY is set to 1 in the software. The proposal is implemented on the MV-HEVC software and adapted when the macro QC_TMVP_MRG_IDX is set to 1.

# Results

The cross-checking results are summarized in the following tables. The detailed results are shown in the attached excel-file. Table 1 and Table 2 show the results in HTM-4.0 and MV-HEVC, respectively.

Table 1: Results in HTM-4.0

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | -3.13% | -2.94% | -1.39% |
| Kendo | 0.0% | -2.42% | -2.28% | -1.03% |
| Newspapercc | 0.0% | -1.55% | -1.17% | -0.59% |
| GhostTownFly | 0.0% | -5.62% | -5.38% | -1.54% |
| PoznanHall2 | 0.0% | -1.61% | -1.24% | -0.64% |
| PoznanStreet | 0.0% | -2.12% | -2.01% | -0.71% |
| UndoDancer | 0.0% | -1.86% | -1.90% | -0.69% |
| 1024x768 | 0.0% | -2.37% | -2.13% | -1.00% |
| 1920x1088 | 0.0% | -2.80% | -2.63% | -0.90% |
| average | 0.00% | -2.62% | -2.42% | -0.94% |

Table 2: Results in MV-HEVC

|  | video 0 | video 1 | video 2 | video only |
| --- | --- | --- | --- | --- |
| Balloons | 0.0% | -3.16% | -2.93% | -1.40% |
| Kendo | 0.0% | -2.50% | -2.22% | -1.03% |
| Newspapercc | 0.0% | -1.52% | -1.20% | -0.59% |
| GhostTownFly | 0.0% | -5.70% | -5.49% | -1.55% |
| PoznanHall2 | 0.0% | -1.92% | -1.03% | -0.67% |
| PoznanStreet | 0.0% | -2.14% | -2.06% | -0.71% |
| UndoDancer | 0.0% | -1.89% | -2.00% | -0.69% |
| 1024x768 | 0.0% | -2.39% | -2.12% | -1.01% |
| 1920x1088 | 0.0% | -2.91% | -2.64% | -0.91% |
| average | 0.00% | -2.69% | -2.42% | -0.95% |

# Comments

JCTVC-K0239 can improve the coding efficiency in JCT-3V test sequences. Thus it seems that this proposal is significant as a hook for 3D extension unless there are harmful effects in HEVC ver.1.

There is a complexity concern about the searching process of non-zero reference index. The number of searches depends on the number of slices and reference pictures in a picture. According to the current HEVC specification, a picture can be divided into 600 slices and the maximum number of reference pictures is 16 pictures in each reference picture list. Thus the number of searches reaches to 18,000 (=600 slices x (16-1) ref. pic. x 2 List0/List1) in the worst case where a picture includes 600 slices and the number of reference picture is 16 pictures in each reference list.

# Proposal

As mentioned in above comment, there is a concern about the search complexity of non-zero reference index in the worst case. It is suggested that a restriction to JCTVC-K0239 is introduced in order to reduce the search comprexity in the worst case. 

In JCTVC-K0239, the search process is always executed once in all slices since each slice may have the different reference picture lists. This is the main cause of the search complexity increase in the worst case where a picture includes 600 slices.

It is proposed that the search process is executed only if all slices in a picture share the same reference picture lists, otherwise, the search process is skipped.

In current HEVC specification, the syntax restricted_ref_pic_lists_flag in SPS indicates that all slices in a picture have the same reference picture lists. Thus it is possible to switch the above condition by checking this syntax.

The proposed restriction can be summarized as following,

- If restricted_ref_pic_lists_flag is equal to 1, the search process of non-zero reference index and TMVP on merge mode in JCTVC-K0239 are adopted.

- Otherwise, if restricted_ref_pic_lists_flag is equal to 0, the search process is skipped and current TMVP is adopted.

The proposed restriction can restrict the number of search to 30 (=1 picture x (16-1) ref. pic. x 2 List0/List1) in the worst case.

# Conclusion

In this contribution, the cross-checking results on JCTVC-K0239 are reported. It is confirmed that the results match with the ones provided by the proponent. It is also suggested that a restriction to JCTVC-K0239 is introduced in order to reduce the search complexity in the worst case.

# Reference

[1] ISO/IEC JTC1/SC29/WG11, “Temporal motion vector prediction hook for MV-HEVC”, Doc. JCTVC-K0239, Shanghai, CN, Oct. 2012.
[2] ISO/IEC JTC1/SC29/WG11, “Common Test Conditions of 3DV Core Experiments”, Doc. JCT3V-A1100, Stockholm, SE, July 2012.
[3] ISO/IEC JTC1/SC29/WG11, “MV-HEVC Working Draft 1”, Doc. JCT3V-A1004, Stockholm, SE, July 2012.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).