| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0309 |
| --- | --- |

| Title: | The cross-check for use of Chroma filter for Luma interpolation (JCTVC-K0260) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

Test results for Use of Chroma interpolation filter for Luma MC are verified in this report. Proposed by Motorola change leads to average performance degradation 6%/2.8%/2.8% (in terms of Luma/Cb/Cr BD-rate). If half-pel Chroma filter is changed to sharper one then losses are smaller: 3.6%/2.0%/1.9% (in terms of Luma/Cb/Cr BD-rate). Despite of loss in average suggested modifications bring gain for class A sequences in RA test case.

# Introduction

During JCTVC-9th (Geneva) meeting bi-prediction for PU smaller than 8x8 was restricted in order to resolve memory band-width issue. Among several solutions that time the simplest (but not the best performance) variant was accepted.

In verified contribution Motorola suggests [1] to use Chroma interpolation filter (which is 4 taps in HEVC) for Luma motion compensation. Potential benefit from this is memory band-width reduction. But after bi-pred restriction for blocks smaller than 8x8 this is not an issue any more.

Additionally they suggest modifying half-pel Chroma interpolation filter as follows:

- HM8.0 Chroma interpolation {-4,36,36,-4}

- Suggested Chroma interpolation {-7,39,39,-7}

Frequency responses for both these filters are shown on Figure 1 together with “ideal” frequency response (solid line). It is easy to notice that frequency response of suggested modification for half-pel Chroma interpolation filter has local maximum in middle-high frequency zone, which may lead to interpolation error accumulation. This phenomenon was studied during CE on motion compensation interpolation during HEVC development, and it is known that visual quality degradation is also possible, especially if SAO is not applied.

Figure 1. Frequency response of HEVC Chroma interpolation filter and suggested in K0260modification.

# Verification tests results

Objective performance was tested using common test conditions [2] and code provided by proponent. Provided s/w implementation matches with the description in the contribution document [1]. Performance test results for test 1 (Chroma filter is applied fo Luma MC without modification) are summarized in Table 1. For test 2 (additionally Chroma filter is modified) test results are shown in Table 2. Decoder run-time is noticeable higher than anchopr, but this is due to MD5 calculation which was not disabled in provided code.

Table 1. Performance test results summary: Chroma filter is used for Luma interpolation.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -0.8% | -0.9% | -0.8% | -0.7% | -0.9% |
| Class B | 3.8% | 0.4% | 0.4% | 4.0% | 0.5% | 0.6% |
| Class C | 9.8% | 3.5% | 3.8% | 9.8% | 3.7% | 3.8% |
| Class D | 20.4% | 8.8% | 8.8% | 20.4% | 8.6% | 8.6% |
| Class E |  |  |  |  |  |  |
| Overall | 8.0% | 2.8% | 2.9% | 8.1% | 2.9% | 2.9% |
| Class F | 0.9% | 0.0% | 0.1% | 1.0% | 0.1% | 0.2% |
| Enc Time[%] | 92% |  |  | 93% |  |  |
| Dec Time[%] | 102% |  |  | 107% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 5.2% | 1.4% | 1.1% | 5.7% | 2.3% | 2.4% |
| Class C | 9.9% | 4.1% | 4.5% | 10.0% | 4.8% | 5.3% |
| Class D | 19.2% | 10.8% | 9.9% | 19.3% | 12.2% | 11.3% |
| Class E | 5.0% | 3.8% | 4.3% | 6.0% | 5.2% | 5.0% |
| Overall | 9.8% | 4.9% | 4.8% | 10.2% | 5.9% | 5.8% |
| Class F | 1.3% | -0.1% | -0.3% | 1.2% | 0.1% | -0.8% |
| Enc Time[%] | 92% |  |  | 92% |  |  |
| Dec Time[%] | 107% |  |  | 110% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.9% | -0.9% | -1.1% | 0.7% | -0.2% | -0.7% |
| Class C | 7.1% | 2.6% | 3.3% | 6.8% | 2.8% | 2.8% |
| Class D | 15.2% | 9.1% | 7.8% | 14.8% | 8.2% | 9.0% |
| Class E | 1.3% | 1.8% | 1.2% | 1.5% | 2.2% | 2.8% |
| Overall | 6.1% | 3.0% | 2.7% | 5.9% | 3.1% | 3.3% |
| Class F | 0.5% | -0.6% | 0.0% | 0.3% | -1.0% | 0.0% |
| Enc Time[%] | 95% |  |  | 94% |  |  |
| Dec Time[%] | 109% |  |  | 115% |  |  |

Table 2. Performance test results summary: Modified Chroma filter is used for Luma interpolation.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.1% | -1.3% | -1.2% | -1.0% | -1.1% | -1.1% |
| Class B | 1.7% | -0.7% | -0.6% | 1.8% | -0.5% | -0.3% |
| Class C | 7.2% | 1.8% | 2.0% | 7.2% | 2.1% | 2.1% |
| Class D | 15.0% | 5.4% | 4.8% | 14.9% | 5.5% | 5.2% |
| Class E |  |  |  |  |  |  |
| Class F | 0.7% | -0.2% | -0.1% | 0.7% | -0.2% | -0.1% |
| Overall | 4.6% | 0.9% | 0.9% | 4.6% | 1.1% | 1.1% |
| Enc Time[%] | 92% |  |  | 93% |  |  |
| Dec Time[%] | 105% |  |  | 111% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.9% | 0.0% | -0.5% | 3.3% | 3.5% | 4.4% |
| Class C | 7.6% | 2.1% | 1.9% | 7.7% | 3.8% | 4.3% |
| Class D | 14.9% | 7.1% | 5.0% | 15.1% | 10.3% | 9.6% |
| Class E | 2.6% | 4.1% | 4.5% | 3.6% | 8.7% | 8.4% |
| Class F | 1.0% | -0.5% | -0.2% | 0.8% | -0.1% | -0.3% |
| Overall | 5.8% | 2.3% | 1.9% | 6.1% | 5.0% | 5.1% |
| Enc Time[%] | 92% |  |  | 92% |  |  |
| Dec Time[%] | 111% |  |  | 114% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.8% | -0.9% | 0.3% | 3.2% | 3.7% |
| Class C | 5.8% | 1.4% | 1.7% | 5.6% | 2.9% | 3.5% |
| Class D | 12.2% | 6.1% | 4.2% | 12.1% | 9.1% | 8.3% |
| Class E | 0.7% | 4.5% | 3.1% | 1.4% | 10.1% | 10.2% |
| Class F | 0.4% | -0.5% | 0.3% | 0.2% | -0.3% | -0.5% |
| Overall | 3.8% | 1.9% | 1.5% | 3.9% | 4.7% | 4.7% |
| Enc Time[%] | 95% |  |  | 94% |  |  |
| Dec Time[%] | 111% |  |  | 116% |  |  |

Both suggested modifications lead to huge performance drop for classes D,C and E; moderate drop for classes B and A. In random access case suggested modifications show gain for class A.

It is well know that for noisy signals short-tap interpolation filters outperform long-tap filters. But in opposite for high quality signals longer filters provide much more accurate interpolation which results in BD-rate gain. Capture devices for low resolution video provide much higher quality of captured signal, compare to high resolution. So some noise exists originally in class A sequences, so long-tap filter doesn’t perform well for them. But quick evolution of capture devices will lead to better performance of long interpolation filters for high resolution videos. It is known from video codec development history that ALF gain for KTA sequences (captured long time ago) is much higher than ALF gain for video sequences currently used for test (captured recently). Due to improvement of capture devices there is less noise to fix by ALF. We would expect the same tendency for long/short interpolation filters: with improvement of capture devices long-taps filters will work better.

Switching between long and short filters according to the resolution of video doesn’t seem to as to be a complexity reduction, since in the worst case decoder will still use 8 taps filetr.

# Conclusion

Samsung confirms objective performance reported by Motorola in tests related to Chroma interpolation filter usage for Luma MC. We have found performance degradation (in average 6%/2.8%/2.8% in terms of Luma/Cb/Cr BD-rate). If half-pel Chroma filter is changed to sharper one then losses are smaller: 3.6%/2.0%/1.9% (in terms of Luma/Cb/Cr BD-rate). Despite of loss in average suggested modifications bring gain for class A sequences in RA test case.

# References

- J. Lou, K. Minoo, Y. Yu, L. Wang (Motorola Mobility), “Syntax to support use Chroma filter for Luma interpolation”, Joint Collaborative Team on Video Coding, JCTVC-K0260, Oct., 2012.

- F. Bossen “HM 8 Common Test Conditions and Software Reference Configurations”, Joint Collaborative Team on Video Coding, JCTVC-J1100, July, 2012.