| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0310 |
| --- | --- |

| Title: | Cross-check of JCTVC-K0139 (Contouring artefact and solution) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi | Tel: Email: | +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for JCTVC-K0139 (Contouring artifact and solution). It is reported that there is a match to the BD-Rate results generated by the proponents. Informal visual testing was also done and comments on those are also provided in this contribution.

# BD-Rate results on common test conditions

|  | Class A-E | Class F |
| --- | --- | --- |
| AI-Main | 0.1% | 0.0% |
| RA-Main | 0.0% | 0.0% |
| LB-Main | 0.0% | 0.1% |
| LP-Main | 0.0% | 0.2% |
| AI-HE10 | 0.0% | 0.0% |
| RA-HE10 | 0.0% | 0.1% |
| LB-HE10 | 0.0% | 0.0% |
| LP-HE10 | 0.0% | -0.1% |

# Visual quality evaluation

Johnny and KristenAndSara reconstructed sequences for LB, QP=37 were viewed in comparison to HM-8.0. The strong diagonal contouring in HM-8.0 for these sequences is observed to be weakened in JCTVC-K0139.

In KarstenAndSara, instead of a few strong contours in HM8 (KristenAndSara_QP37_FR60_IP0_BD8-HM8_frame000000.bmp) we observe many weak contours in JCTVC-K0139 (KristenAndSara_QP37_FR60_IP0_BD8-Contour_frame000000.bmp).

In Johnny, the strong diagonal contours of HM8 (Johnny_QP37_FR60_IP0_BD8-HM8_frame000000.bmp) are reduced, however in JCTVC-K0139 (Johnny_QP37_FR60_IP0_BD8-Contour_frame000000.bmp) we observe grid type of artifacts.

The .bmp files are attached in the zip archive.