| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0311 |
| --- | --- |

| Title: | Cross-check of JCTVC-K0130 Method A |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jun Xu, Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This document reports cross verification results for Method A of JCTVC-K0130 “Support of weighted prediction in the HEVC deblocking filter”.  Cross-check simulation confirms that results match with those from proponents. Study of software reports that implement matches the description of Method A in JCTVC-K0130.

# Introduction

In JCTVC-K0130, changes for deblocking filter are proposed for weighted prediction enabled cases. Method A in JCTVC-K0130 changes the conditions for deblocking filter strength related to reference list are presented. The condition of “different reference pictures” is changed to “different prediction lists” and “different reference indices for the same list”.

# Software and simulation set up

The software provided in JCTVC-K0130 is built based HM8.0 revision 2794. All changes proposed in JCTVC-K0130 is implemented under macro “APPLE_DEBLOCK” defined in typedef.h. The changes are all located in the xGetBoundaryStrengthSingle function in TComLoopFilter.cpp.

The cross-check is set up with comparison between “APPLE_DEBLOCK” OFF and “APPLE_DEBLOCK” ON. The anchor is “APPLE_DEBLOCK” OFF.  Since the algorithm only changes deblocking filter of B slices, Randomaccess main and Lowdelay B main in JCTVC-J1100 are the only two conditions tested in this cross-check report.

# Simulation results

The simulation results by Sony are summarized in Table 1. Detailed results can be found in accompanying Excel file. Since the simulation is run under a hybrid cluster, the encoding and decoding time is not accurate.

Table 1 Summary of cross-check for JCTVC-K0130 Method A

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.3% | -0.2% |
| Class C | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | 0.2% | -0.1% |
| Class E | 0.0% | 0.3% | 0.5% |
| Overall | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.2% | 0.0% |
| Class F | 0.0% | 0.0% | -0.2% |