| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0312 WG11 Number: m26710 |
| --- | --- |

| Title: | AHG7: Cross-verification of JCTVC-K0302, On processing 4:2:2 chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of JCTVC-K0302, “AHG7: On processing 4:2:2 chroma format.” Simulations were performed using the HM-8.0-dev-ahg7 software as a reference and subsequent software modifications provided by the BBC for testing. The simulation results performed for this cross-verification match those provided by the authors of JCTVC-K0302.

# Description of cross-verification and results

The HM-8.0-dev-ahg7 [1], which is used to generate the 4:2:2 anchors for this cross-verification, is capable of processing 4:2:2 and 4:4:4 video. The software provided by the authors of JCTVC-K0302 [2] includes modifications that upsample 4:2:2 video input to the encoder, so that the encoder processes everything using the 4:4:4 format. The decoded video is then downsampled to 4:2:2. Experiments are performed on the set of 4:2:2 sequences selected by AHG7 [3], using the common test conditions [4] for all-intra and low-delay (B) configurations. BD-Rate results for this cross-verification are as follows:

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | 5.1% | -0.5% | 2.9% | 4.8% | -3.4% | -3.1% |
| Parkscene | 2.9% | 9.5% | 17.4% | 2.7% | 7.2% | 14.1% |
| BirdsInCage | 3.5% | 11.2% | 10.3% | 2.7% | 4.7% | -4.5% |
| DucksAndLegs | 0.3% | 25.3% | 18.4% | 0.2% | 23.8% | 16.2% |
| Traffic | 4.0% | 7.8% | 9.6% | 3.8% | 4.9% | 4.2% |
| CrowdRun | 1.5% | 25.4% | 22.3% | 1.4% | 22.6% | 19.1% |
| OldTownCross | -1.2% | 21.2% | 10.2% | -1.6% | 18.6% | 4.7% |
| Seeking | 0.8% | 18.6% | 13.3% | 0.6% | 15.7% | 10.1% |
| Overall | 2.1% | 14.8% | 13.0% | 1.8% | 11.7% | 7.6% |
| Enc Time[%] | 118% |  |  | 117% |  |  |
| Dec Time[%] | 130% |  |  | 132% |  |  |

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Kimono | 10.6% | -18.7% | -8.9% | 10.6% | -25.4% | -16.6% |
| Parkscene | 10.8% | -5.4% | 2.4% | 10.5% | -10.8% | -1.4% |
| BirdsInCage | 22.8% | 1.3% | 22.2% | 24.8% | -10.8% | -2.5% |
| DucksAndLegs | 11.6% | 17.8% | 11.4% | 11.3% | 12.8% | 7.5% |
| Traffic | 12.6% | -3.3% | 1.6% | 12.7% | -9.5% | -8.5% |
| CrowdRun | 4.7% | 5.7% | 3.0% | 4.5% | 1.1% | -1.5% |
| OldTownCross | 5.3% | -3.6% | -11.8% | 4.5% | -12.5% | -21.9% |
| Seeking | 4.9% | -1.9% | -7.0% | 4.7% | -6.1% | -11.0% |
| Overall | 10.4% | -1.0% | 1.6% | 10.5% | -7.7% | -7.0% |
| Enc Time[%] | 112% |  |  | 112% |  |  |
| Dec Time[%] | 131% |  |  | 125% |  |  |

# Summary and conclusions

The BD-Rate results for this cross-verification match the corresponding set of results in JCTVC-K0302. The encoder and decoder run-times are close enough to be considered comparable.

# References

- Online: https://hevc.kw.bbc.co.uk/svn/jctvc-hm/branches/HM-8.0-dev-ahg7

- A. Gabriellini and M. Mrak, “AHG7: On processing 4:2:2 chroma format,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K0302, Shanghai, CN, October 2012

- Online: ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/

- F. Bossen, “HM 8 Common Test Conditions and Software Reference Configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J1100, Stockholm, SE, July 2012.