| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0315 |
| --- | --- |

| Title: | On default scaling list values (WD changes) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Andrea Gabriellini |  | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This document presents the changes to the working draft text specification when the default quantisation matrices (i.e. the default scaling lists) are removed. The changes are highlighted in yellow.

# Changes to the scaling list data semantics (Section 7.4.2.4)

scaling_list_pred_mode_flag equal to 0 specifies that the value of the scaling list is the same as the value of the scaling list already sent. The scaling list to copy the value is specified by scaling_list_pred_matrix_id_delta. scaling_list_pred_mode_flag equal to 1 speficies that the value of the scaling list is encoded by DPCM and exp-Golomb code.

scaling_list_pred_matrix_id_delta specifies the target reference matrix to copy the value of scaling list. When scaling_list_pred_mode_flag is equal to 0, scaling_list_pred_matrix_id_delta specifies which matrix should be used in the current matrix by the following:

RefMatrixID = MatrixID – scaling_list_pred_matrix_id_delta	(7-1)

where MatrixID is specified in Error: Reference source not found. The variable RefMatrixID shall be greater than or equal to 0. When scaling_list_pred_matrix_id_delta is equal to 0, i.e. RefMatrixID is equal to MatrixID, the scaling list values ScalingList[ SizeID ][ MatrixID ][ i ] is set to 16 for each i and SizeID. is inferred from the default scaling list as follows.

–	If transform_skip_enabled is equal to 1 and SizeID is equal to 0, ScalingList[ SizeID ][ MatrixID ][ i ] is set equal to 16 for i=0..15

–	Otherwise, ScalingList[ SizeID ][ MatrixID ] is specified in Table 7-4 and Table 7-5.

# Changes to the scaling list for matrix semantics (Section 7.4.2.5)

scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] plus 8 specifies the DC value of the scaling list for 16x16 size when sizeID is equal to 2 and specifies the DC value of the scaling list for 32x32 size when sizeID is equal to 3. The value of scaling_list_dc_coef_minus8[ sizeID − 2 ][ matrixID ] shall be in the range of −7 to 247, inclusive.

scaling_list_delta_coef specifies the difference of the matrix coefficient from the previous matrix coefficient, when pred_mode_flag is equal to 1. The value of scaling_list_delta_coef shall be in the range of −128 to 127, inclusive and set so that ScalingList[ sizeID ][ matrixID ][ i ] is greater than 0.

Table 7-4 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15 when transform_skip_enabled is equal to 0

| - I | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][ i ] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 20 | - 20 | - 21 | - 25 | - 30 | - 25 | - 41 | - 41 | - 70 |
| - ScalingList[0][3..5][ i ] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 21 | - 21 | - 21 | - 24 | - 24 | - 24 | - 36 | - 36 | - 57 |

Table 7-5 – Specification of default values of ScalingList[ 1..3 ][ MatrixID ][ 0 ][ i ] with i=0..63

| - I | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[1..2][0..2][ i ] - ScalingList[3][0][ i ] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 17 | - 16 | - 17 | - 16 | - 17 | - 18 |
| - ScalingList[1..2][3..5][ i ] - ScalingList[3][1][ i ] | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 17 | - 17 | - 18 |
| - i – 16 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[1..2][0..2][ i ] - ScalingList[3][0][ i ] | - 17 | - 18 | - 18 | - 17 | - 18 | - 21 | - 19 | - 20 | - 21 | - 20 | - 19 | - 21 | - 24 | - 22 | - 22 | - 24 |
| - ScalingList[1..2][3..5][ i ] - ScalingList[3][1][ i ] | - 18 | - 18 | - 18 | - 18 | - 18 | - 20 | - 20 | - 20 | - 20 | - 20 | - 20 | - 20 | - 24 | - 24 | - 24 | - 24 |
| - i − 32 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[1..2][0..2][ i ] - ScalingList[3][0][ i ] | - 24 | - 22 | - 22 | - 24 | - 25 | - 25 | - 27 | - 30 | - 27 | - 25 | - 25 | - 29 | - 31 | - 35 | - 35 | - 31 |
| - ScalingList[1..2][3..5][ i ] - ScalingList[3][1][ i ] | - 24 | - 24 | - 24 | - 24 | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 | - 25 | - 28 | - 28 | - 28 | - 28 | - 28 |
| - i − 48 | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| - ScalingList[1..2][0..2][ i ] - ScalingList[3][0][ i ] | - 29 | - 36 | - 41 | - 44 | - 41 | - 36 | - 47 | - 54 | - 54 | - 47 | - 65 | - 70 | - 65 | - 88 | - 88 | - 115 |
| - ScalingList[1..2][3..5][ i ] - ScalingList[3][1][ i ] | - 28 | - 33 | - 33 | - 33 | - 33 | - 33 | - 41 | - 41 | - 41 | - 41 | - 54 | - 54 | - 54 | - 71 | - 71 | - 91 |

# Patent rights declaration(s)

British Broadcasting Corporation (BBC) R&D does not have any current or pending patent rights relating to the technology described in this contribution.