| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0315 |
| --- | --- |

| Title: | On default scaling list values |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Andrea Gabriellini |  | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

Default scaling lists, also denoted as default quantisation matrices, are integral part of the latest version of HEVC draft specification. The usage of these scaling lists in the HEVC codec may improve the perceived video quality by exploiting some properties of the human visual system. This contribution describes an experiment which evaluates the impact of the default quantisation matrices in HM-8.0 codec. Informal visual tests have been carried out and PSNR and bitrate computed. The obtained results show that the overall impact on the quality of reconstructed videos seems to be minimal and therefore the benefits of having the default scaling lists remain unclear.

# Introduction

The High Efficiency Video Coding (HEVC) standard [1] allows the use of scaling lists in transform domain quantisation to improve the subjective quality of coded contents by taking advantage of some Human Visual System (HVS) properties. Examples of HVS properties are spatial and temporal masking and different sensitivity to the distortion introduced at different spatial frequencies. The values for these scaling lists are transmitted to the decoder as specified in the HEVC syntax. Alternatively, some default values are provided with the standard specification and may be used for actual compression. The default values were proposed in [2] and adopted at the 7th JCT-VC meeting. Document [2] uses a model of the HVS which quantifies the human eye sensitivity to the distortion introduced at different spatial frequencies [3]. This model uses a nonlinear relationship obtained by experimental data collected by a study described in [4]. The scaling list values derived using [3] are characterised by having lower values associated to the low frequencies and higher values in high frequencies. These numbers reflect the well known HVS property such that the quantisation noise introduced at high frequencies is less noticeable than the one introduced at low frequencies. An example of 4×4 matrix for the luma component of intra coded blocks derived with the model in [3] is show in Figure 1.

| 12 | 12 | 13 | 16 |
| --- | --- | --- | --- |
| 12 | 13 | 15 | 19 |
| 13 | 15 | 22 | 31 |
| 16 | 19 | 31 | 53 |

Figure 1: Example of 4×4 scaling list obtained with the model in [3].

In the HEVC standard, the values for the default quantisation matrices are derived by the model in [3] and then they are divided by 12, multiplied by 16 and rounded to the nearest integer. Therefore, by applying these operations to the matrix in Figure 1, the default scaling list for 4×4 intra coded blocks is obtained as in Figure 2. As may be noted from Figure 2, the matrix values will be greater or equal to 16, whereas a value equal to 16 means that the quantisation step is not varied for the corresponding coefficient. Although the values are numerically different from what proposed in [2] and [3], the trend whereby higher values are associated to high spatial frequencies is still preserved in the HEVC default quantisation matrices.

This proposal presents an experiment whereby the rate-distortion performance of the HM-8.0 codec using the default scaling lists is compared with the one of the HM-8.0 codec using flat scaling lists. Moreover, for some relevant coded contents, an informal viewing has been organised to assess whether the default scaling list may improve the perceived video quality against the HM-8.0 anchors. The experiment description together with the obtained results is provided in the following section.

| 16 | 16 | 17 | 21 |
| --- | --- | --- | --- |
| 16 | 17 | 20 | 25 |
| 17 | 20 | 30 | 41 |
| 21 | 25 | 41 | 70 |

Figure 2: Default scaling list for the luma component of 4×4 intra coded blocks in the HEVC standard.

# Experiment description and results

## Coding conditions and evaluation methodology

The JCT-VC test videos have been encoded with the HM-8.0 software according to the main configurations for intra, random access and low delay [5] and enabling the usage of default scaling lists (i.e. setting ScalingList : 1 in the encoder configuration file). The used quantisation parameters are the ones for common test conditions (i.e. 22, 27, 32, and 37) plus some additional values to test different quality points. These additional values are: 29, 30, 31, 33, 34, 35 and 36.

The performance of HM-8.0 with default scaling lists is compared with HM-8.0 codec both in objective and subjective terms. For objective comparison, the BD-rates are computed for the tested configurations.

For the subjective comparison the quality of the decoded videos coded with defaults scaling lists is compared with the HM-8.0 anchors. To this end, some video contents have been selected from the JCT-VC test set for an informal subjective viewing organised at the BBC R&D premises. The contents selected for viewing are those which present a high percentage of texture details, complex motion or sharp edges. The selected contents are listed in the following:

- Kimono

- Cactus

- Park-Scene

- China-Speed

- Basketball-Drive

- Kristen-and-Sara

- Slide-Editing

Five video coding experts took part to the experiment and inspected the decoded videos in a viewing room with controlled lighting and with two full HD plasma displays where the anchor and the test contents are played out simultaneously to allow side-by-side comparison. The subjects played the contents several times and paused onto several frames the used video player to make a deeper side-by-side comparison.

## Results and discussion

The BD-rates for the tested configurations are computed and listed in Table 1. As may be noted the usage of the default scaling list slightly decreases the overall objective performance of the HEVC codec. This result is expected as the scaling lists are designed to improve the subjective quality or to keep the same subjective quality but reducing the coding rate. Moreover, for almost all video classes in the three tested configurations there is a bitrate re-distribution among the three components (Y, U and V). In fact, it may be noted that nearly for each BD-rate loss in the luma component, there is a corresponding BD-rate improvement in the chroma components.

Table 1: BD-rates for the tested configurations.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.3% | 0.1% | 0.8% |
| Class B | 1.4% | -1.3% | -1.5% |
| Class C | 0.9% | 0.1% | 0.0% |
| Class D | 0.9% | -0.1% | -0.1% |
| Class E | 0.4% | -0.9% | -1.0% |
| Overall | 0.8% | -0.4% | -0.4% |
|  | 0.8% | -0.4% | -0.4% |
| Class F | 0.5% | -0.1% | -0.1% |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class A | 0.2% | 1.1% | 2.4% |
| Class B | 0.5% | -1.3% | -1.5% |
| Class C | 0.8% | -0.2% | -0.2% |
| Class D | 0.9% | -0.6% | -0.7% |
| Class E |  |  |  |
| Overall | 0.6% | -0.3% | -0.1% |
|  | 0.6% | -0.3% | -0.1% |
| Class F | 0.5% | -0.1% | 0.0% |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.4% | -1.8% | -2.3% |
| Class C | 0.9% | -0.8% | -1.1% |
| Class D | 0.7% | -1.0% | -0.8% |
| Class E | 0.4% | -1.2% | -0.7% |
| Overall | 0.6% | -1.2% | -1.3% |
|  | 0.6% | -1.2% | -1.4% |
| Class F | 0.6% | 0.1% | -0.3% |

From the informal subjective viewing the observers were not able to spot any difference in quality (improvement or deterioration) with respect to the HM-8.0 anchors when the contents are played out. Furthermore, Table 2 reports the rate and PSNR difference for some contents which provided significant bitrate reductions. These contents have been also viewed and again the quality of both anchors and videos coded with quantisation matrices was the same. It should be noted that at that level of bitrate/QP the same or even larger reduction can be achieved by increasing the QP by one unit.

Table 2: High rates and PSNR differences with respect to the anchors for some viewed contents.

| Configuration | Name | QP | Bitrate [kbps] | PSNR-Y [dB] | Bitrate [kbps] | PSNR-Y [dB] | ∆ Bitrate [%] | ∆ PSNR-Y [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ldP_main | Bqterrace | 22 | 64350.55 | 38.2756 | 57750.43 | 37.8765 | 10.26 | 0.40 |
| ra_main | Bqterrace | 22 | 39633.58 | 37.4211 | 36306.79 | 37.1687 | 8.39 | 0.25 |
| ld_main | Cactus | 22 | 19894.91 | 38.6817 | 19031.27 | 38.6229 | 4.34 | 0.06 |
| ld_main | Basketballdrive | 22 | 19783.01 | 39.4126 | 18720.98 | 39.3206 | 5.37 | 0.09 |
| ld_main | Bqterrace | 22 | 52632.9 | 38.2296 | 46497.65 | 37.8209 | 11.66 | 0.41 |
| i_main | Nebutafestival | 22 | 402497.3 | 42.221 | 382476 | 41.8901 | 4.97 | 0.33 |
| i_main | Bqterrace | 22 | 180164.4 | 42.3661 | 173975.6 | 41.1219 | 3.44 | 1.24 |

# Conclusions

This proposal described an experiment designed to assess the objective and subjective quality improvement brought by the HM codec when the default scaling lists are used. It should be noted that this experiment was also motivated by some concerns expressed by the JCT-VC experts at the 10th JCT-VC meeting in Stockholm (see [6] page 128) on having default quantisation matrices since each implementer/broadcaster would use its own defined values.

From the informal subjective viewing results, it seems that the default scaling lists bring a minimal contribution to the overall video quality. Some significant bitrate reductions (up to 11%) have been noted but for very high bitrates which provides a visually lossless reconstructed quality with respect to the original. At these bitrates, higher reductions can be achieved by simply increasing the QP by one unit. The limited performance brought by the default scaling lists can be attributed to two main factors. First, the model used to derive the default values has been designed for JPEG image compression ([3]) whereby the frequency coefficients refer to image pixels rather than to the residues as in the HEVC standard. Second, some of the model parameters were tuned for 512×512 image pixels which are quite different from the resolution used in the context of the HEVC standardisation.

# Changes to the current working draft text

The changes are highlighted in yellow in the document attached to this contribution.

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 8”, JCTVC-J1003, 10th meeting, Stockholm, SE, July 2012.

- M. Haque, A. Tabatabai, Y. Morigami, “HVS Model based Default Quantization Matrices”, JCTVC-G880, 7th meeting, Geneva, CH, November 2011.

- L.-W. Chang, C.-Y. Wang and S.-M. Lee, “Designing JPEG quantization tables based on Human Visual System”, Signal Processing: Image Communication, Vol. 16, no. 5, pp. 501-506, January 2001.

- J. L. Mannos and D. J. Sakrison, “The effect of a visual fidelity criterion in the encoding of images”, IEEE Transactions on Information Theory, Vol. IT-20  No. 4, pp. 525-536, July 1974.

- F. Bossen, “HM 8 Common Test Conditions and Software Reference Configurations”, JCTVC-J1100, 10th meeting, Stockholm, SE, July 2012.

- G. J. Sullivan and J. R. Ohm, “Meeting report of the 10th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Stockholm, SE, 11-20 July 2012”, JCTVC-J_notes_d9, 10th meeting, Stockholm, SE, July 2012.

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) does not have any current or pending patent rights relating to the technology described in this contribution.