| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0316 |
| --- | --- |

| Title: | Semantics of no_output_of_prior_pics_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez, Anil Kumar Katti, Hsiang-Yeh Hwang 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This document proposes enhancements to the semantics of no_output_of_prior_pics_flag to facilitate graceful transitions in consecutive coded video sequences with different picture formats.

Apology

The authors sincerely apologize for the tardiness of this contribution and would greatly appreciate its consideration.

# Introduction

In broadcast applications, upon tuning and acquiring a video signal, the physical output clock of the video port driving the display device is typically set and maintained constant. Even if the picture format in the bitstream changes, such as during a commercial,  the resolution of the sequence of pictures presented and displayed device does not change.  Broadcast applications consequently permit different picture resolutions but compensate by properly signaling values for sample_aspect_ratio and/or sample_scale_factor (see JCTVC-K0263) so that the resulting picture resolution is constant.

The following enhancements to the semantics of the no_output_of_prior_pics_flag are proposed to minimize the number of output blank pictures when transitioning between consecutive video sequences (CVSes) with different picture formats, and to prevent CPB issues, such as underflow/overflow, that may not manifest instantly but at a future time.  The following changes are proposed to the specification draft:

7.4.5 Slice header semantics

7.4.5.1 General slice header semantics

no_output_of_prior_pics_flag specifies how the previously-decoded pictures in the decoded picture buffer are treated after decoding of an IDR or a BLA picture. See Annex C. When the IDR or BLA picture is the first picture in the bitstream, the value of no_output_of_prior_pics_flag has no effect on the decoding process. When the IDR or BLA picture is not the first picture in the bitstream, perform the following steps.

- If a RAP picture does not correspond to the first picture of a second of two consecutive CVSes

- Then, the value of the no_output_of_prior_pics_flag associated with that RAP picture is ignored.

- Else, Derive horiz_spatial_span and vert_spatial_span as follows:

horiz_spatial_span = output_pic_width_in_luma_samples * sample_aspect_ratio * sample_scale_factor

vert_spatial_span = output_pic_heigth_in_luma_samples * sample_scale_factor

where:

output_pic_width_in_luma_samples and output_pic_heigth_in_luma_samples specify the samples of the pictures in the coded video sequence that are output from the decoding process, according to the values of pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, and pic_crop_bottom_offset; and

sample_scale_factor is a sample magnification factor described in JCT-VC-J0287 and JCT-VC-K0263.

- If the value of horiz_spatial_span or vert_spatial_span derived from the active sequence parameter set is different from the value of horiz_spatial_span or vert_spatial_span derived from the sequence parameter set active for the preceding picture, respectively.

- Then, the value of no_output_of_prior_pics_flag shall be inferred equal to 1.

- Else, If the value of fixed_pic_rate_flag is not equal to 1 in either the active sequence parameter set or the sequence parameter set active for the preceding picture

- Then, the value of no_output_of_prior_pics_flag shall be inferred equal to 1.

- Else, If the value of sps_max_dec_pic_buffering [sps_max_temporal_layers_minus1 ] derived from the active sequence parameter set is different from the value of sps_max_dec_pic_buffering [sps_max_temporal_layers_minus1 ] derived from the sequence parameter set active for the preceding picture.

- Then, the value of no_output_of_prior_pics_flag shall not be inferred.

- If the no_output_of_prior_pics_flag is not inferred

- Then, its value is according to the value provided in the slice header.

If the value of no_output_of_prior_pics_flag is equal to 1 or inferred equal to 1

- Then, the first picture from the active sequence parameter set shall be output after the latest output time of any DPB picture corresponding to the sequence parameter set active for the preceding picture that was not output.

Else,

- Pictures in the DPB that were not output yet are output according to their respective output times.

8.1 General decoding process

– The value of no_output_of_prior_pics_flag is derived according to 7.4.5.1 set to 1.

C.2.1 Removal of pictures from the DPB

If the current picture is an IDR or a BLA picture, the following applies:

- When the IDR or BLA picture is not the first picture decoded and the value of horiz_spatial_span or vert_spatial_span or sps_max_dec_pic_buffering[ i ] for any possible value of i derived from the active sequence parameter set is different from the value of horiz_spatial_span or vert_spatial_span  or sps_max_dec_pic_buffering[ i ] derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle picture or DPB size changes more gracefully than the HRD in regard to changes in pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering[ i ].

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1:

all picture storage buffers in the DPB except the one corresponding to the current IDR or a BLA picture are emptied without output of the pictures they contain, and DPB fullness is set to 1 if the IDR or a BLA picture is output at a later time otherwise 0, and the first picture from the active sequence parameter set shall be output after the latest output time of any DPB picture corresponding to the sequence parameter set active for the preceding picture that was not output.

Option 2

Alternatively, the proposed semantics for no_output_of_prior_pics_flag may be applicable when fixed_pic_rate_flag as indicated in the SPS (equal to 1) is not allowed to change across CVSes.

7.4.5 Slice header semantics

7.4.5.1 General slice header semantics

no_output_of_prior_pics_flag specifies how the previously-decoded pictures in the decoded picture buffer are treated after decoding of an IDR or a BLA picture. See Annex C. When the IDR or BLA picture is the first picture in the bitstream, the value of no_output_of_prior_pics_flag has no effect on the decoding process. When the IDR or BLA picture is not the first picture in the bitstream, and fixed_pic_rate_flag equal to 1, perform the following steps.

- If a RAP picture does not correspond to the first picture of a second of two consecutive CVSes,

- Then, the value of the no_output_of_prior_pics_flag associated with that RAP picture is ignored.

- Else, Derive horiz_spatial_span and vert_spatial_span as follows:

horiz_spatial_span = output_ pic_width_in_luma_samples * sample_aspect_ratio * sample_scale_factor

vert_spatial_span = output_ pic_heigth_in_luma_samples * sample_scale_factor

where:

output_ pic_width_in_luma_samples and output_ pic_heigth_in_luma_samples specify the samples of the pictures in the coded video sequence that are output from the decoding process, according to the values of pic_crop_left_offset, pic_crop_right_offset, pic_crop_top_offset, and pic_crop_bottom_offset; and

sample_scale_factor is a sample magnification factor described in JCT-VC-J0287 and JCT-VC-K0263.

- If the value of horiz_spatial_span or vert_spatial_span derived from the active sequence parameter set is different from the value of horiz_spatial_span or vert_spatial_span derived from the sequence parameter set active for the preceding picture, respectively:

- Then, the value of no_output_of_prior_pics_flag shall be inferred equal to 1.

- Else, If the value of sps_max_dec_pic_buffering [sps_max_temporal_layers_minus1 ] derived from the active sequence parameter set is different from the value of sps_max_dec_pic_buffering [sps_max_temporal_layers_minus1 ] derived from the sequence parameter set active for the preceding picture.

- Then, the value of no_output_of_prior_pics_flag shall not be inferred.

- If the no_output_of_prior_pics_flag is not inferred

- Then, its value is according to the value provided in the slice header.

If the value of no_output_of_prior_pics_flag is equal to 1 or inferred equal to 1

- Then, the first picture from the active sequence parameter set shall be output after the latest output time of any DPB picture corresponding to the sequence parameter set active for the preceding picture that was not output.

Else,

- Pictures in the DPB that were not output yet are output according to their respective output times.

8.1 General decoding process

– The value of no_output_of_prior_pics_flag is derived according to 7.4.5.1 set to 1.

C.2.1 Removal of pictures from the DPB

If the current picture is an IDR or a BLA picture, the following applies:

- When the IDR or BLA picture is not the first picture decoded and the value of horiz_spatial_span or vert_spatial_span or sps_max_dec_pic_buffering[ i ] for any possible value of i derived from the active sequence parameter set is different from the value of horiz_spatial_span or vert_spatial_span  or sps_max_dec_pic_buffering[ i ] derived from the sequence parameter set that was active for the preceding picture, respectively, no_output_of_prior_pics_flag is inferred to be equal to 1 by the HRD, regardless of the actual value of no_output_of_prior_pics_flag.

NOTE – Decoder implementations should try to handle picture or DPB size changes more gracefully than the HRD in regard to changes in pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering[ i ].

- When no_output_of_prior_pics_flag is equal to 1 or is inferred to be equal to 1:

all picture storage buffers in the DPB except the one corresponding to the current IDR or a BLA picture are emptied without output of the pictures they contain, and DPB fullness is set to 1 if the IDR or a BLA picture is output at a later time otherwise 0, and the first picture from the active sequence parameter set shall be output after the latest output time of any DPB picture corresponding to the sequence parameter set active for the preceding picture that was not output.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).