| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0317 |
| --- | --- |

| Title: | Cross-check of GOP structures for interlaced video (JCTVC_K0153) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | David Baylon 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2285 david.baylon@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This document reports the cross-verification results for JCTVC-K0153 [1] on GOP structures for interlaced video.  The simulation results confirm those given by the proponent.  The results using the proposed field GOP structure reportedly show improvement for field sequence coding compared to frame sequence coding, although there is still a significant gap in chroma coding performance for some sequences.  It would be interesting to compare performance to AVC with adaptive frame-field coding modes.  Also, it is not known what impact the proposed field GOP structures will have on 3:2 pulldown material.

# Introduction

JCTVC-K0153 [1] compares the performance of HM8.0 for coding of interlaced video as a frame sequence and a field sequence.  It proposes a modification in the GOP structure for field sequence coding compared to the current common test conditions that can improve the performance of field sequence coding for sequences that may favor frame sequence coding.

# Software and simulations

Source code for [1] was provided by the proponent based on HM8.0.  Since the proposed modification to the GOP structure doubles the number of reference pictures relative to the common test condition (CTC), HM8.0 was modified by the proponent to allocate more memory to support the longer list.

For the cross-check simulations, the code was compiled in a Windows environment and the simulations were performed on the nine 480i MPEG-2 test sequences.  Simulations for six configurations (RA-Main, LB-Main, LP-Main, RA-HE10, LB-HE10, and LP-HE10) were performed for the following test conditions: (1) frame sequence coding using a CTC frame GOP structure, (2) field sequence coding using a CTC field GOP structure, and (3) proposed field sequence coding using a proposed field GOP structure.  The simulation results reportedly confirm the results given by the proponent.

# Comments

The cross-check results using the proposed field GOP structure reportedly show improvement for field sequence coding of sequences that favor frame sequence coding without impacting much the performance for sequences that favor field sequence coding.  However, there is still a significant gap in chroma coding performance for some sequences.  It would also be interesting to compare performance of the proposed HM field GOP structure to JM (AVC) with adaptive frame-field coding modes.

For both RA and low delay (LB/LP) configurations, the reference picture list is doubled with the proposed field GOP structure over the CTC field GOP structure.  For the RA case, the proposed field GOP structure allows prediction using opposite parity fields, although the traditional hierarchical structure is somewhat altered.  For the low delay case, the CTC field GOP structure allows cross field prediction but the proposed field GOP structure additionally allows same field prediction using a temporally near reference field.  It is not known what impact the proposed field GOP structures will have on 3:2 pulldown material.

# Reference

[1]	C. Auyeung, “GOP structures for Interlaced Video”, JCTVC-K0153, 11th JCT-VC Meeting, Shanghai, China, Oct 2012.