| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0318 |
| --- | --- |

| Title: | Cross-check of coding interlaced video using sequence-adaptive field-frame (JCTVC-K0250) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | David Baylon 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2285 david.baylon@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This document reports the cross-verification results for JCTVC-K0250 [1] on coding interlaced video using sequence-adaptive field-frame (SAFF).  Simulations were performed using RA-Main on six 1080i test sequences.  The simulation results based on a modified version of HM8.0 reportedly confirm the results given by the proponent.  However, the simulation results based on JM18.4 reportedly do not match the results given by the proponent.  The source of this mismatch appears to be caused by differences in bitstreams generated from 32-bit and 64-bit builds of JM18.4, as was confirmed by the proponent.  In the comparisons of HM vs. JM, this mismatch reportedly causes a difference of up to 0.1% in BD-rate on average relative to that reported by the proponent.  Although SAFF was compared to JM18.4 using MBAFF, it would be interesting to also see comparisons with MBAFF+PictureAFF.

# Introduction

JCTVC-K0250 [1] investigates the performance of sequence-adaptive field-frame (SAFF) coding of a sequence where each GOP is encoded in both field and frame modes and the best mode is chosen for the GOP in a post-processing step using BD-rates.  The performance of HM SAFF is compared to JM18.4 using HM-like settings with MBAFF.

# Software and simulations

Source code for [1] based on HM-8.0-dev revision 2764 was provided by the proponent.  Some modifications were added by the proponent to support SEI field_indication messages, field-frame buffer conversion, and efficient interpolation computation.  Modifications to this version of the HM and to JM18.4 were also added to compute field-based PSNRs.

For the cross-check simulations, the code was compiled in a Windows environment and the simulations were performed on six 1080i test sequences.  Simulations for RA-Main were performed with the modified HM for GOPs in field mode and frame mode, and simulations using HM-like settings were also performed with the modified JM using MBAFF.  The simulation results based on the modified HM reportedly confirm the results given by the proponent.  However, the simulation results based on the modified JM reportedly do not match the results given by the proponent.  The source of this mismatch appears to be caused by differences in bitstreams generated from 32-bit and 64-bit builds in the original (unmodified) JM18.4, as was confirmed by the proponent.  The cross-check simulation results were generated from a 32-bit build whereas the proponent simulation results were generated from a 64-bit Linux build.  In the comparisons of HM vs. JM, this mismatch reportedly causes a difference of up to 0.1% in BD-rate on average relative to that reported by the proponent.

The cross-check simulations results are shown in the tables below.  For the Mad1 sequence, it was confirmed with the proponent that the frames selected were 580 to 879.  Also, for the last table below (HEVC Adaptive vs JM 18.4 MBAFF), the summary results in the Excel spreadsheet appears to have had a cut/paste error that is corrected in the cross-check spreadsheet and below.

|  | HEVC Frame Coding vs HEVC Field Coding |  |  |
| --- | --- | --- | --- |
|  | BD-rate (piecewise cubic) |  |  |
| Clip | Y | U | V |
| Cheer1 | 76.8% | 62.7% | 74.6% |
| Cheer2 | 98.7% | 115.9% | 117.3% |
| Cheer3 | 88.5% | 98.2% | 94.4% |
| Cheer4 | 14.2% | -5.1% | -5.9% |
| Mad1 | 48.9% | 33.9% | 15.6% |
| Music1 | -12.7% | -49.1% | -51.0% |
| Average | 52.4% | 42.8% | 40.8% |

|  | HEVC Field Coding vs HEVC Adaptive |  |  |
| --- | --- | --- | --- |
|  | BD-rate (piecewise cubic) |  |  |
| Clip | Y | U | V |
| Cheer1 | 0.0% | 0.0% | 0.0% |
| Cheer2 | 0.0% | 0.0% | 0.0% |
| Cheer3 | 0.0% | 0.0% | 0.0% |
| Cheer4 | 0.1% | 5.7% | 6.0% |
| Mad1 | 4.6% | 8.0% | 20.6% |
| Music1 | 15.7% | 92.2% | 99.8% |
| Average | 3.4% | 17.6% | 21.0% |

|  | HEVC Frame Coding vs HEVC Adaptive |  |  |
| --- | --- | --- | --- |
|  | BD-rate (piecewise cubic) |  |  |
| Clip | Y | U | V |
| Cheer1 | 76.8% | 62.7% | 74.6% |
| Cheer2 | 98.7% | 115.9% | 117.3% |
| Cheer3 | 88.5% | 98.2% | 94.4% |
| Cheer4 | 14.2% | 0.2% | -0.5% |
| Mad1 | 54.6% | 43.2% | 37.6% |
| Music1 | 1.0% | -2.4% | -2.4% |
| Average | 55.6% | 53.0% | 53.5% |

|  | HEVC Field Coding vs JM 18.4 MBAFF |  |  |
| --- | --- | --- | --- |
|  | BD-rate (piecewise cubic) |  |  |
| Clip | Y | U | V |
| Cheer1 | -40.3% | -45.9% | -43.2% |
| Cheer2 | -41.7% | -47.5% | -43.3% |
| Cheer3 | -42.5% | -41.9% | -41.8% |
| Cheer4 | -25.3% | -17.5% | -20.2% |
| Mad1 | -20.6% | -31.5% | -25.6% |
| Music1 | -5.2% | -2.3% | 9.2% |
| Average | -29.3% | -31.1% | -27.5% |

|  | HEVC Frame Coding vs JM 18.4 MBAFF |  |  |
| --- | --- | --- | --- |
|  | BD-rate (piecewise cubic) |  |  |
| Clip | Y | U | V |
| Cheers1 | 5.5% | -11.1% | -0.4% |
| Cheers2 | 15.7% | 13.9% | 23.8% |
| Cheers3 | 8.9% | 16.5% | 14.3% |
| Cheers4 | -14.1% | -20.6% | -23.7% |
| Mad1 | 20.5% | -3.1% | -7.8% |
| Music1 | -15.5% | -48.0% | -43.5% |
| Average | 3.5% | -8.7% | -6.2% |

|  | HEVC Adaptive vs JM 18.4 MBAFF |  |  |
| --- | --- | --- | --- |
|  | BD-rate (piecewise cubic) |  |  |
| Clip | Y | U | V |
| Cheers1 | -40.3% | -45.9% | -43.2% |
| Cheers2 | -41.7% | -47.5% | -43.3% |
| Cheers3 | -42.5% | -41.9% | -41.8% |
| Cheers4 | -25.3% | -21.6% | -24.3% |
| Mad1 | -23.3% | -34.8% | -35.4% |
| Music1 | -16.5% | -47.0% | -42.4% |
| Average | -31.6% | -39.8% | -38.4% |

# Comments

In comparisons of HM performance with JM, the JM simulations in [1] used only MBAFF.  It would be interesting to do comparisons with MBAFF+PictureAFF, as performance with the JM may improve for sequences that favor field sequence coding, narrowing the gap between HM field mode coding and JM coding.

As for encoder complexity, the modified HM computation complexity is at least double that of the reference HM since each GOP is coded in both field and frame modes.  In addition, the GOP mode decision uses a BD-rate comparison that is based on four RD points per mode.

# Reference

[1]	D. Hoang and J. Xin, “Coding interlaced video using sequence-adaptive field-frame”, JCTVC-K0250, 11th JCT-VC Meeting, Shanghai, China, Oct 2012.