| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0319 |
| --- | --- |

| Title: | Cross-check of deblocking of large block artifacts in JCTVC-K0138 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for deblocking of larger block artifacts proposed in JCTVC-K0138. Source codes provided by the proponents were compiled and tested for CE1 Type 1 test sequences. It was confirmed that the cross-check results exactly match. Informal visual testing was done for some of sequences and comments on visual quality are also provided in this contribution.

# Overview of JCTVC-K0138

JCTVC-K0138 introduces two independent methods to reduce blocking artifacts at large transform size. First, it restricts intra prediction boundary filtering to block sizes below 32x32 as proposed in [1]. Second, it increases bS to 3 for max TU boundary and increases clipping bounds for weak filtering. This proposal has advantage in that it does not introduce any change in filtering operation itself.

# Quality Evaluation

Objective quality was evaluated for CE1 Type 1 sequences suggested in [2]. It was confirmed that the cross-check results exactly match to the results provided by the proponent. Its visual qualities were compared with HM-8.0 anchor and CE1 Test 8 [2] for DucksTakeOff, Riverbed and WestWindEasy sequences at LB QP 37. When compared to HM-8.0 anchor, it was observed that JCTVC-K0138 improves visual qualities for these sequences. When compared to CE1 Test 8, it was better for DucksTakeOff, but slightly worse for Riverbed. JCTVC-K0138 and CE1 Test 8 resulted in similar quality for WestWindEasy sequence. Formal visual quality test is recommended in the meeting.

# References

- A. Norkin, “AHG6: Reduction of block artifacts in HEVC for large blocks”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J0286, July 2012.

- A. Norkin, T. Suzuki, M. Budagavi and G. V. Auwera, “Description of Core Experiment 1: Deblocking filter”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J1101, July 2012.