| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0320 |
| --- | --- |

| Title: | Cross-check of simple improvement of deblocking filter in JCTVC-K0150 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for simple improvement of deblocking filter in JCTVC-K0150. Source codes provided by the proponents were compiled and tested for common test condition and CE1 Type 1 test sequences. It was confirmed that the cross-check results exactly match to the results provided by the proponent. Informal visual testing was done for some of sequences and comments on visual quality are also provided in this contribution.

# Overview of JCTVC-K0150

Two methods to remove blocking artifacts at large block boundaries are proposed in JCTVC-K0150. In the first method (Test 1), it is proposed to increase beta and tc values instead of directly using table-derived beta and tc values. Beta and tc value are modified by (Bs + 1) * beta and (Bs + 1) * tc, respectively. In the second method, on top of Test 1, it is proposed to restrict intra prediction boundary filtering to block sizes below 32x32 based on [1].

# Quality Evaluation

Objective qualities of both Test 1 and Test 2 were evaluated for common test condition and CE1 Type 1 sequences [2] for maximum TU size of 32x32. It was confirmed that the cross-check results exactly match to the results provided by the proponent. Visual qualities of Test 2 were compared with HM-8.0 anchor and CE1 Test 8 [2] for DucksTakeOff, Riverbed and WestWindEasy sequences at LB QP 37. When compared to HM-8.0 anchor, it was observed that JCTVC-K0150 improves visual qualities for all of three sequences. When compared to CE1 Test 8, it was better for DucksTakeOff, but very slightly worse for Riverbed. JCTVC-K0150 and CE1 Test 8 resulted in similar quality for WestWindEasy sequence.

The proposed method in JCTVC-K0150 has advantage that the required change is minor and shows visual quality improvements for the sequences having severe blocking artifacts at large TU boundaries. Formal visual test is recommend in the meeting.

# References

- A. Norkin, “AHG6: Reduction of block artifacts in HEVC for large blocks”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J0286, July 2012.

- A. Norkin, T. Suzuki, M. Budagavi and G. V. Auwera, “Description of Core Experiment 1: Deblocking filter”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J1101, July 2012.