| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0322 |
| --- | --- |

| Title: | AHG7: Comments on 422 and 444 coding tools and software |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Informative |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Geert Van Der Auwera Joel Sole Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

AHG7 recently released software that adds capability to code extended chroma formats (422 and 444) to HM8.0. The software contains a number of coding options that can be configured using #defines. This document presents some observations regarding the software and choice of default coding tools. It recommends that by default, the 422 and 444 chroma processing should be aligned with 420 chroma processing. It is remarked that more time and better documentation is needed to understand the AHG7 software and extension of coding tools to 422. Finally, a bugfix for planar mode intra prediction calculation for 422 chroma formats is reported. For the intra_he10 configuration, a BD-rate of -0.6%, -0.3% and -0.4% is reported for Y, Cb and Cr components, respectively, with respect to the AHG7 anchor.

# Introduction

AHG7 recently released software that adds capability to code extended chroma formats (422 and 444) to HM8.0. This software is primarily based on JCTVC J0191. Several options from JCTVC J0357 were also added to the software. The software contains 44 (!) different coding options related to 422 and 444 formats. To exhaustively evaluate these options and their combinations and make decisions on a case-by-case basis would be very difficult. This document puts forward a default criterion to make these decisions. We also present some observations regarding the AHG7 software. Finally, a bugfix for planar mode intra prediction calculation for 422 chroma format is reported.

# Common criterion for evaluation

The macros contained in the AHG7 software can be classified into the following categories: Block structure, intra prediction and filtering, interpolation filters for motion, transform and quantization, contexts for CABAC, scans, and encoder search. Generally, the choice is whether chroma processing for the 422 and 444 formats is aligned with 420 chroma processing or luma processing. Some examples of this choice can be found in contexts for CABAC, whether the 420 TU structure is used (no 4×4 chroma blocks), chroma block sizes for which horizontal/vertical scans are applied, etc.

In many hardware implementations, the processing path for chroma is separate from luma. Also, a decoder capable of decoding extended chroma formats would typically be required to decode 420 formats as well. Thus, the processing path for chroma would need to handle 420, 422 and 444 formats. If the 422 and 444 chroma processing differs substantially from 420 chroma processing, additional area and/or logic is needed to implement the different processing paths. Hence our recommendation is that by default, the 422 and 444 chroma processing should be aligned with 420 chroma processing. Any deviation from 420 chroma processing should be justified by sufficient BD-rate improvement. We would like to note that many of the default values for the macros, as recommended by AHG7, already satisfy this criterion.

As an example, this criterion can be applied to coefficient coding. Since in HM8.0, all the chroma blocks are processed as 4×4 subblocks, this should also be the default choice for 422 and 444 chroma formats. The context derivations from HM8.0 can be directly applied to the extended chroma formats. The only decision that needs to be made is the order in which the subblocks are scanned.

# AHG7 software and changes related to 422 chroma format

The AHG7 software modifies HM8.0 extensively. There are over 2000 changes between HM8.0 and AHG7 software. Although some of these changes are trivial, others are more substantial. For example all the TUs and RQT structure are handled through a new class. There are several reasonable ways in which the 420 chroma processing methods may be extended to 422. Some of these choices are clearly defined through macros, but others are not under macros. Without examining all the changes in the AHG7 software compared to HM8.0, it is nearly impossible to figure out the choices made for extending 420 chroma processing to 422. We believe that these choices should be documented more precisely.

Sometimes, the choice of one macro may affect other processing. As an example, if the 420 TU structure is maintained for 444 chroma components, deblocking is carried out at 16×16 blocksize, whereas if chroma blocks are allowed to split to 4×4, deblocking is carried out at 8×8 blocksize. The existing documentation seems to indicate that there is no change in deblocking process. Moreover, the deblocking filter modifications are not under a macro.

Thus we feel that more time is needed to study the software before any decisions regarding tools are made. Furthermore it is desirable to clearly document how 420 coding tools have been extended to 422.

## Bug in planar prediction in AHG7 software

The macro ECF__CHROMA_422_INTRA_PLANAR_SINGLE_STAGE_CALCULATION controls the behaviour of the planar intra prediction mode. If the macro is set to 0 (default), the following code calculates planar prediction:

for(Int k=0;k<width;k++)

{

bottomRow[k]  = bottomLeft - topRow[k];

topRow[k]     <<= shift1Dhor;

}

for(Int k=0;k<height;k++)

{

rightColumn[k]  = topRight - leftColumn[k];

leftColumn[k]   <<= shift1Dver;

}

// Generate prediction signal

for (Int y=0;y<height;y++)

{

Int horPred = leftColumn[y] + offset2D;

for (Int x=0;x<width;x++)

{

horPred += rightColumn[y];

topRow[x] += bottomRow[x];

// if width!=height, then width must be half height (i.e. 4:2:2)

rpDst[y*dstStride+x] = ( horPred + (topRow[x]<< ((width!=height) ? 1 : 0)) ) >> (shift1Dver+1);

}

}

In the above code, the weights for horizontal and vertical predictions are incorrect when width and height are not equal (422 chroma components). For example, for a block of size 4×8, for calculating horizontal prediction, the left column is weighted by (7−x) instead of (3−y), where x is the column index. The code should instead be changed to the following. The changes are highlighted in yellow.

for(Int k=0;k<width;k++)

{

bottomRow[k]  = bottomLeft - topRow[k];

topRow[k]     <<= shift1Dver;

}

for(Int k=0;k<height;k++)

{

rightColumn[k]  = topRight - leftColumn[k];

leftColumn[k]   <<= shift1Dhor;

}

// Generate prediction signal

for (Int y=0;y<height;y++)

{

Int horPred = leftColumn[y] + offset2D;

for (Int x=0;x<width;x++)

{

horPred += rightColumn[y];

topRow[x] += bottomRow[x];

// if width!=height, then width must be half height (i.e. 4:2:2)

Int vertPred;

if (width != height)

{

vertPred = (topRow[x] + 1) >> 1;

}

else

{

vertPred = topRow[x];

}

rpDst[y*dstStride+x] = ( vertPred + horPred ) >> (shift1Dhor+1);

}

}

Table 1 compares the BD rate results with the above change to the AHG7 anchor for the intra_he10 configuration. For the intra_he10 configuration, a BD-rate improvement of -0.6%, -0.3% and -0.4% is observed for Y, Cb and Cr components, respectively. Since fixing the bug affects the AHG7 anchor substantially, we feel that it is necessary to rerun tests for all the proposed 422 tools (especially intra).

Table 1: BD-rate results for intra_he10 configuration with the bugfix for planar intra prediction (422)

# Conclusions

We recommend that by default, the 422 and 444 chroma processing should be aligned with 420 chroma processing. Any deviation from 420 chroma processing should be justified by sufficient BD-rate improvement. More time and better documentation is needed to understand the AHG7 software and extension of coding tools to 422. Finally, a bugfix for planar mode intra prediction calculation for 422 chroma formats is reported. For the intra_he10 configuration, a BD-rate of -0.6%, -0.3% and -0.4% is observed for Y, Cb and Cr components, respectively, compared to the AHG7 anchor. We recommend rerunning the tests for all the proposed 422 tools (especially intra), since the results may get affected by the bugfix.

# Patent rights declaration(s)

Qualcomm Inc. does not have any current or pending patent rights relating to the technology described in this contribution.