| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0323 |
| --- | --- |

| Title: | Cross-check of context modeling unification for large transforms significance coefficient flags (JCTVC-K0270) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the results of the cross-check performed by Qualcomm of the two methods proposed by Mediatek in JCTVC-K0270. The first method reduces by 6 the number of contexts for the coefficients significance. The second method reduces the number of contexts by 12. The cross-check performance results for the common test conditions settings in HM8.0 match those provided by the proponent. The implemented algorithm agrees with what is described in Mediatek’s proposal.

# Description

Software implementing JCTVC-K0270 on top HM8.0 basis was provided by Mediatek.

For the 8x8 transform blocks (TB), the set of contexts that HM8.0 applies depends on whether the scan is diagonal or horizontal/vertical.  Also, the set of contexts is different depending on the TB being 8x8 or 16x16/32x32 or on whether the sub-block is the top-left sub-block or not.

Method 1 in JCTVC-K0270 combines the contexts for the diagonal and vertical/horizontal scans for the 8x8 TB, thus reducing the number by 6.

Method 2 further combines the contexts for the high-frequency sub-blocks (all except the top-left sub-block) for the 8x8, 16x16 and 32x32 TB, thus reducing the number of contexts by 12 in total. The contexts for the top-left sub-blocks are kept separated for the 8x8 TB with respect to the 16x16/32x32 TBs.

Code changes for both methods are the following:

- ContextTables.h: Modification of the context tables to handle the updated number of contexts. Initialization values are kept the same.

- TComTrQuant.cpp: Modification of the logic for the assignment of the contexts sets for the significance flags of the 8x8, 16x16 and 32x32 TB.

# Results

Coding results using common conditions with HM8.0 anchor have been obtained and match those of the proponent. Results for both methods are provided in the accompanying excel files.