| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0330_r7 |
| --- | --- |

| Title: | Introduction of temporal_id_type Single layer: conformance and temporal id semantics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This contribution introduces highestTid_info_only_flag in the SPS for applications that do not require downward temporal scalability but require temporal_id values greater than zero and only one HRD RD RDconformance point for the complete coded video sequence.

Apology

The authors sincerely apologize for the tardiness of this contribution and would greatly appreciate its consideration.

# Proposed syntax and semantics

#### Proposed changes are highlighted in yellow.

Accepted changes per JCTVC-K0120 are highlighted in aqua.

Comments on potential specification errors (in HRD) are highlighted in green.

#### 7.3.2.2:   Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| sps_reserved_zero_bit | u(1) |
| profile_and_level( 1, sps_max_sub_layers_minus1) |  |
| seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pic_cropping_flag | u(1) |
| if( pic_cropping_flag ) { |  |
| pic_crop_left_offset | ue(v) |
| pic_crop_right_offset | ue(v) |
| pic_crop_top_offset | ue(v) |
| pic_crop_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| highestTid_info_only_flag | u(1) |
| for( i = (highestTid_info_only_flag ? sps_max_sub_layers_minus1 : 0) ; i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| restricted_ref_pic_lists_flag | u(1) |
| if( restricted_ref_pic_lists_flag ) |  |
| lists_modification_present_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| if( pcm_enabled_flag ) { |  |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_coding_block_size | ue(v) |
| } |  |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| if( pcm_enabled_flag ) |  |
| pcm_loop_filter_disable_flag | u(1) |
| sps_temporal_id_nesting_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### 7.3.3 Profile, tier and level syntax

| profile_tier_level( ProfilePresentFlag, MaxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( ProfilePresentFlag ) { |  |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( i = 0; i < 32; i++ ) |  |
| general_profile_compatibility_flag[ i ] | u(1) |
| general_reserved_zero_16bits | u(16) |
| } |  |
| general_level_idc | u(8) |
| If( ! highestTid_info_only_flag) { |  |
| for( i = 0; i < MaxNumSubLayersMinus1; i++ ) { |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| if( ProfilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_reserved_zero_16bits[ i ] | u(16) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |
| } |  |

#### 7.4.2.2 Sequence parameter set RBSP semantics

sps_temporal_id_nesting_flag, when sps_max_sub_layers_minus1 is greater than 0, specifies whether inter prediction is additionally restricted for coded video sequences referring to the sequence parameter set. When vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag shall be equal to 1. When sps_max_sub_layers_minus1 is equal to 0, sps_temporal_id_nesting_flag has no meaning, and when sps_max_sub_layers_minus1 is equal to 0 and vps_temporal_id_nesting_flag is equal to 1, sps_temporal_id_nesting_flag may have any value. When highestTid_info_only_flag is equal to 1, sps_temporal_id_nesting_flag is inferred to be equal to 0.

NOTE 1 – The syntax element sps_temporal_id_nesting_flag is used to indicate that temporal up-switching, i.e. switching from decoding of up to a specific TemporalId tIdN to decoding up to a TemporalId tIdM that is greater than tIdN, is always possible.

highestTid_info_only_flag equal to 0 specifies the presence of sps_max_dec_pic_buffering[ i ], sps_max_num_reorder_pics[ i ],  and sps_max_latency_increase[ i ] for values of i from 0 to sps_max_sub_layers_minus1.  highestTid_info_only_flag equal to 1 specifies their presence only for i = sps_max_sub_layers_minus1.

#### 7.4.1.2	NAL unit header semantics

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit.

The variable TemporalId is specified as

TemporalId = nuh_temporal_id_plus1 − 1		(7-1)

If nal_unit_type is in the range of 7 to 12 (coded slice of a RAP picture), inclusive, TemporalId shall be equal to 0; otherwise, when nal_unit_type is in the range of 3 to 6, inclusive (coded slice of a TSA or STSA picture),TemporalId shall not be equal to 0.

The value of TemporalId shall be the same for all VCL NAL units of an access unit. The value of TemporalId of an access unit is the value of the TemporalId of the VCL NAL units of the access unit.

When highestTid_info_only_flag is equal to 1, the following applies:

- Except for non-decodable leading pictures, any picture with TemporalId k shall be decodable if all pictures with TemporalId ≤  k that are after and including the preceding RAP are decoded.

- NAL units with nal_unit_type equal to VPS_NUT may or may not be present when video_parameter_set_id is equal to 15.

C.1 Bitstream conformance

The HRD parameter sets for bitstream conformance are selected as follows. The variable layerIdList is a list of values for nuh_reserved_zero_6bits present in the bitstream. When highestTid_info_only_flag is equal to 0, the hrd_parameters( ) structure for which the set of values specified by op_layer_id[ opIdx ][ i ] for i in the range of 0 to op_num_layer_id_values_minus1[ opIdx ], inclusive, is identical to layerIdList is used for bitstream conformance. Within that hrd_parameters( ) structure, the hrd_parameters_sub_layer( HighestTid ) structure or structures are used for testing bitstream conformance.

It is a requirement of bitstream conformance that all of the following conditions shall be fulfilled for each of the tests:

- After the decoding process for reference picture set as specified in subclause 8.3.2 has been invoked, the number of decoded pictures in the DPB, including all pictures marked as "used for reference" and all pictures n that have PicOutputFlag equal to 1 and tr( n ) < tr( currPic ) and to,dpb( n ) >= tr( currPic ), where currPic is the current picture, but not including the current picture, with TemporalId lower than or equal to the TemporalId of the current picture, shall be less than or equal to

Min(0, sps_max_dec_pic_buffering[highestTid_info_only_flag ? sps_max_sub_layers_minus1 : TemporalId ] – 1).

Comment:  Min does not appear to make sense  Some reference pictures with higher TemporalId may be retained for later used.

- C.4.2 Removal of pictures from the DPB

Otherwise (the current picture is not an IDR or a BLA picture), picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). When one or more of the following conditions are true, the "bumping" process specified in subclause C.4.2.1 is invoked repeatedly until there is an empty picture storage buffer to store the current decoded picture.

- The number of pictures in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[highestTid_info_only_flag ? sps_max_sub_layers_minus1 : TemporalId]

- The number of pictures in the DPB with TemporalId lower than or equal to the TemporalId of the current picture is equal to sps_max_dec_pic_buffering[highestTid_info_only_flag ? sps_max_sub_layers_minus1 : TemporalId]

C.4.2.1 "Bumping" process

The "bumping" process is invoked in the following cases.

- The current picture is an IDR or a BLA picture and no_output_of_prior_pics_flag is not equal to 1 and is not inferred to be equal to 1, as specified in subclause C.4.2.

- The number of pictures in the DPB that are marked "needed for output" is greater than

sps_max_num_reorder_pics[highestTid_info_only_flag ? sps_max_sub_layers_minus1 : TemporalId], as specified in subclause C.4.2.

- The number of pictures in the DPB with TemporalId lower than or equal to the TemporalId of the current picture is equal to sps_max_dec_pic_buffering[highestTid_info_only_flag ? sps_max_sub_layers_minus1 : TemporalId], as specified in subclause C.4.2.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).