| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0331 |
| --- | --- |

| Title: | HEVC performance evaluation on interlaced video sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Akira Minezawa Shun-ichi Sekiguchi Tokumichi Murakami 5-1-1 Ofuna, Kamakura Kanagawa 247-8501 Japan | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports performance evaluation of HM8.0 on interlaced video sequences by comparing JM18.4. A new GOP structure for field coding is proposed. Simulation results on the second version of the ITE/ARIB interlaced video materials show that the coding performance improvement using the proposed GOP structure for HM8.0 compared with JM18.4 is 20.7% on average.

# Introduction

According to JCTVC-J0466 [1], it was reported that in some cases the coding efficiency of HM6.2 on interlaced video in field sequence in random access configuration compared to JM18.3.

In this contribution, performance evaluation of HM8.0 on interlaced video sequences is provided. Additional GOP structure for HM8.0 for interlaced video coding is also proposed.

# Proposed GOP structure

The GOP structure of the random access condition in the common test condition is shown in Figure 1. As shown in figure 1, this GOP has 8 pictures, which means that this GOP has only 4 frames in field sequences.

Figure 1: GOP structure of random access in CTC

Figure 2 shows the proposed GOP structure which has 8 frames (16 field pictures) for interlaced video sequences.

Figure 2: Proposed GOP structure for interlaced video sequences

The configuration of this GOP structure is provided below:

Frame1:  B    1   1    0.4420   0   2    1   2     -1  -16       0

Frame2:  B    16  1    0.4420   0   2    1   2    -15  -16       0

Frame3:  B    8   2    0.3536   1   2    1   3     -7   -8   8       0

Frame4:  B    9   2    0.3536   1   2    1   4     -1   -8  -9   7   0

Frame5:  B    4   3    0.3536   2   2    1   5     -3   -4   4   5  12   0

Frame6:  B    5   3    0.3536   2   2    1   6     -1   -4  -5   3   4  11   0

Frame7:  B    2   4    0.6800   3   2    1   7     -1   -2   2   3   6   7  14   0

Frame8:  B    3   4    0.6800   3   2    0   7     -1   -2   1   2   5   6  13   0

Frame9:  B    6   4    0.6800   3   2    1   6     -1   -2  -5   2   3  10   0

Frame10: B    7   4    0.6800   3   2    0   6     -1   -2  -6   1   2   9   0

Frame11: B    12  3    0.3536   2   2    1   4     -3   -4 -11   4   0

Frame12: B    13  3    0.3536   2   2    1   5     -1   -4  -5 -12   3   0

Frame13: B    10  4    0.6800   3   2    1   6     -1   -2  -9   2   3   6   0

Frame14: B    11  4    0.6800   3   2    0   6     -1   -2 -10   1   2   5   0

Frame15: B    14  4    0.6800   3   2    1   4     -1   -2 -13   2   0

Frame16: B    15  4    0.6800   3   2    0   4     -1   -2 -14   1   0

# Simulation Results

Performance evaluation of HM8.0 for on interlaced video sequences has been performed. We used 37 sequences in series C in the second version of ITE/ARIB (The Institute of Image Information and Television Engineers / Association of Radio Industries and Businesses) test materials. Summary of image characteristics is provided in JCTVC-J0152[2].

To compare JM18.4, we used the modified version of the configuration used in JCTVC-J0466. For HM8.0, we also turned off the RDOQ for this comparison. Random access condition (with RDOQ=OFF) and the proposed GOP structure on top of it was tested.

Tested settings are as follows:

[settings 1: JM field]

JM18.4 with JCTVC-J0466 configurations (RDOQ=off, PocMemoryManagement=on)

Interlaced video in frame sequence is input to the encoder.

[settings 2 : HM RA-main frame]

HM8.0 with RA-main configurations (RDOQ=off)

Interlaced video in frame sequence is input to the encoder.

[settings 3 : HM RA-main field]

HM8.0 with RA-main configurations (RDOQ=off)

Interlaced video in field sequence is input to the encoder.

[settings 4 : HM proposed field]

HM8.0 with RA-main configurations (RDOQ=off) using the proposed GOP structure

Interlaced video in field sequence is input to the encoder.

We used the same QP and Intra period as used in CTC. For the fair comparison, the original test images are extended to 1920x1088 resolutions from 1920x1080 by padding the bottom lines. For field coding using HM8.0, 1920x544 resolutions in field format is used. Test results are shown in Table 1.

Table 1. Test results on interlaced video sources

We also compared the coding performance of JM18.4 and HM-8.0 on progressive source materials. We used 11 sequences in series B in the second version of ITE/ARIB test materials. These sequences in series B were captured in almost the same time and angle by progressive camera in series C. We used RA-main configurations for HM-8.0 and HM-like/ encoder_JM_RA_B_HE.cfg for JM18.4. The test results on progressive sources are shown in Table 2. For comparison, the test results on interlaced sources for the corresponding sequences are also shown.

Table 2. Test results on progressive and interlaced video sources

According to the results, the performance difference between progressive coding and interlaced coding is less than 7%. If appropriate GOP structure can be selected for each sequence, the performance gap becomes smaller.

In addition to the simulations above, an informal subjective viewing was also done by experts in Mitsubishi. Some sequences of 5, 4 and 3 Mbps encoded with settings 4 were compared with sequences of 10, 8 and 6Mbps encoded with settings 1. In some sequences, the results of settings 1 looks slightly better than the results of setting 4, however, most of the sequences gives almost the same visual quality.

# Conclusions

According to the objective test results and the informal subjective viewing by experts, it was observed that the performance improvement of HEVC comparing with AVC on interlaced video sources is close to the case on progressive video sources. We recommend keeping the SEI message based interlaced coding in DIS as it is for the first version of HEVC specification.

# References

- A. Luthra and D. Baylon, “Performance of HEVC for interlaced video”, JCTVC-J0466, 10th Meeting: Stockholm, SE, 11–20 July 2012.

- T.Suzuki, P. Silcock, K. Sharman, N. Saunders, J. Gamei, and A. Tabatabai, “Proposals on HEVC professional extensions”, JCTVC-J0152, Meeting: Stockholm, SE, 11–20 July 2012.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).