| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0337 |
| --- | --- |

| Title: | Cross-check of reduction of worst-case memory bandwidth for SCU size larger than 8x8 (JCTVC-K0144) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-2429 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution presents the results of the cross-check performed by Qualcomm of the two methods proposed by Canon in JCTVC-K0144. The first method removes the partition PART_NxN for all the inter PUs. The second method restricts the PART_2NxN and PART_Nx2N to be uni-prediction for the CU of sizes equal to SCU. Both methods reduce the worst-case memory bandwith when the SCU is set to a size different from 8x8. The cross-check performance results match those provided by the proponent. The implemented algorithm agrees with what is described in Canon’s proposal.

# Description

Software implementing JCTVC-K0144 on top of the HM8.0 was provided by Canon.

In HEVC, smallest motion compensation units are of size 4x8 and 8x4, thereby prohibiting 4x4 inter prediction, and are constrained to make use of only the first reference picture list (i.e., no bi-prediction for 4x8 and 8x4 units). The proposal extends these concepts to any SCU size, not only 8x8. The first method removes inter PART_NxN in all instances. The second method restricts PART_2NxN and PART_Nx2N to be uni-prediction for the SCUs and is applied on the top of the first one.

Code changes for these methods are small. The modifications are the following:

- TComDataCU.cpp: Modification of the condition of bi-prediction.

- TEncSbac.cpp, TDecSbac.cpp: Removal of the partition signaling for PART_NxN. Extension of the restriction of bi-prediction to the new cases.

- TEncCu.cpp: Remove the computation of the RD cost of the inter partition PART_NxN.

# Results

Coding results are obtained modifying the common conditions to have the SCU size equals to 16x16 by setting MaxPartitionDepth to 3 on both, the anchor and the proposal. Also, CroppingMode is set to 1 to handle class B sequences, which have a size non-multiple of 16. Results match those of the proponent and are provided in the accompanying excel files.