| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0338 WG11 Number: m27085 |
| --- | --- |

| Title: | AHG7: Cross-verification of JCTVC-K0171, Transforms for extended chroma formats |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of JCTVC-K0171, “AHG7: Transforms for extended chroma formats.” Simulations were performed using the HM-8.0-dev-ahg7 software as a reference and subsequent software modifications provided by Canon Information Systems Research Australia / Canon Inc. for testing. The BD-Rate simulation results performed for this cross-verification match those provided by the authors of JCTVC-K0171. Encode time ratios match as well. Decode time ratios for this cross-verification were close to 100%, whereas JCTVC-K0171 report ratios of 112%. The software and two options for changes to Working Draft text were examined as well.

# Description of cross-verification and results

## Simulation results

When the luma transform size is 32x32, the current HM-8.0-dev-ahg7 software [1] applies a 16x32 transform for chroma in 4:2:2 video and a 32x32 transform for chroma in 4:4:4 video. The JCTVC-K0171 [2] document proposes splitting these 16x32 and 32x32 chroma TUs into multiple 16x16 transforms, thus eliminating the 32-point transform for chroma. For this cross-verification, software provided by the authors of JCTVC-K0171 [2] was tested against a reference using the HM-8.0-dev-ahg7 software, for the All-Intra HE10 common conditions [3]. Experiments were performed on the set of 4:2:2 and 4:4:4 sequences selected by AHG7 [4]. BD-Rate results for this cross-verification are shown in Table 1 for 4:2:2 sequences and in Table 2 for 4:4:4 sequences.

Table 1. BD-Rate change for 4:2:2 sequences

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Kimono | 2.7% | 4.1% | 4.5% |
| Parkscene | 0.4% | 1.2% | 1.2% |
| BirdsInCage | 3.8% | 5.5% | 10.5% |
| DucksAndLegs | 0.6% | 1.3% | 1.7% |
| Traffic | 0.5% | 1.1% | 1.3% |
| CrowdRun | 0.3% | 0.4% | 0.5% |
| OldTownCross | 0.4% | 1.5% | 1.9% |
| Seeking | 0.6% | 1.0% | 1.1% |
| Overall | 1.2% | 2.0% | 2.8% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2. BD-Rate change for 4:4:4 sequences

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Kimono | 2.9% | 3.8% | 4.7% |
| Parkscene | 0.4% | 1.1% | 1.1% |
| BirdsInCage | 3.6% | 4.7% | 8.7% |
| DucksAndLegs | 0.6% | 1.3% | 1.3% |
| Traffic | 0.5% | 0.9% | 1.0% |
| CrowdRun | 0.3% | 0.8% | 0.6% |
| OldTownCross | 0.6% | 2.5% | 1.5% |
| Seeking | 0.6% | 1.2% | 1.2% |
| Overall | 1.2% | 2.0% | 2.5% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 98% |  |  |

## Examination of software

A quick examination of the software was performed. Basically, if the height of a chroma transform is greater than 16 then a new loop is called that traverses the 16x16 sub-transforms for performing or signaling the transform. Generation of the cbf flags for chroma are similarly affected. Note that this examination and the above simulations used the following parameters defined in TypeDef.h:

#define INFERRED_CHROMA_SPLIT            1

#define NO_CHROMA_32_CBF                 1

#define INFERRED_CHROMA_SPLIT_INTER      0

## Examination of changes to Working Draft text

### Option 1: Inferred chroma split using loop construct

The changes affect the transform_tree() and transform_unit() syntax. In transform_tree(), because the proposed method splits a TU into multiple sub-transforms, a cbf flag is signaled for each of smaller transforms, for both Cb and Cr. Changes to the transform_unit() syntax allow residual_coding() to be performed on each of the sub-transforms. Changes to semantics introduce horizontal and vertical counters to allow the sub-transforms to be traversed.

### Option 2: transform_tree() invoked for each of the split transforms

For the syntax changes, a flag is added as a parameter passed to transform_tree(). If the flag is true, then transform_tree() is invoked recursively if the transform is to be split. This flag is also passed to tansform_unit( ), which affects whether the qp_delta-related values are signaled. Changes to the semantics are similar to those in Option 1, except a few parameters from Option are not needed.

# Summary and conclusions

The BD-Rate results for this cross-verification match the corresponding set of results in JCTVC-K0171. The encod time ratios are comparable. Decode time ratios for this cross-verification were close to 100%, whereas those in JCTVC-K0171 were 112%. A quick examination of the software indicates that if the theight of a chroma transform is greater than 16, then it is split into 16x16 sub-transforms, depending upon the chroma format. The changes to syntax and semantics, for which two methods were described, allow these sub-transforms to be traversed, with signaling of corresponding cbf or qp_delta-related flags.

# References

- Online: https://hevc.kw.bbc.co.uk/svn/jctvc-hm/branches/HM-8.0-dev-ahg7

- C. Rosewarne and M. Maeda, “AHG7: Transforms for extended chroma formats,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K0171, Shanghai, CN, October 2012

- F. Bossen, “HM 8 Common Test Conditions and Software Reference Configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J1100, Stockholm, SE, July 2012.

- Online: ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/