| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0339 |
| --- | --- |

| Title: | BoG report on general high-level syntax topics |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BoG Report |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela (BoG chair) Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | BoG on general high-level syntax topics |  |  |

_____________________________

# Abstract

This document contains the notes of the breakout group of general high-level syntax topics.

# High-level syntax cleanups (9)

JCTVC-K0120 AHG9: High-level syntax clean-ups [Y.-K. Wang (Qualcomm)]

It was proposed to have a restriction to say that the NAL unit header of a dependent slice shall be identical to that of its parent slice. It was found that there is already a more generic constraint that the nal_unit_type must the same for all VCL NAL units of a coded picture. The BoG recommended taking no action on this issue.

The BoG recommended

- renumbering NAL units in the range of 1 to 20 as 0 to 19

- categorizing the NAL unit types 20..23 as reserved VCL NAL units

- renumbering NAL units in the range of 25 to 32 as 24 to 31

In other words, the BoG recommended the following nal_unit_type table:

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0,1 | TRAIL_R, TRAIL_N | Coded slice of a non-TSA, non-STSA trailing picture slice_layer_rbsp( ) | VCL |
| 2, 3 | TSA_R, TSA_N | Coded slice of a TSA picture slice_layer_rbsp( ) | VCL |
| 4, 5 | STSA_R, STSA_N | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 6, 7, 8 | BLA_W_LP BLA_W_DLP BLA_N_LP | Coded slice of a BLA picture slice_layer_rbsp( ) | VCL |
| 9, 10 | IDR_W_DLP IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 11 | CRA_NUT | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 12 | DLP_NUT | Coded slice of a DLP picture slice_layer_rbsp( ) | VCL |
| 13 | TFD_NUT | Coded slice of a TFD picture slice_layer_rbsp( ) | VCL |
| 14..23 | RSV_VCL14.. RSV_VCL23 | Reserved | VCL |
| 24 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 25 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 26 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 27 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 28 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 29 | EOB_NUT | End of bitsteam end_of_bitstream_rbsp( ) | non-VCL |
| 30 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 31 | SEI_NUT | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 32..46 | RSV_NVCL32.. RSV_NVCL46 | Reserved | non-VCL |
| 47..63 | UNSPEC47.. UNSPEC63 | Unspecified | non-VCL |

There was a comment that, if possible, instead of referring to specific nal_unit_type values in the specification text, a table should map nal_unit_type values to variables and those variables should be referred to in the specification text.

The BoG recommended adopting the proposal to move sps_temporal_id_nesting_flag to the beginning to replace sps_reserved_zero_bit, which was there only for the purpose of byte alignment for the profile information in the SPS.

Mixed opinions were expressed on the proposal to either remove lists_modification_present_flag or make the persence of restricted_ref_pic_lists_flag independent of restricted_ref_pic_lists_flag.

The BoG recommended taking no action on the proposal to change tiles_fixed_structure_flag to 2-bit tiles_fixed_structure_idc, such that whether all PPSs referred in the coded video sequence have tiles_enabled_flag equal to 0 is signalled in addition to whether they have the same tiles structure.

The BoG recommended revisiting the proposal to add a flag to VUI to indicate whether POC value differences are proportional to presentation time differences, and if the flag is equal to 1, add the number of ticks minus 1 that corresponds to any POC difference equal to 1 – when specification text is available. There was interest towards this proposal.

The BoG recommended no constraints on pic_output_flag equal to 0 for all pictures in a coded video sequence or a bitstream.

The BoG recommended taking no action on the proposal to include a flag in the PPS to control the presence of num_ref_idx_active_override_flag in the slice header.

It was proposed to include pic_temporal_mv_enabled_flag in the slice header also for I slices, under the same condition as for B or P slices. This proposal is similar to an aspect in JCTVC-K0251. The BoG recommended revisiting this aspect in the JCT-VC plenary/tracks.

The proposal to remove the text for entropy slices had been already resolved by notes of JCTVC-K0288.

The BoG recommended changing the semantics of end of sequence RBSP semantics as follows:

When the next subsequent NAL unit in decoding order is not an end of bitstream NAL unit, tThe end of sequence RBSP specifies that the next subsequent access unit in the bitstream in decoding order (if any) is an IDR or BLA access unit. The syntax content of the SODB and RBSP for the end of sequence RBSP are empty.

One aspect of the proposal is to remove the SPS ID from the buffering period SEI message, and specify VPS and SPS activation through the parameter sets active SEI message

It was commented that the buffering period SEI cannot be parsed without either resolving the active SPS or an explicit reference to sps_id.

It was commented that there may also be other SEI messages that could implicitly refer to active VPS and active SPS rather than an explicit reference to vps_id and sps_id.

The BoG recommended adopting this aspect of the proposal with the following modifications:

- Keep seq_parameter_set_id in the buffering period SEI message.

- Require num_sps_ids_minus1 to be equal to 0 in HEVC v1 bitstreams.

One aspect of the proposal is to change the definition picture count as follows:

picture order count: A variable that is associated with each picture that indicates the position of the associated picture in output order relative to the output order positions of the another pictures in the same coded video sequence when both the associated picture and the other picture have PicOutputFlag equal to 1.

The BoG recommended leaving this aspect for editors’ discretion.

One aspect of the proposal is to change a constraint as follows:

- Any picture that has PicOutputFlag equal to 1 that precedes a RAP CRA or BLA picture in decoding order shall precede any DLP picture associated with the RAP CRA or BLA picture in output order.

The BoG recommended leaving this aspect for editors’ discretion.

The BoG recommended adding the following restriction as one of the RPS restrictions in subclause 8.3.2:

- There shall no entry in RefPicSetLtCurr or RefPicSetLtFoll for which the difference between the picture order count value of the current picture and the picture order count value of the entry is equal to or greater than 224.

One aspect of the proposal is to align the maximum value SPS ID and PPS ID in the semantics when compared to those in the current Main profile definition. To be more specific, this aspect of the proposal proposes that unless there is a clear conclusion that the values 31 and 255 for SPS and PPS ID values are too expensive to support by envisioned HEVC decoders to be put into the market, the restrictions in the profile definition should be removed. If indeed it is too expensive, the profile restrictions should also be removed, but in this case the values in the semantics should be changed to be smaller.

The BoG recommended revisiting this aspect in the JCT-VC plenary/tracks.

The BoG recommended removing the following paragraph from the end of subclause 7.4.1.4.2 (Order of access units and association to coded video sequences):

It is a requirement of bitstream conformance that, when an SEI NAL unit contains data that pertain to more than one access unit (for example, when the SEI NAL unit has a coded video sequence as its scope), it shall be contained in the first access unit to which it applies. [Ed. (GJS): Is this needed?]

One aspect of the proposal is to add the support of suffix SEI NAL units using their own NAL unit type, and specify the decoded picture hash SEI message as a suffix SEI message.

A question was asked whether the same or different numbering space for SEI messages would be used for prefix SEI NAL units and suffix NAL units.

The BoG recommended revisiting this aspect after specification text is available. There was support towards this aspect.

One aspect of the proposal is to include an extension flag to the end of all types of SEI messages, such that they all become extensible, with the cost of at most one bit (in 7/8 of all cases, zero bit cost).

The BoG recommended revisiting this aspect after specification text is available. There was support towards this aspect.

One aspect of the proposal relates to allowing SEI messages to be repeated (while currently some messages are constrained to appear only at the beginning of an access unit). Furthermore, a related aspect is that for SEI messages that apply to multiple AUs, which are basically sequence-level, it should be restricted that they can only be present in AUs with TemporalId equal to 0, similarly as requiring TemporalId to be 0 for all RAP pictures. The proposal includes a table including the following type of information for each SEI message:

| SEI message (payloadType) | Application scope | May be present | Multi-instance restriction |
| --- | --- | --- | --- |
| Buffering period (0) | The buffering period containing the SEI message | In any AU with TemporalId equal to 0 | All instances within a buffering period shall be identical |
| Picture timing (1) | The AU containing the SEI message | In any AU with any value of TemporalId | All instances within an AU shall be identical |
| … |  |  |  |

A comment was expressed that the column “May be present” might be interpreted to mean the only AU where the SEI message (and repeated SEI messages) may be included or the only AU where the SEI message may be included but repeated SEI messages may be present in any AU. This aspect could be clarified.

The BoG suggested experts to check the table in the proposal carefully and coordinate any issues with the proponent.

It was commented that the buffering period SEI message must not be repeated within more than one access unit (that also contains the first occurrence of the buffering period SEI message).

Having such a table in the specification text was generally felt desirable.

The BoG recommended revisiting this aspect when specification text is available.

JCTVC-K0136 AHG9: High-level syntax cleanups [C.-Y. Chen, C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0183 proposed a similar modification as in items 1.1 of the proposal and was resolved as semantics restriction. Therefore the BoG recommended no action.

Item 1.2 has been resolved by notes of JCTVC-K0120.

Item 1.3 of the proposal is the same as an aspect JCTVC-K0125 and an aspect in JCTVC-K0254.

The BoG recommended adopting item 1.3:

| profile_tier_level( ProfilePresentFlag, MaxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i < MaxNumSubLayersMinus1; i++ ) { |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| if( ProfilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_present_flag[ i ] |  |
| if( sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_reserved_zero_16bits[ i ] | u(16) |
| } |  |
| … |  |

JCTVC-J0184 proposed a similar modification as in items 1.4 of the proposal, but was resolved as semantics restriction.

The BoG recommended adopting item 1.4 to change the short_term_ref_pic_set syntax and semantics as follows:

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( idx != 0 ) |  |
| inter_ref_pic_set_prediction_flag | u(1) |
| … |  |

inter_ref_pic_set_prediction_flag equal to 1 specifies that the reference picture set of the current picture is predicted using another reference picture set in the active sequence parameter set. When idx is equal to 0 not present, the value of inter_ref_pic_set_prediction_flag shall be is inferred to be equal to 0.

The BoG recommended revisiting items 1.5 and 1.6 after replacing the new clipping operations with semantic restrictions. [To be reviewed next in the BoG.]

The BoG recommended no action on items 1.7, 1.8, 1.9, and 1.10.

JCTVC-K0174 On PPS [K Sato (Sony)]

JCTVC-K0213 AHG9: On Tickets 637, 763, and, 777 [K. Chono (NEC)]

JCTVC-K0217 Proposed Changes on Coding Tree Unit Syntax and Sequence parameter set RBSP syntax [Xue Fang (Motorola Mobility)]

JCTVC-K0251 Modification of slice_temporal_mvp_enable_flag [Yue Yu, Jian Lou, Limin Wang (Motorola Mobility)]

JCTVC-K0254 AHG9: Parsing profile and level information of temporal sub-layers [T. Tsukuba, T. Yamamoto (Sharp)]

JCTVC-K0256 Cleanup of Slice Header [Yue Yu, Jian Lou, Limin Wang(Motorola Mobility)]

JCTVC-K0257 Modification of Enabling Weighted Prediction [Yue Yu, Jian Lou, Limin Wang (Motorola Mobility)]

JCTVC-K0316 Semantics of no_output_of_prior_pics_flag [Arturo Rodriguez, Anil Kumar Katti, Hsiang-Yeh Hwang (??)] [late]

# VUI and SEI messages (17)

## Video usability information (VUI) (2)

JCTVC-K0234 VUI Extension [M. Haque, K. Sato (Sony)]

JCTVC-K0263 Sample scale factor in VUI [Arturo Rodriguez (Cisco)]

## Frame packing arrangement (FPA) SEI message and related (4)

JCTVC-K0105 FPA SEI message modification [O. Nakagami (Sony), T. Suzuki (Sony), G. Ballocca (Sisvel Technology), V. Giovara (Sisvel Technology), P. Sunna (RAI), M. Arena (RAI)]

JCTVC-K0116 2D compatible frame packing stereo 3D video [Xiaofeng Yang, Peiyu Yue, Yuanyuan Zhang (??)]

JCTVC-K0119 AHG9: Indication of frame-packed or interlaced video [Y.-K. Wang (Qualcomm)]

JCTVC-K0240 Frame packing arrangement SEI for 4:4:4 content in 4:2:0 bitstreams [Y. Wu, S. Kanumuri, S. Sadhwani, L. Zhu, S. Sankuratri, G. J. Sullivan, B. A. Kumar (Microsoft)]

This contribution proposes a method to extend the use of the frame packing arrangement SEI message to represent 4:4:4 content in nominally 4:2:0 bitstreams. With the proposed method, it is reported that one constituent frame (e.g. in a top-bottom packing or alternating-frame coding scheme) can be decoded compatibly as an ordinary 4:2:0 image, or can be supplemented with the data from another constituent frame to form a complete 4:4:4 image representation. It is proposed to include support for the additional scheme into the frame packing arrangement SEI message in both AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10) and HEVC, to facilitate deployment of systems using this method. Since 4:2:0 is the most widely supported format in products, it is asserted that having an effective way of conveying 4:4:4 content through such decoders can provide the substantial benefit of enabling widespread near-term deployment of 4:4:4 capabilities (especially for screen content coding). The proposed method operates by packing the samples of a 4:4:4 frame into two 4:2:0 frames and encoding the two 4:2:0 frames as the constituent frames of a frame packing arrangement. The content_interpretation_type is extended to signal this packing arrangement. The proposed scheme is asserted to be of high practical value for applications involving screen content. Relative to native 4:4:4 encoding, the proposed scheme can provide the advantage of compatibility with the ordinary 4:2:0 decoding process that is expected to be more widely supported in decoding products.

A question was asked which frame packing method would be used with the proposal. It was intended that the proposal would be always used with frame packing arrangement SEI message and hence any frame packing method can be used.

A question was asked how the scheme performs compared to an actual 4:4:4 codec. There was no answer to this question.

Some more results as well as test sequences were requested.

A question was asked whether different filters were tried on the decoding end (to see if that has an impact on subjective quality). The answer was that different filters were not tried (but the chroma position information was used).

A question was asked how to use the cropping rectangle with this proposal. It was answered that the issue is the same as with stereoscopic video (i.e., the same solution should be taken in both).

A comment was expressed that an alternative design would be to define the chroma enhancement as another type of scalability (and otherwise use the coding scheme of frame alternating frame packing part of the proposal).

The BoG recommended a follow-up contribution with more results and test sequences.

## Region of interest (ROI) SEI message and related (4)

JCTVC-K0128 AHG9: Signalling of regions of interest and gradual decoding refresh [Y.-K. Wang (Qualcomm)]

JCTVC-K0218 Tile-based region-of-interest signalling with sub-picture SEI messages [Robert Skupin, Valeri George, Thomas Schierl (HHI)]

JCTVC-K0247 Region of interest (ROI) SEI message [G. Dedeoglu, M. Budagavi (TI)]

JCTVC-K0248 ROI tile sections [Yan Ye, Yuwen He, Yong He (InterDigital Communications), Michael Horowitz (eBrisk Video)]

## Other SEI messages (7)

JCTVC-K0117 Parameter ID change information SEI for bitstream splicing [Xiaofeng Yang, Peiyu Yue, Yuanyuan Zhang (??)]

JCTVC-K0127 AHG9: Signalling of bitstream and elementary stream properties [Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

JCTVC-K0131 Deblocking filter display preference SEI message [Alexis Tourapis, Athanasios Leontaris (Apple)]

JCTVC-K0141 AHG9: Low delay display hint SEI [K. Kazui, J. Koyama, S. Shimada, A. Nakagawa (Fujitsu)]

JCTVC-K0142 AHG9: Modification of SEIs specified in AVC [K. Kazui, J. Koyama, S. Shimada, A. Nakagawa (Fujitsu)]

JCTVC-K0205 TL0 index SEI message [J. Boyce, D. Hong, W. Jang(Vidyo)]

JCTVC-K0262 AHG9: NAL Packet Priority SEI Message [E.-S. Ryu, Y. Ye, Y. He, Y. He (InterDigital Communications)]