| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0341 |
| --- | --- |

| Title: | AHG9: On slice_temporal_mvp_enable_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes the same proposal piece as item 3 in Subsection 1.3 of JCTVC-K0120, with proposed spec text changes. The syntax element slice_temporal_mvp_enable_flag syntax element is proposed to be signalled in the slice header also for I slices, under the same condition as for B or P slices.

# Introduction

The slice_temporal_mvp_enable_flag is signalled for B or P slices, not for I slices, as shown in the slice_header() syntax structure copied below.

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( !dependent_slice_flag ) { |  |
| ... |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| ... |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| ... | se(v) |
| } |  |
| ... |  |
| } |  |

The semantics of slice_temporal_mvp_enable_flag is as follows:

slice_temporal_mvp_enable_flag specifies whether temporal motion vector predictors can be used for inter prediction. If slice_temporal_mvp_enable_flag is equal to 0, temporal motion vector predictors shall not be used in decoding of the current picture. If slice_temporal_mvp_enable_flag is equal to 1, temporal motion vector predictors may be used in decoding of the current picture. When not present, the value of slice_temporal_mvp_enable_flag is inferred to be equal to 0.

When both slice_temporal_mvp_enable_flag and TemporalId are equal to 0, all coded pictures that follow the current picture in decoding order shall not use temporal motion vectors from any picture that precedes the current picture in decoding order.

As can be seen, the flag not just specifies an inter prediction behavior of the current picture itself, but also for following pictures in decoding order. In particular, the value equal to 0 specifies the refreshing or flushing of motion information. Therefore, it also applies to I slices.

Currently, the following behaviors are specified by the syntax and the semantics of slice_temporal_mvp_enable_flag:

- Any intra slice would specify the flushing of motion information of earlier pictures in decoding order.

- If a picture contains both intra slices and inter slices, motion information of earlier pictures in decoding order is flushed, and due to the requirement of the value of slice_temporal_mvp_enable_flag being identical for all slices in a picture, the value is implicitly required to be 0 for all inter slices in the picture.

We believe the above behaviors are not the design intent, which in our understanding is to provide a mechanism to enable the encoder to flexibly control temporal motion vector prediction at any types of pictures, including intra pictures.

Therefore, it is proposed to include slice_temporal_mvp_enable_flag in the slice header also for I slices, under the same condition as for B or P slices.

# Proposal

The proposed syntax change is as follows (no change to the semantics is proposed):

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( !dependent_slice_flag ) { |  |
| ... |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| ... |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| ... | se(v) |
| } |  |
| ... |  |
| } |  |

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.