| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0343 |
| --- | --- |

| Title: | AHG9: On POC and its timing hint |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes the same proposal piece as the following item in Subsection 1.2 of JCTVC-K0120, with proposed spec text changes.

It is proposed to add a flag to VUI to indicate whether POC value differences are proportional to presentation time differences, and if the flag is equal to 1, add the number of ticks minus 1 that corresponds to any POC difference equal to 1.

With this information, decoders or other entities may be able to figure out the true presentation time difference between pictures for purposes like verifying system timing, better error concealment, and so on.

# Proposal

The following spec text changes are proposed, in relative to JCTVC-K0030v4 (additions are highlighted in yellow, and removals are strikethroughs with red fonts).

| vui_parameters( ) { | Descriptor |
| --- | --- |
| ... |  |
| hrd_parameters_present_flag | u(1) |
| if( hrd_parameters_present_flag ) |  |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| poc_proportional_to_timing_flag | u(1) |
| if( poc_proportional_to_timing_flag && timing_info_present_flag ) |  |
| num_ticks_poc_diff_one_minus1 | ue(v) |
| bitstream_restriction_flag | u(1) |
| ... |  |
| } |  |

poc_proportional_to_timing_flag equal to 1 indicates that the picture order count value for each picture in the coded video sequence that is not the first picture in the coded video sequence, in decoding order, is proportional to the output time of the picture relative to the output time of the first picture in the coded video sequence. poc_proportional_to_timing_flag equal to 0 indicates that the picture order count value for each picture in the coded video sequence that is not the first picture in the coded video sequence, in decoding order, may or may not be proportional to the output time of the picture relative to the output time of the first picture in the coded video sequence.

num_ticks_poc_diff_one_minus1 plus 1 specifies the number of clock ticks corresponding to a difference of picture order count values equal to one.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.