| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10-19 Oct., 2012 | Document: JCTVC-K0345r1 |
| --- | --- |

| Title: | Suggested design of initial software model for scalable HEVC extension proposal by Fraunhofer HHI, Vidyo and Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Thomas Wiegand (Fraunhofer HHI) H. Schwarz, C. Bartnik, P. Helle, T. Hinz, A. Khairat, H. Kirchhoffer, H. Laksman, D. Marpe, M. Siekmann, J. Stegemann, K. Sühring JeongHoon Park (Samsung) Ken McCann JaeHyun Kim, Chanyul Kim, Jung-Hye Min, Elena Alshina, Alexander Alshin, Il-Koo Kim, Tammy Lee, Byeongdoo Choi, YinJi Piao, SeungSoo Jeong, Sunil Lee, Yongjin Cho, Jeong Yong Choi, Felix C. A. Fernandes, Zhan Ma Jill Boyce (Vidyo) Danny Hong, Wonkap Jang, Adeel Abbas, Srinath Reddy | Email: | thomas.wiegand@hhi.fraunhofer.de jeonghoon@samsung.com ken@zetacast.com jill@vidyo.com |
| Source: | Fraunhofer HHI, Samsung Electronics Co., Ltd. and Vidyo, Inc. |  |  |

_____________________________

# Abstract

This proposal provides a suggestion for an initial software model for the scalability extension of HEVC. It is based on merging subsets of the software implementations from Fraunhofer HHI, Samsung and Vidyo.

The suggestion for reference software is made considering:

- All test cases have been verified by the source software implementations (HEVC-BL and AVC-BL)

- Only inter-layer coding tools for improved enhancement layer coding efficiency are included

- All suggested tools were proposed in a large number of responses to the CfP

- Multi-loop decoding design is assumed

- The software has high commonality with the regular single-layer HM software, and can be merged and maintained with it

The revision r1 of this document includes experimental results that were missing from the initial version.

# Contents

Abstract	1

Contents	2

1	Introduction	3

2	Algorithm description	3

2.1	Overview	3

2.2	Upsampling of reconstructed BL samples	3

2.2.1	Intra_BL mode	3

2.2.2	Up-sampling filters	4

2.3	Inter-layer motion and partition prediction	4

2.3.1	Inter_BL mode	4

2.3.2	Inter-layer motion vector prediction	5

2.3.3	Inter-layer merge candidates derivation	5

2.4	Difference coding mode	6

2.4.1	Intra difference coding mode	6

2.4.2	Inter difference coding mode	7

3	Syntax and semantics description	7

3.1.1	Coding unit syntax	7

3.1.2	Transform tree syntax	8

3.1.3	Coding unit semantics	10

4	Compression performance discussion	11

4.1	Category 1 (HEVC base layer)	11

4.1.1	Spatial scalability	11

4.1.2	Intra-only spatial scalability	12

4.1.3	SNR scalability	12

4.1.4	Overall	13

4.2	Category 2 (AVC base layer)	13

4.2.1	Spatial scalability	13

5	Complexity analysis	14

5.1	Encoding time and measurement methodology	14

5.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM	14

6	Comparison with original CfP responses from Fraunhofer HHI, Samsung and Vidyo	15

7	Patent rights declaration(s)	15

# Introduction

The suggested reference software is based on software that was verified against all test categories.

 	Category 1 (HEVC base layer) spatial scalability

 	Category 1 (HEVC base layer) intra-only spatial scalability

 	Category 1 (HEVC base layer) SNR scalability

 	Category 2 (AVC base layer) spatial scalability

# Algorithm description

## Overview

The base layer is referred to as BL, and the enhancement layer as EL.

The following inter-layer tools are suggested for scalability support.

- Upsampling of reconstructed BL samples (i.e. Intra_BL mode)

- Inter-layer motion and partition prediction

- Difference coding mode (coding the difference between EL and coded BL)

We also suggest the inclusion of asymmetric motion partitioning in the reference software, since it is part of Main Profile of HEVC.

A simplified block diagram of the proposed scalable extension of HEVC is depicted in Figure 1. Similar concepts as for single layer HEVC are used for coding a spatial or quality enhancement layer. A picture is partitioned into coding units, for which a prediction mode can be chosen. Inter-coded CUs are further partitioned into prediction units (PUs) that are associated with a set of motion parameters. The residual for a CU is coded using a partitioning in transform units (TUs).

In order to improve the coding efficiency relative to simulcast, several inter-layer prediction techniques have been included in the design.

Figure 1: Simplified block diagram of the proposed scalable extension of HEVC

The proposed inter-layer prediction techniques and coding modes are described in the following subsections.

## Upsampling of reconstructed BL samples

### Intra_BL mode

Intra_BL mode has similar functionality to the mode of the same name in SVC, where a block of reconstructed BL pixels are scaled to the EL CU resolution and used as the predictor for the 2Nx2N PU of the CU.  One difference from the SVC mode is that the Intra_BL mode can be used for any CU in the EL even though the corresponding BL block is not coded as intra.  This design is allowed due to double-loop decoding.

### Up-sampling filters

The up-sampling filters are 2-D separable, i.e., 1-D filters operate horizontally and vertically over the entire picture. The filter coefficients used for the phase offsets required for the investigated scalability ratios are listed in the following table:

Table 1. Up-sampling filter coefficients for Luma.

| Phase offset | Filter coefficients |
| --- | --- |
| 0 | No filter |
| 1/3 | [ -1, 4, -11, 52, 26, -8, 3, -1]/64 |
| 1/2 | [-1, 4, -11, 40, 40, -11, 4, -1]/64 (Same as HEVC half-pel interpolation filter) |

Table 2. Up-sampling filter coefficients for Chroma.

| Phase offset | Filter coefficients |
| --- | --- |
| 0 | No filter |
| 1/3 | [ -5, 50, 22,-3 ]/64 |
| 1/2 | [-4, 36, 36, -4]/64 (Same as HEVC half-pel interpolation filter) |

Both for Luma and Chroma up-sampling filters for phase offset of 1/2 are the same as the half-sample motion-compensated interpolation filter specified in HEVC.

The filter coefficients for phase offset 1/3 were generated by using the same algorithm as motion compensation interpolation (DCT-IF, refer to [1]) in the current HEVC specification. Filter length (8 taps for Luma and 4 taps for Chroma) is also synchronized with base layer interpolation motion compensation design. No overflow of 16-bit temporal buffers needed for up-sampling is guaranteed in suggested design.

## Inter-layer motion and partition prediction

For inter-layer motion prediction, three different methods are provided:

- In the Inter_BL mode, all motion data are inferred from the co-located base layer signal. This mode is available for both the standard inter mode (using reconstructed reference picture) and the difference inter coding mode (using difference samples for motion-compensated prediction).

- An additional inter-layer candidate is inserted into the AMVP candidate list.

- An inter-layer candidate is used in the merge candidate list.

### Inter_BL mode

The Inter_BL mode is signalled at a CU level. If a CU is coded in Inter_BL mode, all prediction parameters are inferred from the co-located base layer block.

The prediction parameters are derived based on 8x8 luma blocks.

For each 8x8 luma block, the following applies:

- The top-left luma sample inside the 8x8 block is projected into the base layer coordinate system and rounded to the nearest integer sample location (xB,yB).

- Let cuB be the base layer coding unit that covers the luma sample location (xB,yB) in the base layer.

- Depending on whether cuB represent a CU coded in intra or inter prediction mode, the following applies:

  - If cuB is coded in inter prediction mode, let puB be the base layer PU that covers the luma sample location (xB,yB), let bl_ref_idx0 and bl_ref_idx1 be the reference indices for list 0 and 1 for this PU, and let bl_mv0 and bl_mv1 be the motion vectors for list 0 and 1 for this PU.

The reference indices ref_idx0 and ref_idx1 for the enhancement layer 8x8 block are set equal to bl_ref_idx0 and bl_ref_idx1, respectively.

The base layer motion vectors bl_mv0 and mv_mv1 are scaled according to the resolution ratio and rounding to the next 1/4 luma sample motion vector. The scaled base layer motion vectors are then assigned to the motion vectors of the enhancement layer 8x8 block.

  - Otherwise (the cuB is coded in intra prediction mode), both reference indices of the corresponding 8x8 block in the enhancement layer are set equal to 0 and the components of both motion vectors are set equal to 0. The corresponding 8x8 block is predicted using the co-located upsampled base layer signal.

### Inter-layer motion vector prediction

In single-layer HEVC, the advanced motion vector prediction (AMVP) uses two spatial (A, B) and one temporal (T) motion vector predictor that are used as possible candidates for motion vector prediction. For an enhancement layer, an additional inter-layer motion vector predictor (IL) may be derived extending the candidate list by one item. The co-located base layer prediction block, which covers the central sample position is used to derive the motion data for the possible candidate.

Fig. 1 - Motion vector predictor candidate list construction.

If the identified base layer block contains motion information for the considered reference picture list and reference picture index, the corresponding motion vector is inserted at the first position in the candidate list of the current prediction block in the enhancement layer. The inserted motion vector is scaled according to the resolution ratio. If the candidate is added to the list or if there is no motion data available the list construction and motion compensation is pursued subsequently as in single-layer HEVC.

### Inter-layer merge candidates derivation

At the beginning of the merge list construction the collocated base layer prediction block that covers the central sample position (see Fig. 2) is used to derive a possible set of motion parameters as a candidate to the top of the list. However, the derived set of motion parameters are only inserted to the list if the merge flag of the co-located block in the base layer is equal to 0. Otherwise the set of motion parameters is not considered as a candidate and the list construction is continued in the same manner as in single-layer HEVC. For spatial scalability, the base layer motion vectors are scaled according to the resolution ratio. The total length of the merge candidate list is not extended and remains the same. After the candidate list construction is finished, a re-ordering process may be invoked depending on the collocated base layer prediction block that covers the top left sample position inside the current prediction block. If the merge flag of this block equals to 1 the origin O of the motion vector predictor in the base layer is retrieved. The candidate list of the enhancement layer is now checked if a merge candidate exists which is from the corresponding origin. If a candidate is found it is moved to the top of the list and addressed by the lowest index. The order of the remaining entries in the candidate list is retained and their indices are increased by one.

Fig. 2 - Derivation of merge candidate list.

## Difference coding mode

In difference coding mode, the prediction is not done using reconstructed enhancement layer samples, but using difference samples that represent the difference between reconstructed enhancement layer samples and the (upsampled) base layer signal. The final prediction signal is generated by adding the (upsampled) reconstructed base layer signal for the current block to the difference prediction signal.

The difference coding mode is supported for both intra and inter prediction. The usage of the difference coding mode is signaled on a CU level.

### Intra difference coding mode

In the intra difference coding mode, difference samples for the samples of the neighboring blocks are derived by subtracting the co-located base layer samples from the reconstructed enhancement layer samples. The intra prediction is then done using the difference samples. The final prediction signal for a block is obtained by adding the (upsampled) reconstructed base layer signal to the difference prediction signal.

The same intra prediction modes as in conventional intra prediction are used. The difference mode is signaled at CU level.

The difference intra prediction is illustrated in Fig 3.

Fig. 3 - Illustration of intra difference coding mode.

### Inter difference coding mode

In the inter difference coding mode, motion compensation is not done using reconstructed reference pictures, but using difference reference pictures. A difference reference picture represents the difference of the reconstructed picture and the (upsampled) base layer for the corresponding picture.

The motion vector coding is not modified for the difference mode, but sub-sample positions are interpolated using a bi-linear filter instead of the 8-tap filter for luma and the 4-tap filter for chroma (which are used for standard inter modes).

The difference mode is signaled at CU level.

The difference inter prediction is illustrated in Fig 4.

Fig. 4 - Illustration of inter difference coding mode.

# Syntax and semantics description

### Coding unit syntax

Proposed modifications to the CU syntax are highlighted in the table below.

| coding_unit( x0, y0, log2CbSize, RefLayerId ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else { |  |
| if( RefLayerId != -1 ) { // a dependent layer exists |  |
| intra_bl_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| inter_bl_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( !intra_bl_flag [ x0 ][ y0 ] && !inter_bl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| x2 = x1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| y2 = y1 − ( ( 1 << log2CbSize ) >> 2 ) |  |
| x3 = x1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| y3 = y1 + ( ( 1 << log2CbSize ) >> 2 ) |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y2 , log2CbSize ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x0, y3 , log2CbSize ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x2, y0 , log2CbSize ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x3, y0 , log2CbSize ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0 , log2CbSize ) |  |
| prediction_unit( x1, y0 , log2CbSize ) |  |
| prediction_unit( x0, y1 , log2CbSize ) |  |
| prediction_unit( x1, y1 , log2CbSize ) |  |
| } |  |
| } // !intra_bl_flag[ x0 ][ y0 ] && !inter_bl_flag[ x0 ][ y0 ] |  |
| if( RefLayerId != -1 && !intra_bl_flag[ x0 ][ y0 ] ) |  |
| diff_coding_flag[ x0 ][ y0 ] | ae(v) |
| if( !pcm_flag ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2CbSize, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |

### Transform tree syntax

Proposed modifications to the CU syntax are highlighted in the table below.

| transform_tree( x0, y0, xC, yC, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( trafoDepth = = 0 && IntraSplitFlag = = 0 && (PredMode != MODE_INTRA ¦¦ intra_bl_flag[x0][y0]) && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0] && !diff_coding_flag[x0][y0])) |  |
| no_residual_data_flag | ae(v) |
| if( !no_residual_data_flag ) { |  |
| log2TrafoSize = ( log2TrafoWidth + log2TrafoHeight ) >> 1 |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| interSplitFlag = ( max_transform_hierarchy_depth_inter = =0 && PredMode = = MODE_INTER && PartMode != PART_2Nx2N && trafoDepth = = 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter + InterSplitFlag ) |  |
| xBase = x0 − ( x0 & ( 1 << log2TrafoWidth ) ) |  |
| yBase = y0 − ( y0 & ( 1 << log2TrafoHeight ) ) |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && !interSplitFlag ) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( log2TrafoSize <= Log2MaxTrafoSize ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ) ? 1 : 0 |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cb[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if( !readCbf ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| Else |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && log2TrafoSize < Log2MaxTrafoSize ) |  |
| readCbf = cbf_cr[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_cr[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] |  |
| if( !readCbf ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| Else |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| if( InterTUSplitDirection = = 2 ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 1 ) |  |
| y1 = y0 |  |
| x2 = x0 |  |
| y2 = y0 + ( ( 1 << log2TrafoHeight ) >> 1 ) |  |
| x3 = x1 |  |
| y3 = y2 |  |
| } else { |  |
| x1 = x0 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y1 = y0 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x2 = x1 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y2 = y1 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| x3 = x2 + ( ( 1 << log2TrafoWidth ) >> 2 ) * InterTUSplitDirection |  |
| y3 = y2 + ( ( 1 << log2TrafoHeight) >> 2 ) * ( 1 − InterTUSplitDirection ) |  |
| log2TrafoHeight = log2TrafoHeight + 2 * InterTUSplitDirection – 1 |  |
| log2TrafoWidth = log2TrafoWidth − 2 * InterTUSplitDirection + 1 |  |
| } |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y1, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2, y2, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3, y3, x0, y0, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| readCbf = TRUE |  |
| if( blkIdx = = 3 && PredMode != MODE_INTRA && ( ( log2CbSize <= Log2MaxTrafoSize+1 ) ¦ ¦ ( log2TrafoSize < Log2MaxTrafoSize ) ) |  |
| readCbf = cbf_luma[ xBase ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase + ( 1 << log2TrafoWidth ) ][ yBase ][ trafoDepth ] ¦ ¦ cbf_luma[ xBase ][ yBase + ( 1 << log2TrafoHeight ) ][ trafoDepth ] ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1] ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1] |  |
| if( !readCbf ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] = 1 |  |
| Else |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| transform_unit (x0, y0, xC, yC, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |
| } |  |

### Coding unit semantics

intra_bl_flag[ x0 ][ y0 ] equal to 1 specifies prediction_unit( ) is not present for the current coding unit, and is determined using the reference layer with RefLayerId. Additionally, inter_bl_flag and diff_coding_flag are not present.  intra_bl_flag[ x0 ][ y0 ] equal to 0 specifies prediction_unit( ), inter_bl_flag, and diff_coding_flag may be present for the current coding unit.

inter_bl_flag[ x0 ][ y0 ] equal to 1 specifies prediction_unit( ) is not present for the current coding unit, and is determined using the reference layer with RefLayerId.  inter_bl_flag[ x0 ][ y0 ] equal to 0 specifies prediction_unit( ) may be present for the current coding unit.

diff_coding_flag[ x0 ][ y0 ]  equal to 1 specifies that the current coding unit is coded in difference coding mode.  diff_coding_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in difference coding mode.

# Compression performance discussion

The performance is reported with 4 BD rates of EL-only actual and target bit rates, and EL+BL actual and target bit rates. As the performance anchor, the performance of the non-scalable HEVC encodings is used; single layer for EL-only comparison, and simulcast for EL+BL comparison.

## Category 1 (HEVC base layer)

### Spatial scalability

As specified in the CfP document, 2 down-scale factors of 1.5 and 2 in each spatial direction are tested. We directly followed coding conditions for the evaluation of the category 1 spatial scalability performance evaluation. In Tables 4 and 5, the EL encoding performance of the proposed scalable codec at the actual and the target bit rates is provided with respect to the non-scalable HEVC anchor. In case of 2x scalability, the average gain of 30.5 %, 18.8 %, and 18.7 % were achieved for Y, U, and V components, respectively. With the 1.5x scalability test, the proposed scalable codec achieved 47.9%, 38.9 %, and 38.2 % for Y, U, and V components, respectively.

Table 4. Performance of EL-only actual rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -33,3% | -19,9% | -20,7% |  |  |  |
| Class B | -27,8% | -17,8% | -16,7% | -47,9% | -38,9% | -38,2% |
| Overall | -30,5% | -18,8% | -18,7% | -47,9% | -38,9% | -38,2% |

Table 5. Performance of EL-only target rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -33,4% | -20,0% | -20,8% |  |  |  |
| Class B | -29,8% | -20,0% | -18,9% | -48,7% | -39,8% | -39,1% |
| Overall | -31,6% | -20,0% | -19,8% | -48,7% | -39,8% | -39,1% |

Tables 6 and 7 summarize the performance gain of scalable bitstream (BL+EL) with respect to the simulcast anchor. Average gains of 21.2 %, 11.1 %, 10.9 % for Y, U, and V components are achieved in case of 2x scalability, and 29.2 %, 21.0 % and 20.5 % of Y, U, V gains are observed with 1.5x scalability.

Table 6. Performance of EL+BL actual rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -22,9% | -11,4% | -12,1% |  |  |  |
| Class B | -19,4% | -10,7% | -9,8% | -29,2% | -21,0% | -20,5% |
| Overall | -21,1% | -11,1% | -10,9% | -29,2% | -21,0% | -20,5% |

Table 7. Performance of EL+BL target rate

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| *Class A+ | -23,0% | -11,5% | -12,2% |  |  |  |
| Class B | -20,8% | -12,2% | -11,3% | -29,7% | -21,6% | -21,1% |
| Overall | -21,9% | -11,8% | -11,7% | -29,7% | -21,6% | -21,1% |

### Intra-only spatial scalability

This configuration examines the intra frame compression performance of the proposed scalable video codec with 2 downscaling factors of 1.5 and 2. The coding gains are summarized in Tables 8, 9, 10, and 11, where the first 2 are the EL-only results, and the latter 2 are the results of scalable bitstream (BL+EL). With the 2x scalability EL-only comparison,  34.8%, 32.6%, and 33.0% of average coding gains are reported for Y, U, and V components, respectively. In case of 1.5x scalability EL-only comparison, 54.3%, 52.6%, and 52.8% of Y, U, and V coding gains are observed.

Table 8. Performance of EL-only actual rate

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -38,4% | -36,3% | -37,1% |  |  |  |
| Class B | -31,2% | -29,0% | -28,8% | -54,3% | -52,6% | -52,8% |
| Overall | -34,8% | -32,6% | -33,0% | -54,3% | -52,6% | -52,8% |

Table 9. Performance of EL-only target rate

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -38,4% | -36,3% | -37,1% |  |  |  |
| Class B | -31,4% | -29,3% | -29,1% | -53,4% | -51,7% | -51,9% |
| Overall | -34,9% | -32,8% | -33,1% | -53,4% | -51,7% | -51,9% |

In case of EL+BL performance comparison, the observed average gains for Y, U, and V components are 24.0%, 22.0%, and 22.3% in case of 2x scalability, and 32.1%, 30.7%, and 30.9% for 1.5x scalability.

Table 10. Performance of EL+BL actual rate

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -26,6% | -24,5% | -25,2% |  |  |  |
| Class B | -21,4% | -19,5% | -19,4% | -32,1% | -30,7% | -30,9% |
| Overall | -24,0% | -22,0% | -22,3% | -32,1% | -30,7% | -30,9% |

Table 11. Performance of EL+BL target rate

|  | All Intra HEVC 2x |  |  | All Intra HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -26,6% | -24,5% | -25,2% |  |  |  |
| Class B | -21,6% | -19,7% | -19,6% | -31,6% | -30,2% | -30,4% |
| Overall | -24,1% | -22,1% | -22,4% | -31,6% | -30,2% | -30,4% |

### SNR scalability

With this test, the performance of SNR scalability is examined, where the resolution ratio between a BL and an EL is 1. In table 12, EL-only gains are summarized, where average gains of 37.6%, 25.6% and 23.8% are observed for Y, U and V components, respectively.

Table 12. Performance of EL-only rate

|  | Actual rate |  |  | Target rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -39,2% | -27,6% | -27,2% | -40,1% | -28,8% | -28,4% |
| Class B | -36,1% | -23,6% | -20,5% | -37,6% | -25,3% | -22,2% |
| Overall | -37,6% | -25,6% | -23,8% | -38,9% | -27,0% | -25,3% |

In case of EL+BL performance comparison, the observed average gains for Y, U, and V components are 25.2%, 15.0%, and 13.5%.

Table 13. Performance of EL+BL rate

|  | Actual rate |  |  | Target rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -26,0% | -16,3% | -16,0% | -26,7% | -17,2% | -16,8% |
| Class B | -24,4% | -13,7% | -11,0% | -25,4% | -14,8% | -12,2% |
| Overall | -25,2% | -15,0% | -13,5% | -26,0% | -16,0% | -14,5% |

### Overall

With the category 1 test, the scalable bitstream achieves average Y, U, and V gains 41.0%, 33.7% and 33.3% in case of EL-only rate comparison, and 26.3%,20.0% and 19.6% in case of EL+BL rate comparison.

## Category 2 (AVC base layer)

With the category 2 tests, the performance of the coding standard scalability is examined, where BLs are encoded with H.264/AVC standard.

### Spatial scalability

Similarly to the category 1 spatial scalability in Section 4.1.1, 2 downscaling factors of 1.5 and 2 are considered and RA from the common test condition is applied. In the EL-only comparison, the proposed scalable codec achieves average Y, U, and V gains of 28.4%, 17.3%, and 16.9% for 2x scalability and 43.6%, 35.0%, and 33.6% for 1.5x scalability as summarized in Tables 14 and 15.

Table 14. Performance of EL-only actual rate with respect to HEVC anchor

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -31,5% | -18,7% | -19,4% |  |  |  |
| Class B | -25,3% | -15,9% | -14,4% | -43,6% | -35,0% | -33,6% |
| Overall | -28,4% | -17,3% | -16,9% | -43,6% | -35,0% | -33,6% |

Table 15. Performance of EL-only target rate with respect to HEVC anchor

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -32,5% | -19,8% | -20,5% |  |  |  |
| Class B | -27,7% | -18,5% | -17,0% | -44,5% | -36,0% | -34,7% |
| Overall | -30,1% | -19,2% | -18,8% | -44,5% | -36,0% | -34,7% |

In case of EL+BL rate performance comparison, the observed average gains for Y, U, and V components are 19.2%, 9.7%, and 9.4% in case of 2x scalability, and 25.2%, 17.9%, and 16.8% for 1.5x scalability.

Table 16. Performance of EL+BL actual rate with respect to HEVC anchor

|  | Hybrid vs HEVC 2x |  |  | Hybrid vs HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -21,2% | -10,3% | -10,9% |  |  |  |
| Class B | -17,1% | -9,1% | -7,8% | -25,2% | -17,9% | -16,8% |
| Overall | -19,2% | -9,7% | -9,4% | -25,2% | -17,9% | -16,8% |

Table 17. Performance of EL+BL target rate with respect to HEVC anchor

|  | Random Access HEVC 2x |  |  | Random Access HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -21,9% | -11,0% | -11,6% |  |  |  |
| Class B | -18,6% | -10,7% | -9,5% | -25,8% | -18,6% | -17,5% |
| Overall | -20,2% | -10,9% | -10,6% | -25,8% | -18,6% | -17,5% |

# Complexity analysis

## Encoding time and measurement methodology

For encoding time measurement, a clustered system with multiple CPUs and RAMs was used. Each CPU has at most similar clock-speed with AMD Opteron 2.6GHz. The 32 bit executable is generated by GCC 4.12. The execution time is measured on RedHat Linux using intrinsic API functions of Linux.

As specified in the CfP document, the encoding time at the second lowest QP is measured.

With the current implementation, the encoding complexity of the proposed scalable codec appears to be excessively higher than that of the HM 6.1 anchor. However, it has to be noted that the test system was overloaded with lots of simulations, and thus, the measured time may not reflect the actual encoding time of the proposed scalable video codec.

Table 18. Encoding time measurement result of the HM 6.1 anchor

| Class | Resolution | Sequence | Spatial scalability |  |  |  | SNR scalability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RA HEVC |  | AI HEVC |  | RA HEVC |  |
|  |  |  | [s] | [hr] | [s] | [hr] | [s] | [hr] |
| A+ | 3840x2048 | Traffic | 26871.64 | 7.46 | 9218.81 | 2.56 | 29338.69 | 8.15 |
|  | 3840x2160 | PeopleOnStreet | 37398.48 | 10.39 | 10000.44 | 2.78 | 41382.43 | 11.50 |
| B | 1920x1080 | Kimono | 12855.80 | 3.57 | 3554.03 | 0.99 | 13208.29 | 3.67 |
|  |  | ParkScene | 11912.20 | 3.31 | 4216.42 | 1.17 | 12377.12 | 3.44 |
|  |  | Cactus | 27720.10 | 7.70 | 9085.57 | 2.52 | 31600.89 | 8.78 |
|  |  | BasketballDrive | 31269.95 | 8.69 | 8281.43 | 2.30 | 33020.80 | 9.17 |
|  |  | BQTerrace | 36395.72 | 10.11 | 11503.32 | 3.20 | 41295.75 | 11.47 |
|  |  | Average | 24196.05 | 6.72 | 7383.02 | 2.05 | 26234.55 | 7.29 |

Table 19. Encoding time measurement result of the proposed scalable extension

| Class | Resolution | Sequence | Spatial scalability |  |  |  | SNR scalability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RA HEVC |  | AI HEVC |  | RA HEVC |  |
|  |  |  | [s] | [hr] | [s] | [hr] | [s] | [hr] |
| A+ | 3840x2048 | Traffic | 81108,46 | 22,53 | 24603,04 | 6,83 | 107269,84 | 29,79717778 |
|  | 3840x2160 | PeopleOnStreet | 117152,36 | 32,54 | 27562,42 | 7,66 | 156634,09 | 43,50946944 |
| B | 1920x1080 | Kimono | 40793,48 | 11,33 | 9810,86 | 2,73 | 51853,42 | 14,40372778 |
|  |  | ParkScene | 38055,63 | 10,57 | 11205,92 | 3,11 | 49024,96 | 13,61804444 |
|  |  | Cactus | 84093,80 | 23,36 | 23763,42 | 6,60 | 115926,16 | 32,20171111 |
|  |  | BasketballDrive | 93256,28 | 25,90 | 21625,92 | 6,01 | 126763,07 | 35,21196389 |
|  |  | BQTerrace | 114514,73 | 31,81 | 28457,72 | 7,90 | 163005,18 | 45,27921667 |
|  |  | Average | 81282,10 | 22,58 | 21004,18 | 5,83 | 110068,10 | 30,57 |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM

For decoding time measurement, a clustered system with multiple CPUs and RAMs was used. Each CPU has at most similar clock-speed with AMD Opteron 2.6GHz. The 64 bit executable is generated by GCC 4.12. The execution time is measured on RedHat Linux using intrinsic API functions of Linux.

Similarly to the encoding time measurement, the decoding time at the second lowest QP was recorded. As we stated in Section 5.1, the time measurement could be biased due to the overload of the test system.

Table 20. Decoding time measurement result of the HM 6.1 anchor (sec)

| Class | Resolution | Sequence | Spatial scalability |  | SNR scalability |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RA HEVC | AI HEVC | RA HEVC |
| A+ | 3840x2048 | Traffic | 91.87 | 138.68 | 73.95 |
|  | 3840x2160 | PeopleOnStreet | 98.16 | 148.76 | 108.45 |
| B | 1920x1080 | Kimono | 31.53 | 42.93 | 32.77 |
|  |  | ParkScene | 32.57 | 61.76 | 34.06 |
|  |  | Cactus | 82.32 | 130.07 | 77.64 |
|  |  | BasketballDrive | 89.89 | 109.81 | 77.87 |
|  |  | BQTerrace | 110.38 | 163.84 | 118.31 |
|  |  | Average | 69.15 | 103.59 | 67.53 |

Table 21. Encoding time measurement result of the proposed scalable extension (sec)

| Class | Resolution | Sequence | Spatial scalability |  | SNR scalability |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RA HEVC | AI HEVC | RA HEVC |
| A+ | 3840x2048 | Traffic | 130,94 | 206,55 | 171,55 |
|  | 3840x2160 | PeopleOnStreet | 180,43 | 228,04 | 237,79 |
| B | 1920x1080 | Kimono | 66,59 | 78,44 | 77,69 |
|  |  | ParkScene | 65,61 | 94,06 | 80,06 |
|  |  | Cactus | 137,12 | 194,57 | 158,44 |
|  |  | BasketballDrive | 136,62 | 178,09 | 177,52 |
|  |  | BQTerrace | 181,89 | 238,98 | 228,77 |
|  |  | Average | 128,46 | 174,10 | 161,69 |

# Comparison with original CfP responses from Fraunhofer HHI, Samsung and Vidyo

For this contribution only a reduced set of scalable tools used in original CfP responses from Fraunhofer HHI (JCTVC-K0042), Samsung and Vidyo (JCTVC-K0045) was selected:

- Texture prediction from BL (i.e. Intra_BL mode)

- Inter-layer motion and partition prediction

- Difference coding mode (coding the difference between EL and coded BL)

Among the various implementations of these tools in JCTVC-K0042 and JCTVC-K0045 the simplest and the most efficient variant was carefully selected considering short experimental tests. Full length simulation was performed using the QP settings from JCTVC-K0042 in order to have minimal deviation from target bit-rate.

The luma BD-rate performance of JCTVC-K0042, JCTVC-K0045 and the suggested tool set combination is summarized in Table 22, for the actual results. Performance tests for this contribution and for JCTVC-K0045 simulations were performed using Samsung’s cluster. The run-time comparison between the suggested tool set and JCTVC-K0045 is also shown in the Table 22.

Table 22. Preliminary Performance summary of original CfP responses and set of selected tools.

| Variant | x2 AI | x1.5 AI | x2 RA | x1.5 RA | SNR RA | AVC, x2 | AVC, x1.5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JCTVC-K0345 | 34.8% | 54.3% | 30.5% | 47.9% | 37.6% | 28.4% | 43.6% |
| JCTVC-K0045 | 35.4% | 54.1% | 30.3% | 47.1% | 36.8% | 28.9% | 43.2% |
| JCTVC-K0042 | 35.8% | 54.5% | 31.5% | 48.9% | 39.3% | 30.0% | 44.9% |
| Dec. t. vs K0045 | 93% | 94% | 92% | 94% | 95% | 93% | 92% |

The Excel spreadsheet attached to this contribution reports test results compared to the CfP anchor and additionally show run-time difference between JCTVC-K0345 and JCTVC-K0045. These tests results show a 5-8% decoding time reduction compared to the original CfP response from Samsung and Vidyo (JCTVC-K0045). The encoding times are either slightly reduced or slightly increased for the different test categories.

# Patent rights declaration(s)

Fraunhofer HHI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Vidyo, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).