Software manual

- The following encoder configuration options are added to support scalable coding:

  - ReconFile1: name of the enhancement layer reconstructed YUV file

  - InputFile1: name of the enhancement layer input YUV file

  - SourceWidth1: width of the enhancement layer pictures

  - SourceHeight1: height of the enhancement layer pictures

  - QP1: QP to use to code the enhancement layer pictures

As an example, the following configuration can be used to encode 2 spatial layers with 1:2 ratio between the dimensions of the reference layer and those of the enhancement layer.  Currently only 1:1, 1:1.5, and 1:2 ratios are supported.

ReconFile                     : C:\hmEnc.yuv

InputFile                     : C:\BQTerrace_960x536_60.yuv

SourceWidth                   : 960

SourceHeight                  : 536

ReconFile1                    : C:\hmEnc1.yuv

InputFile1                    : C:\BQTerrace_1920x1072_60.yuv

SourceWidth1                  : 1920

SourceHeight1                 : 1072

QP                            : 35

QP1                           : 37

Just like the case with JSVM, the above configuration will produce a single bitstream (specified by the field “BitstreamFile”).  The decoder, by default, decodes the highest layer available in the bitstream.  A new option (-l) is added to the decoder to specify the maximum layer to decode.  For example, “–l 0” can be used to decode only Layer 0 (the base layer); and “–l 1” can be used to decode layers up to Layer 1 (the base layer and the enhancement layer).

- The provided code also supports coding of the enhancement layer using pre-coded reference layer data.  This feature can be used to code the enhancement layer using pre-coded bitstream as well as to support “mixed codec scalability”.  To support “difference coding mode”, reference layer’s decoded YUV file is needed and to support inter-layer motion prediction, a file containing reference layer’s motion information is needed.  The following encoder configuration options are added to support this:

  - BlReconFile: name of the base layer reconstructed YUV file

  - BlMetaFile: name of the base layer meta data file that contains the motion information

  - BlWidth: width of the base layer pictures

  - BlHeight: height of the base layer pictures

As an example, the following configuration can be used to encode the enhancement layer of 2 spatial layers with 1:2 ratio between the dimensions of the reference layer and those of the enhancement layer.

ReconFile                     : C:\hmEnc.yuv

InputFile                     : C:\BQTerrace_960x536_60.yuv

SourceWidth                   : 960

SourceHeight                  : 536

BlReconFile                   : C:\baseLayer.yuv

BlMetaFile                    : C:\baseLayerInfo.bin

BlWidth                       : 960

BlHeight                      : 536

ReconFile1                    : C:\hmEnc1.yuv

InputFile1                    : C:\BQTerrace_1920x1072_60.yuv

SourceWidth1                  : 1920

SourceHeight1                 : 1072

QP                            : 35

QP1                           : 37

In the above case, the information provided by “ReconFile”, “InputFile”, “SourceWidth”, and “SourceHeight” are ignored.  Additionally, the generated bitstream only contains the enhancement layer coded data.  To decode such bitstream, few more options are added to the decoder.  For example, the following can be used to decode the bitstream generated using the above configuration:

hmDecoder.exe –b hm.bin –o hm.yuv -blw 960 -blh 536 -blo baseLayer.yuv -blm baseLayerInfo.bin

The YUVs and the motion information must be stored in display order; the modified HM code reads corresponding reference layer information from the specified files based on its POC values.  This is a hack assuming that POC values in HM increase only by 1.

- The decoder is modified to output the meta data file from a coded base layer bitstream.  The name of the output meta data file can be specified with “–om baseLayerInfo.bin”

- Added support for base layer resolution not in multiples of SCU (8x8).  When the base layer resolution is not in multiples of SCU, then padding can be applied using (“CroppingMode” = 1).  The blocks of the enhancement layer will properly use corresponding BL region.

As an example, the following configuration can be used to encode 2 spatial layers with the base layer width not in multiples of 8.

ReconFile                     : C:\hmEnc.yuv

InputFile                     : C:\BQTerrace_960x540_60.yuv

SourceWidth                   : 960

SourceHeight                  : 540

ReconFile1                    : C:\hmEnc1.yuv

InputFile1                    : C:\BQTerrace_1920x1080_60.yuv

SourceWidth1                  : 1920

SourceHeight1                 : 1080

QP                            : 35

QP1                           : 37

CroppingMode                  : 1

When the base layer resolution is in multiples of SCU and padding is not needed, the following two cases (Case A and Case B) yield exactly the same enhancement layer bitstreams.

Case A: Encode both base and enhancement layers using the following configuration

ReconFile                     : C:\hmEnc.yuv

InputFile                     : C:\BQTerrace_960x536_60.yuv

SourceWidth                   : 960

SourceHeight                  : 536

ReconFile1                    : C:\hmEnc1.yuv

InputFile1                    : C:\BQTerrace_1920x1072_60.yuv

SourceWidth1                  : 1920

SourceHeight1                 : 1072

QP                            : 35

QP1                           : 37

Case B: Encode only the enhancement layer using the coded base layer bitstream

ReconFile                     : C:\hmEnc.yuv

InputFile                     : C:\BQTerrace_960x536_60.yuv

SourceWidth                   : 960

SourceHeight                  : 536

BlReconFile                   : C:\baseLayer.yuv

BlMetaFile                    : C:\baseLayerInfo.bin

BlWidth                       : 960

BlHeight                      : 536

ReconFile1                    : C:\hmEnc1.yuv

InputFile1                    : C:\BQTerrace_1920x1072_60.yuv

SourceWidth1                  : 1920

SourceHeight1                 : 1072

QP                            : 35

QP1                           : 37

When the base layer resolution is NOT in multiples of SCU and padding is needed, then the coded enhancement layer bistreams for the two cases would be slightly different.  Let’s consider 960x540 base layer as an example.  In Case A, the base layer’s input picture is padded to the bottom to be 960x544 and the reconstructed picture is in 960x544.  When scaling up the reconstructed picture to generate difference values, padding is added to the top, left, and right of the picture, but not to the bottom, as the picture already includes enough padding.  In Case B, however, the decoded picture is in 960x540 and padding is added to all boundaries of the picture for the scaling up process.