| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0347 |
| --- | --- |

| Title: | Persistence of VPS: bitstream instead of coded video sequence |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Nokia Corporation, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the current HEVC draft specification, the active VPS is allowed to be changed for each coded video sequence. Hence, the number of VPSes active for the entire bitstream may be large.

Use cases where video parameter set is used in systems specifications include the capability exchange of session negotiation (e.g. the Offer/Answer model of the Session Description Protocol, SDP) and declarative indication of bitstream properties (e.g. a part of session description passed through Real-time Streaming Protocol, RTSP, or a part of media presentation description, MPD, in dynamic adaptive streaming over HTTP, i.e. DASH). It is asserted that it would be convenient in the protocol design and implementation for these use cases, if a VPS pertained for the entire bitstream and consequently only one VPS had to be communicated in session negotiation.

It is proposed that VPS pertains to a bitstream instead of a coded video sequence. In other words, it is proposed that an active VPS remains active until the end of the bitstream.

# Specification Text Changes

3.147	video parameter set (VPS): A syntax structure containing syntax elements that apply to zero or more entire bitstreams coded video sequences as determined by the content of a video_parameter_set_id syntax element found in the sequence parameter set referred to by the seq_parameter_set_id syntax element, which is found in the picture parameter set referred to by the pic_parameter_set_id syntax element found in each slice header.

- 7.4.1.4.1	Order of video, sequence, and picture parameter set RBSPs and their activation

…

- A video parameter set RBSP includes parameters that can be referred to by one or more sequence parameter set RBSPs. Each video parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one video parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular video parameter set RBSP results in the deactivation of the previously-active video parameter set RBSP (if any).

- When a video parameter set RBSP (with a particular value of video_parameter_set_id) is not already active and it is referred to by activation of a sequence parameter set RBSP (using that value of video_parameter_set_id) for the first coded video sequence of the bitstream, it is activated. This video parameter set RBSP is called the active video parameter set RBSP until it becomes deactivated at the end of the bitstream it is deactivated by the activation of another video parameter set RBSP. A video parameter set RBSP, with that particular value of video_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with TemporalId equal to 0, unless the video parameter set is provided through external means. An activated video parameter set RBSP shall remain active for the entire bitstream coded video sequence.

- Any video parameter set NAL unit containing the value of video_parameter_set_id for the active video parameter set RBSP for a coded video sequence shall have the same content as that of the active video parameter set RBSP for the coded video sequence unless it follows the last access unit of the bitstream coded video sequence and precedes the first VCL NAL unit and the first sequence parameter set NAL unit of another coded video sequence.

- 7.4.3	Profile, tier and level semantics

…

NOTE 1 – A greater value of general_level_idc indicates a higher level. The maximum level signalled in the active video parameter set for a coded video sequence may be higher than the level signalled in a the sequence parameter set which is active for a the same coded video sequence of the same bitstream.

# Patent rights declaration(s)

Nokia Corporation does not have any current or pending patent rights relating to the technology described in this contribution.

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).