| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0348 |
| --- | --- |

| Title: | A proposal for Scalable HEVC Test Model |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Chen, V. Seregin, M. Zhou, B. Jeon, S. Lei, E. Nassor, K. Ugur, A. Segall, A. Tabatabai, H. Yu, J.W. Kang, Y. Ye, W. Zhang, P. Bordes | Tel: Email: |  |
| Source: | Qualcomm, TI, LGE, Mediatek, Canon, Nokia, Sharp, Sony, Huawei, ETRI, InterDigital, Intel, Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes a potential software for a starting point for the scalable HEVC proposal.

# Introduction

It is observed that the responses to the Scalable HEVC extension share the following characteristics:

- Many tools are identical or very similar in spirit.

- Coding efficiency results are quite close to each other.

- Almost all the proposals are multi-loop and implemented on top of HM6.1

Based on these observations, we proposed to start with a very simple initial test model representing the lowest common denominator and define CE’s on top of that software [1]. This contribution describes the proposed software.

# Software Description

In order to speed up the process of setting up Core Experiments with the most recent version of HM, we are attaching initial software based on HM8.1. The proposed software is minimalistic and implements only the inter-layer texture prediction tool (as described in JCTVC-K0038) as it was shown that this simple tool brings the majority of the gain. In addition, the software has interface to the base layer motion and intra mode information so that CEs using those information could start immediately. In addition, an MVC-like HLS-only approach of signalling inter layer prediction without any additional coding tools will be added soon so that CEs using HLS signaling could also start immediately. The HLS signalling option is disabled by default in the software.

## Inter-layer texture prediction

The initial software has the CU based inter-layer texture as described in JCTVC-K0038 and explained as follows:

Figure 1. Proposed scalable HEVC codec structure.

The up-sampled reconstructed base-layer picture is used as a reference for inter-layer prediction in enhancement layer. Inter-layer prediction (ILP) is performed at CU level in this proposal. Figure 2 shows the CU encoding flow in enhancement layer. For an input CU, the best CU mode is determined among best intra mode, best inter mode and ILP mode. ILP mode cost w.r.t the collocated up-sampled base-layer CU is calculated additionally in enhancement layer. Once the best mode is chosen, the CU is coded by the same procedure as for base layer.

Figure 2. CU encoding flow in enhancement layer.

Figure 3 shows the CU decoding flow for enhancement layer in detail. ILP flag is signaled after skip_flag only when skip_flag is equal to 0. If skip_flag is equal to 1 in inter picture, the corresponding CU is decoded in the same way as in base layer. If ilp_flag is equal to 0, it is decoded in the same way as in base layer, too. Otherwise, after decoding TU partition and inverse Q/T, the CU is reconstructed by adding the collocated base-layer CU reconstruction.

Figure 3. CU decoding flow in enhancement layer.

A 12-phase DCT-IF up-sampling filter in [2] is employed to scale the reconstructed base-layer picture to the size of enhancement layer in case of spatial scalable coding. Up-sampling filter coefficients are shown in Table 1 and Table 2 for luma and chroma, respectively.

Table 1. 8-tap luma up-sampling filter

| Position | Filter coefficients |
| --- | --- |
| 1/12 | { -1, 1, -4, 63, 6, -2, 1, 0 } |
| 2/12 | { -1, 3, -7, 61, 12, -5, 2, -1} |
| 3/12 | { -1, 4, -10, 57, 19, -7, 3, -1 } |
| 4/12 | { 1, 4, -11, 52, 26, -8, 3, -1} |
| 5/12 | { -1, 5, -12, 47, 32, -9, 3, -1 } |
| 6/12 | { -1, 4, -11, 40, 40, -11, 4, -1} |

Table 2. 4-tap chroma up-sampling filter

| Position | Filter coefficients |
| --- | --- |
| 1/12 | { -2, 62, 5, -1} |
| 2/12 | { -4, 59, 11, -2 } |
| 3/12 | { -4, 54, 16, -2 } |
| 4/12 | { -5, 50, 22,-3 } |
| 5/12 | {-5, 43, 30, -4 } |
| 6/12 | {-4, 36, 36, -4 } |

## Interface to base layer information

Motion and intra mode prediction from base is found to be one of the most common tools among other submissions to the CfP. The proposed software implements an interface to get this information (motion and intra mode information) from base layer. This interface is tested using simple inter-layer coding tools. These tools are added to the software to verify the interface works and will not form the anchor of the initial test model. If needed, the tools could also be removed after verifying the hooks work. The details of those exemplary tools are:

Modification of merge mode: The collocated base-layer MV is inserted as the first candidate in merge list. The base-layer MV is obtained at a location co-located to the central position of the current block. In the case of spatial scalability, the base-layer MV is scaled according to the spatial resolution ratio between the two layers. The pruning is done for each spatial candidate with co-located base layer MV.

Modification of Intra mpm list: Intra modes from the collocated base-layer, the left neighbor and the above neighbor are initially listed as MPM0, MPM1 and MPM2. HEVC method is used to derive the MPM list when duplication happens.

# Next Steps

We propose to form a group (or AhG) with the mandate to finalize the initial software for the scalable HEVC project and make it ready for core experiments defined at this meeting. Potential mandates could be:

  - To verify the test model implements the inter-layer texture prediction tool as described in JCTVC-K0038 and no other coding tools as the anchor.

  - To verify the motion and intra mode hooks are present in the software with simple exemplary tools, where the tools are turned off by default or removed in the initial test model.

  - To identify and add any missing functionalities to the software, such as using MVC-like high level syntax only option.

# Conclusion

We proposed candidate initial software for the scalable extension of HEVC project. The candidate software has the following features:

- HM 8.1 based: This allows the core experiments start immediately instead of waiting to port the old version of the code to the new HM versions.

- Only a single tool: Inter-layer texture prediction is found to be the most common denominator among all the proposals and this is the only tool included in the proposed candidate software.

- Interface to base layer information: In addition to getting the reconstructed base layer samples, the software includes a simple interface to get motion and intra mode information from base layer. The corresponding tools will be turned off for the initial test model anchor.

We believe the benefits of this approach are as follows:

- Simple initial test model allows testing small variations of similar tools proposed by many companies within well-defined experiments. This makes it easier to make fact-based decisions.

- The software is based on the most recent HM version instead of an older version. This will make the start of CEs easier, the data gathered from CEs more reliable and speed-up the process.

# References

- K. Ugur(Nokia), E. Nassor (Canon), J. Chen (Qualcomm), B. Jeon (LGE), S. Lei (Mediatek), A. Segall (Sharp), M. Zhou (TI), Y. Ye (InterDigital), A. Tabatabai (Sony), J. W. Kang (ETRI), “A proposal for Scalable HEVC Test Model”, Document JCTVC-K0348, October 2012.

- J. Boyce, D. Hong and W. Jang, “Information for HEVC scalability extension”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G078, November 2011.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mediatek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).