| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0351 |
| --- | --- |

| Title: | On num_subsequent_pcm specification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Email: | chono@ct.jp.nec.com hao@bu.jp.nec.com wwan@broadcom.com |
| Source: | NEC Corporation and Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution addresses a bitstream conformance issue of the current num_subsequent_pcm specification and suggests a solution avoiding misinterpretation of the concept. This contribution also highlights the potential misuse of num_subsequent_pcm in practice since there is no built-in mechanism preventing an encoder from sending an invalid value in that syntax.

It is recommended that JCT-VC review the suggested the solution and discuss the bitstream conformance issue.

# Background

Ticket 763 reports that when NumPcmUnits is not equal to 0, the default value of split_cu_flag is not specified. Figure 1 shows an example of 4 IPCM CUs within an LCU that are transmitted in burst mode and the split_cu_flags of the last 3 IPCM CUs are omitted. JCTVC-K0213-r3 presents a solution dealing with this issue.

Figure 1	Example of burst IPCM.

Through the investigation of the Ticket 763 issue and its solution presented in JCTVC-K0213-r3, we discovered another issue with the current num_subsequent_pcm specification. Specifically, there is nothing in place to prevent an encoder from creating a bitstream where NumPcmUnits > 0 in all of the possible invalid cases except for the following red-colored text which is highly susceptible to being interpreted in different ways:

num_subsequent_pcm specifies the number of subsequent coding units with pcm_flag equal to 1, PartMode equal to PART_2Nx2N and log2CbSize equal to the current log2CbSize, that successively follow the current coding unit with pcm_flag equal to 1 in the same depth in a coding tree unit. It is a requirement of bitstream conformance that the immediate roots of the current and subsequent coding units with pcm_flag equal to 1 are identical. The value num_subsequent_pcm shall be in the range of 0 to 3, inclusive. [Ed. (GJS): Clarify "immediate roots".]

# Solution

In order to avoid misinterpretation of the requirement of bitstream conformance, we suggest the following two changes to the current text (on top of the JCTVC-K0213-r3 Ticket 763 solution):

- Add two resets of “NumPcmUnits = 0” in the coding quadtree syntax at the start and the end of a new CU layer in order to ensure that successive IPCM CUs on different CU layers are not coded with burst mode;

- Explicitly describe the mandated relationships among the IPCM CUs associated with NumPcmUnits larger than 0 with changes to the  num_subsequent_pcm semantics.

The suggested text changes are described in Section 7.

# Discussion

The bitstream conformance issue is alleviated by the suggested changes. However, there is a case that an encoder violates the bitstream conformance by sending an invalid num_subsequent_pcm. For example, an encoder may send the num_subsequent_pcm value of 3 at the last CU in a CTU since there is no built-in mechanism preventing an encoder from sending an invalid value (Figure 2). Although such invalid values falls outside the bitstream requirements, people may misunderstand the concept of this IPCM burst signalling and generate invalid bitstreams in practice.

Figure 2 Example of requirement violation of bistream conformance.

# Conclusion

This contribution has addressed a bitstream conformance issue of the current num_subsequent_pcm specification and suggested a text modification to avoid misinterpretation of the concept. This contribution also highlights potential misuse of num_subsequent_pcm since there is no built-in mechanism preventing an encoder from sending  invalid values.

It is recommended that JCT-VC reviews the suggested the solution and discuss the bitstream conformance issue.

# References

- K.Chono, “AHG9: On Tickets 637, 763, and 777, ” Document of Joint Collaborative Team on Video Coding, JCTVC-K0213-r3, October 2012

- B. Bross, G. J. Sullivan, T. K. Tan and Ye-Kui Wang, “Proposed Editorial Improvements for High efficiency video coding (HEVC) Text Specification Draft 8,” Document of Joint Collaborative Team on Video Coding, JCTVC-K0030_v6, October 2012.

# References

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text Changes

## Section 7.3.8.4	Coding quadtree syntax

| coding_quadtree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples && log2CbSize > Log2MinCbSizeY && NumPcmUnits = = 0 ) |  |
| split_cu_flag[ x0 ][ y0 ] | ae(v) |
| if( ( cu_qp_delta_enabled_flag && log2CbSize >= Log2MinCuQpDeltaSize ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDelta = 0 |  |
| } |  |
| if( split_cu_flag[ x0 ][ y0 ] ) { |  |
| NumPcmUnits = 0 |  |
| x1 = x0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CbSize ) >> 1 ) |  |
| coding_quadtree( x0, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_quadtree( x1, y0, log2CbSize − 1, ctDepth + 1 ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x0, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| coding_quadtree( x1, y1, log2CbSize − 1, ctDepth + 1 ) |  |
| NumPcmUnits = 0 |  |
| } else { |  |
| if( NumPcmUnits = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize ) |  |
| else |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } |  |
| } |  |

## Section 7.4.8.4 Coding quadtree semantic

split_cu_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When split_cu_flag[ x0 ][ y0 ] is not present, the following applies:

–	If log2CbSize is greater than Log2MinCbSizeY, the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1.

–	Otherwise (log2CbSize is equal to Log2MinCbSizeY), the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

–	If log2CbSize is greater than Log2MinCbSizeY and one or more of the following conditions are true, the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1.

–	(x0 + ( 1 << log2CbSize )) is greater than pic_width_in_luma_samples.

–	(y0 + ( 1 << log2CbSize )) is greater than pic_height_in_luma_samples.

–	Otherwise, the value of split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

## Section 7.4.8.6 Coding unit semantics

num_subsequent_pcm specifies the number of subsequent coding units with pcm_flag equal to 1, PartMode equal to PART_2Nx2N and log2CbSize equal to the current log2CbSize, that successively follow the current coding unit with pcm_flag equal to 1 in the same depth in a coding tree unit. It is a requirement of bitstream conformance that the immediate roots of the current and subsequent coding units with pcm_flag equal to 1 are identical. The value num_subsequent_pcm shall be in the range of 0 to 3, inclusive. [Ed. (GJS): Clarify "immediate roots".] that meet all of the following conditions:

–	They must successively follow the current coding unit with pcm_flag equal to 1.

–	They must be in the same coding tree block.

–	They must have log2CbSize equal to the current log2CbSize.

–	They must have the same parent coding unit location, where the parent coding unit location ( px0, py0 ) is derived from the top-left luma sample location ( x0, y0 ) of a coding unit relative to the top-left luma sample of the picture as follows.

px0 = ( x0 >> ( log2CbSize + 1 ) ) << ( log2CbSize + 1 )	(7-XX)

py0 = ( y0 >> ( log2CbSize + 1 ) ) << ( log2CbSize + 1 )	(7-XX)